3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104264r02

Elbonia, May 17 – 28, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, Xiaomi

Title:	Functionality of Layer-2 UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to add the functionality of Layer-2 UE-to-Network Relay.

1. Introduction/Discussion

This contribution proposes to add the functionality of Layer-2 UE-to-Network Relay.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

4.3.9	ProSe UE-to-Network Relay

4.3.9.1	General

UE-to-Network Relay entity provides the relaying functionality to support connectivity to the network for Remote UEs. It can be used for both public safety services and commercial services (e.g. interactive service).

All UE-to-Network Relay’s supports the following functions to enable connectivity to the network:

-	UE-to-Network Relay Discovery service as defined in clause 6.3.2.3, to allow discovery by the Remote UE;

-	ProSe Direct Communication as specified in clause 6.5.1, for the communication with the Remote UEs for the relay operation;

-	access the 5GS as a UE as defined in TS 23.501 [4] with the enhancements as specified in clauses 6.2 and 6.7;

-	relays unicast traffic (uplink and downlink) between the Remote UE and the network, supporting IP, Ethernet or Unstructured traffic type;

-	end-to-end QoS treatment for the Remote UE's traffic.

#### 4.3.9.2	Layer-3 UE-to-Network Relay

In addition to the common UE-to-Network Relay functions defined in clause 4.3.9.1, Layer-3 UE-to-Network Relay supports the following functions to enable connectivity to the network:

-	ProSe Direct Communication via Layer-3 UE-to-Network Relay as specified in clause 6.5.1, for the communication with the Remote UEs for the relay operations;

-	providing access to an N3IWF for the Remote UE when supported by the serving PLMN and home PLMN;

-	end-to-end QoS treatment for the Remote UE's traffic without N3IWF as defined in clause 5.7.2.1 and when accessing via an N3IWF clause 5.7.2.2;

-	IP address management for the Remote UE as defined in clause 5.6 in case the Remote UE uses IP traffic type.

-	providing access to an N3IWF for the Remote UE when supported by the serving PLMN.

4.3.9.3	Layer-2 UE-to-Network Relay

In addition to the common UE-to-Network Relay functions defined in clause 4.3.9.1, Layer-2 UE-to-Network Relay supports the following functions to enable connectivity to the network:

-	ProSe Direct Communication via Layer-2 UE-to-Network Relay as specified in clause 6.5.2, for the communication with the Remote UEs for the relay operations, including end-to-end QoS treatment.

-	QoS handling for Layer-2 UE-to-Network Relay as defined in clause 5.7.2.3;

# * * * * End of changes * * * *