3GPP TSG-SA WG2 Meeting #145E	S2-2104332r03

# Elbonia, May 17 – May 28 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2919 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 T3: Handling of SUPI/SUCI format when accessing to a SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, the SUPI/SUCI of a SNPN-enabled UE can be format of NAI or IMSI, when a UE performs Initial Registration to an SNPN. The selected NID provided by the NG-RAN is used as the NID of the subscribed SNPN, as in Rel-16 an SNPN-enabled UE is only allowed to access the subscribed SNPN. In Rel-17, an SNPN-enabled UE can also access SNPNs different from the subscribed SNPN. This requires some clarifications on which types of SUPI/SUCIs and which characteristics SUPI/SUCI shall support in order to enable these new scenarios, specifically when IMSI based SUPI is used. For example, if the SNPN and the Credentials Holder are using the same PLMN ID and IMSI based SUPI is used, currently the HNI(MCC/MNC) of the corresponding SUCI is not able to distinguish if the UE is from the Credentials Holder or if the UE is a native subscriber. Enable UE to indicate the NID the SUPI relates to |  |  |  |  |  |  |  |  |
| Summary of change: |  | In order for a UE to be able to connect to an SNPN using credential from Credentials Holder and use IMSI based SUPI, the following changes are proposed: the UE shall indicate the NID of the SUPI The HNI of a SUPI in NSI format shall identify the subscribed SNPN. The SUPI type for SNPN access with CH using AAA is also specified (i.e. NSI based). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An SNPN-enabled UE can not access SNPNs different from the one it is subscribed to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with the following information for each subscribed SNPN:

-	PLMN ID and NID of the SNPN;

-	Subscriber identifier (SUPI) and credentials;

-	Optionally, an N3IWF FQDN and an identifier of the country where the configured N3IWF is located;

-	Optionally, if the UE supports access to an SNPN using credentials from a Credentials Holder:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of GINs.

The Credentials Holder controlled prioritized lists of preferred SNPNs and GINs can be updated by the Credentials Holder.

Editor's note:	Whether the Credentials Holder controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT WG1 and SA WG3.

A subscriber of an SNPN is either:

-	identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.7.2. The realm part of the NAI may include the NID of the SNPN; or

-	identified by a SUPI containing an IMSI.

In the case of access to an SNPN using credentials owned by a Credentials Holder using AUSF/UDM as specified in clause 5.30.2.9.3, the SUPI shall also contain identification for the Credentials Holder, i.e. the realm in the case of Network Specific Identifier based SUPI or the MCC and MNC in the case of an IMSI based SUPI as follow:

-	When IMSI based SUPI is used, the IMSI shall be globally unique as defined in TS 23.003 [19] (i.e. the IMSI shall be assigned by a PLMN operator), and when the UE access an SNPN with a SUPI associated to a CH the UE shall indicate the NID the SUPI is associated with within the Registration Request as defined in TS 23.502 [3], and if the SUPI is not related to any NID the UE indicates that.

-	When Network Specific Identifier based SUPI is used, the realm of the SUPI shall uniquely identify the Credentials Holder as defined in TS 23.003 [19].

NOTE 1:	When Credentials Holder is an SNPN, and the MCC and MNC of the SNPN is not unique, then IMSI based SUPI is not supported as the MCC and MNC need not be unique always; instead USIM credentials are supported using Network Specific Identifier based SUPI.

NOTE 2:	Network Specific Identifier based SUPI is not supported for the case the Credentials Holder is provided by a PLMN.

In the case of access to an SNPN using credentials owned by a Credentials Holder using AAA Server as specified in clause 5.30.2.9.2, the SUPI shall contain a Network Specific Identifier.

An SNPN-enabled UE that supports access to an SNPN using credentials from a Credentials Holder and that is equipped with a PLMN subscription may additionally be configured with the following information for SNPN selection and registration using the PLMN subscription in SNPN access mode:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred GINs.

The Credentials Holder controlled prioritized lists of preferred SNPNs and GINs can be updated by the Credentials Holder.

Editor's note:	Whether the Credentials Holder controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT WG1 and SA WG3.

**** Next Change ****

### 6.3.4	AUSF discovery and selection

In the case of NF consumer based discovery and selection, the following applies:

-	The AMF performs AUSF selection to allocate an AUSF Instance that performs authentication between the UE and 5G CN in the HPLMN. The AMF shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on AMF. The AUSF selection function in the AMF selects an AUSF instance based on the available AUSF instances (obtained from the NRF or locally configured in the AMF).

-	The UDM shall utilize the NRF to discover the AUSF instance(s) unless AUSF information is available by other means, e.g. locally configured on UDM. The UDM selects an AUSF instance based on the available AUSF instance(s) obtained from the NRF or based on locally configured information, and information stored (by the UDM) from a previously successful authentication.

AUSF selection is applicable to both 3GPP access and non-3GPP access.

The AUSF selection function in AUSF NF consumers or in SCP should consider one of the following factors when available:

1.	Home Network Identifier (e.g. MNC and MCC, realm) of SUCI/SUPI (by an NF consumer in the Serving network) along with the selected NID (provided by the NG-RAN) or the NID of the SUPI (provided by the UE) in the case of SNPN and Routing Indicator.

NOTE 1:	The UE provides the Routing Indicator to the AMF as part of the SUCI as defined in TS 23.003 [19] during initial registration. The AMF can provide the UE's Routing Indicator to other AMFs as described in TS 23.502 [3].

NOTE 2:	In the case of SNPN and if the UE provides IMSI as SUPI/SUCI without providing any NID of the SUPI to the AMF and if SUCI/SUPI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from SUCI/SUPI) as the SUCI/SUPI does not always include the NID. In the case of SNPN and if the UE provides IMSI as SUPI/SUCI together with the NID of the SUPI, the AMF uses the NID of the SUPI provided by the UE. In the case of SNPN and if the UE provides IMSI as SUPI/SUCI together with an indication that the SUPI is not associated to any NID, the AMF uses the selected PLMN ID (from SUCI/SUPI). In the case of SNPN and the UE provides NSI as SUPI/SUCI to the AMF, the AMF uses the Home Network Identifier for selection of AUSF.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the AUSF NF consumer can select any AUSF instance within the home network for the UE.

2.	AUSF Group ID the UE's SUPI belongs to.

NOTE 3:	The AMF can infer the AUSF Group ID the UE's SUPI belongs to, based on the results of AUSF discovery procedures with NRF. The AMF provides the AUSF Group ID the SUPI belongs to other AMFs as described in TS 23.502 [3].

3.	SUPI; e.g. the AMF selects an AUSF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for AUSF discovery.

In the case of delegated discovery and selection in SCP, the AUSF NF consumer shall send all available factors to the SCP.

**** Next Change ****

### 6.3.8	UDM discovery and selection

The NF consumer or the SCP performs UDM discovery to discover a UDM instance that manages the user subscriptions.

If the NF consumer performs discovery and selection, the NF consumers shall utilize the NRF to discover the UDM instance(s) unless UDM information is available by other means, e.g. locally configured on NF consumers. The UDM selection function in NF consumers selects a UDM instance based on the available UDM instances (obtained from the NRF or locally configured).

The UDM selection functionality is applicable to both 3GPP access and non-3GPP access.

The UDM selection functionality in NF consumer or in SCP should consider one of the following factors:

1.	Home Network Identifier (e.g. MNC and MCC, realm) of SUCI/SUPI, along with the selected NID (provided by the NG-RAN) or the NID of the SUPI (provided by the UE) in the case of SNPN, and UE's Routing Indicator.

NOTE 1:	The UE provides the Routing Indicator to the AMF as part of the SUCI as defined in TS 23.003 [19] during initial registration. The AMF provides the UE's Routing Indicator to other NF consumers (of UDM) as described in TS 23.502 [3].

NOTE 2:	In the case of SNPN and the UE provides IMSI as SUPI/SUCI without providing any NID of the SUPI  to the AMF and if SUCI/SUPI is for an SNPN served by the AMF, the AMF uses the selected NID provided by the NG-RAN together with the selected PLMN ID (from SUCI/SUPI) as the SUCI/SUPI does not always include the NID. In the case of SNPN and if the UE provides IMSI as SUPI/SUCI together with the NID of the SUPI, the AMF uses the NID of the SUPI provided by the UE. In the case of SNPN and if the UE provides IMSI as SUPI/SUCI together with an indication that the SUPI is not associated to any NID, the AMF uses the selected PLMN ID (from SUCI/SUPI). In the case of SNPN and the UE provides NSI as SUPI/SUCI to the AMF, the AMF uses the Home Network Identifier for selection of UDM.

When the UE's Routing Indicator is set to its default value as defined in TS 23.003 [19], the UDM NF consumer can select any UDM instance within the home network of the SUCI/SUPI.

2.	UDM Group ID of the UE's SUPI.

NOTE 3:	The AMF can infer the UDM Group ID the UE's SUPI belongs to, based on the results of UDM discovery procedures with NRF. The AMF provides the UDM Group ID the SUPI belongs to other UDM NF consumers as described in TS 23.502 [3].

3.	SUPI or Internal Group ID; the UDM NF consumer selects a UDM instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI or Internal Group ID as input for UDM discovery.

4.	GPSI or External Group ID; UDM NF consumers which manage network signalling not based on SUPI/SUCI (e.g. the NEF) select a UDM instance based on the GPSI or External Group ID range the UE's GPSI or External Group ID belongs to or based on the results of a discovery procedure with NRF using the UE's GPSI or External Group ID as input for UDM discovery.

In the case of delegated discovery and selection in SCP, NF consumer shall include one of these factors in the request towards SCP.

*** End of Changes ***