SA WG2 Meeting #145E e-meeting	S2-2104338r04

Elbonia, May 17 – 28, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2801 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional parameters using UE Parameters Update via UDM Control Plane Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ETRI, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  |  | Date: |  | 2021-05-10 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been agreed to use UE Parameters Update via UDM Control Plane Procedure (UPU) for some additional parameters: - Provisioning of NSSAA credentials - Provisioning of PDU session authentication credentials The usage of UPU requires the UE to indicate its capability to support the new parameters. Some parameters have not yet been agreed to use UPU, but it is proposed to use UPU for the following as well: 3. Preferred SNPN and Group ID lis |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extended the UPU procedure with possible parameters to be delivered |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.20.1, 4.20.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

4.20	UE Parameters Update via UDM Control Plane Procedure

4.20.1	General

The purpose of the control plane solution for update of UE parameters is to allow the HPLMN, or SNPN or CH to update the UE with a specific set of parameters, generated and stored in the UDM, by delivering protected UDM Update Data via NAS signalling. The HPLMN, SNPN or CH updates such parameters based on the operator policies.

The UDM Update Data that the UDM delivers to the UE may contain:

-	one or more UE parameters including:

-	the updated Default Configured NSSAI (final consumer of the parameter is the ME);

-	NSSAA credentials per S-NSSAI (final consumer of the par	ameter is the ME or USIM);

-	DN-specific credentials for authentication/authorization of the PDU Session establishment (final consumer of the parameter is the ME or USIM); and

-	Lists for SNPN selection (see TS 23.501 [2] clause 5.30.2.3):

-	Credentials Holder controlled prioritized list of preferred SNPNs (final consumer of the parameter is the ME or USIM);

-	Credentials Holder controlled prioritized list of GINs (final consumer of the parameter is the ME or USIM).

-	the updated Routing Indicator Data (final consumer of the parameter is the USIM).

-	a "UE acknowledgement requested" indication.

-	a "re-registration requested" indication.

4.20.2	UE Parameters Update via UDM Control Plane Procedure

Figure 4.20.2-1: UE Parameters Update via UDM Control Plane Procedure

1.	UDM decides to perform UE parameter update.

2.	From UDM to the AMF: The UDM notifies the changes of the information related to the UE to the affected AMF by the means of invoking Nudm_SDM_Notification service operation. The Nudm_SDM_Notification service operation contains the UDM Update Data (e.g. "Routing Indicator update data", "Default Configured NSSAI update data") that needs to be delivered transparently to the UE over NAS within the Access and Mobility Subscription data. The UDM update data includes:

-	The updated parameters to be delivered to the UE (see clause 4.20.1 for parameters possible to deliver).

-	whether the UE needs to send an ack to the UDM.

-	whether the UE needs to re-register after updating the data.

3.	From AMF to UDM: If AMF determines that the UE is not reachable, then AMF invokes the Nudm_SDM_Info service operation to UDM indicating that the transmission of UE Parameters Update data is not successful. The UDM considers the procedure as UE Parameters Update procedure as pending and subsequent steps from 4-7 are skipped.

4.	From AMF to the UE: the AMF sends a DL NAS TRANSPORT message to the served UE. The AMF includes in the DL NAS TRANSPORT message the transparent container received from the UDM.

The UE verifies based on mechanisms defined in TS 33.501 [15] that the UDM Update Data is provided by HPLMN, SNPN or CH; and:

-	If the security check on the UDM Update Data is successful, as defined in TS 33.501 [15] the UE either stores the information and uses those parameters from that point onwards, or forwards the information to the USIM; and

-	If the security check on the UDM Update Data fails, the UE discards the contents of the UDM Update Data.

5.	The UE to the AMF: If the UE has verified that the UDM Update Data is provided by HPLMN, SNPN, CH and the UDM has requested the UE to send an ack to the UDM, the UE sends an UL NAS TRANSPORT message to the serving AMF with a transparent container including the UE acknowledgement.

6.	The AMF to the UDM: If the AMF receives an UL NAS TRANSPORT message with a transparent container carrying a UE acknowledgement from the UE, the AMF sends a Nudm_SDM_Info request message including the transparent container to the UDM.

7.	If the UDM has requested the UE to re-register, the UE waits until it goes back to RRC idle and initiates a Registration procedure as defined in TS 24.501 [25].

* * * * End of Change * * * *