3GPP TSG-SA WG2 Meeting #145E 	S2-2104395

Elbonia, 17 - 28 May, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	MBS session leave correction

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution updates the main procedure for MBS session leave.

# Background and Introduction

In SA2 #144E meeting, the basic call flow of MBS session leaving was agreed. This document do some corrections to the procedure.

# Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change* * * *

### 7.2.2	MBS leave and Session release procedure

#### 7.2.2.1	General

Session Leave procedure is used by UEs or Application to inform the 5GC of the UE leaving an MBS Session. The user plane management is described in clause 6.6.

#### 7.2.2.2	MBS session Leave

When the UE determines to leave the Multicast MBS Service, it shall send PDU session Modification request to inform the 5GC the leaving operation. The Fig 7.2.2.2-1 describes the procedure.

Figure 7.2.2.2-1: UE initiated multicast MBS session leave

1.	The UE sends the PDU Session Modification Request when the UE determine to leave the Multicast MBS Service. The PDU Session Modification Request carries the MBS session ID which the UE want to leave.

2.	The AMF invokes Nsmf_PDUSession_UpdateSMContext to SMF. The MBS session leaving information (i.e. leave indication, MBS session ID) is included.

3.	[conditional] If the UE is the last UE served with 5GC Individual MBS traffic delivery method in this SMF for this MBS session and the SMF is not collocated with the MB-SMF, the SMF configures the UPF to stop receiving multicast data from the MB-UPF and invokes Nmbsmf_MBSession_Update Request (MBS session ID) to release the tunnel between UPF and MB-UPF for this MBS session.

4.	[conditional] If the tunnel between UPF and MB-SMF shall be released, the MB-SMF request to MB-UPF to release the tunnel.

5.	[conditional] The MB-SMF responds to SMF for step 3.

6. [Optional] If individual delivery is applied, the SMF invokes an N4 Session Modification procedure with the UPF (PSA). The SMF reconfigures UPF to terminate the distribution of multicast data via the unicast PDU session and release the resources for the reception of the multicast data.

7.	The SMF invokes the Nsmf_PDUSession_UpdateSMContext Response (PDU Session ID, N2 SM information, N1 SM container) service operation. In the N2 SM information, the SMF informs the NG-RAN to remove the UE from this MBS session if 5GC Shared MBS traffic delivery method is used.

In the N2 SM information, the SMF also informs the NG-RAN to release the mapped unicast QoS Flow(s), which carry or intend to carry the Multicast MBS traffic for 5GC individual MBS traffic delivery.

The mapped QoS Flow(s) are released as defined in TS 23502 [6] clause 4.3.3.2.

8.	The AMF send N2 message (N2 SM information, N1 SM container (PDU Session Modification Command (MBS session ID)) to the NG-RAN

Editor’s Note: Whether MBS Session ID needs to be included is FFS.

9.	The NG-RAN performs the necessary AN-specific resource modification procedure toward the UE and transports the N1 SM container received in step 7 to the UE.

10.The NG-RAN remove the UE from this MBS session and sends a N2 message to the AMF.

11.The AMF transfers the N2 message received in step 9 to the SMF via the Nsmf_PDUSession_UpdateSMContext service operation.

The SMF removes the UE from the MBS Session context and removes the MBS Session ID from the associated UE SM context..

12.If the UE is the last UE in this RAN node for this MBS session, the NG-RAN release MBS session between NG-RAN and MB-UPF.

# * * * * End of changes * * * *