3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104701r02

Elbonia, May 17 – May 28, 2021	(revision of S2-2102828)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0325 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User consent revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of KI#15: User consent for UE data collection/analytics, the UDR (via UDM services) holds the user consent for user related data which is provisioned by MNO as a user subscription information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Description of user consent for data analytics - New procedure for data request for UE data collection subject to user consent - New procedure for analytics request subject to user consent - |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User data might be used for analytics purposes without having a possiblity to check if user consent for this type of data collection and processing is available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 2862 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of the changes (all new text) ***

### 6.2.X	User consent for data collection

#### 6.2.X.1	General

Based on local policy or regulations, to protect the privacy of user data, a user consent can be required before the data collection for some or all of these user data, which includes UE level data from UE or application or NFs.

It is network operator's responsibility to collect and manage the user consent information, it is stored in the UDR as subscription information.

NOTE:	Which data and data processing require prior user consent is not in the scope of 3GPP specification, due to the multitude of different legislations and regulatory environments in which 3GPP systems are being deployed, each with their own set of privacy and teleservices regulations.

When NWDAF needs to do data collection for some UE level data, e.g. triggered by UE data specific analytics, the NWDAF shall check "user consent for analytics" information from the UDM before the data collection.

Editor's Note:	Specification of user consent revocation is FFS based on the conclusions of TR 23.700-91 and alignment with SA3.

#### 6.2.X.2	Data request for UE data subject to user consent

Figure 6.2.X.2-1 depicts the procedure for checking user consent for UE data collection. The user consent for UE data collection is managed within the 5GC via subscription management procedures.

Figure 6.2.X.2-1: User consent for UE data collection

1.	NWDAF needs to collect UE related data. NWDAF determines an NF instance providing the required data, e.g., it may query UDM to discover the NF serving the UE.

2.	Based on local policy or regulations, NWDAF may request to UDM using Nudm_SDM_Get (key=SUPI, user consent for UE data collection for analytics) service operation whether user consent is available and valid for the given UE Id, and for the purpose of data collection to do data analytics related to specific analytic ID(s). This requires that UDM may retrieve this information from UDR by Nudr_DM_Query.

3.	UDM responds to the NWDAF with the user consent data for the requested UE data collection if available.

4.	If requested UE data is subject to user consent and user consent is available, the NWDAF subscribes to changes in the user consent for UE data collection information via Nudm_SDM_Subscribe (key=SUPI, user consent for UE data collection) service operation. UDM may subscribe to UDR by Nudr_DM_Subscribe.

NOTE 1:	If the subscription data is updated (e.g. to remove the user consent of a particular UE), the UDM notifies consumers subscribed to changes in the user consent for UE data collection for that UE. Based on operator policies and legal requirements, the consumers receiving such notification might need to delete any affected data and stop any active data collection no longer allowed by the user consent.

5a.	If user consent is available or no user consent is needed, NWDAF requests the data from the data source, e.g., as a data request or event subscription.

5b.	Data source provides the data, e.g. as Data response or as notifications related to the requested subscription.

### *** End of the changes ***