TSG SA2 Meeting #146E	S2-2105752r07

16 - 27 August 2021, Electronic meeting	(Revision of S2-210xxxx)

Source:	ZTE, LG Electronics, Nokia, Nokia Shanghai Bell, Samsung, Alibaba, AT&T, CATT ,China Telecom, China Unicom, Convida Wireless LLC, Intel, InterDigital, Lenovo, Matrixx, Motorola Mobility NEC, NTT Docomo, OPPO, Sanechips, T-Mobile USA, Spreadtrum, Tencent, Xiaomi

Title:	New SID: New SID on Network Slicing Phase 3

Document for:	Approval

Agenda Item:	9.1.3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on Enhancement of Network Slicing Phase 3

## Acronym: FS_eNS_Ph3

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_EASNS | SA1 | 880035 | Study on Enhanced Access to and Support of Network Slice |
| EASNS | SA1 | 910032 | Enhanced Access to and Support of Network Slice |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860039 | Study on enhancement of RAN slicing for NR | Enhancements for RAN slicing for NR were studied by RAN3 in Rel-17. The solutions with system impacts should be studied in SA2. |
| 911007 | Enhancement of RAN slicing for NR | The solutions in Enhancements for RAN slicing for NR can be taken into account by this study. |

## 3	Justification

RAN3 has studied the scenarios and candidate solutions in TR 38.832 on the support of slice service continuity during UE mobility due to no support of the network slice or resource limitation of the network slice in the target RAN node. SA2 sent LS out in S2-2102068 to confirm that the scenarios described in the TR are valid(i.e. they do happen, due to the fact that network slices are not required to be available in all TAs of a PLMN). However the solutions with CN/UE impacts were discouraged, and before being pursued would require an SA2 study from end to end system point of view. Deployment improvement should be analysed accordingly.

For disjointed network slice deployment the S-NSSAI in the Requested NSSAI or subscribed NSSAI may be supported by the existing TA but not supported by the serving AMF. If the rejection cause code is "not supported in the RA", the UE cannot attempt to register with the rejected S-NSSAI and reselects a different serving AMF till the UE moves out of the RA. This study should analyse the deployment improvement and investigate the solutions if existing mechanism is not sufficient.

There is a scenario that an S-NSSAI in the Requested NSSAI is rejected as it is not supported in a TA but it may be supported in nearby TAs. The AMF may need to include the nearby TAs in a Registration Area as it is optimal to do so for the S-NSSAIs in the Allowed NSSAIs. So, if this is done and the rejection cause code is "not supported in the RA", the UE cannot attempt to register with the rejected S-NSSAI until the UE moves out of the RA. If this is not desirable, then the AMF can only assign a RA that is limited to the current TA and TAs where the rejected S-NSSAI is not supported, which enable the UE to register with the rejected S-NSSAI when the UE moves into the nearby TA which supports the rejected S-NSSAI. This study should investigate the solutions to improve the ability to select network slices marked as "not supported in the RA" due to being unavailable in the TA, but which are available in other TAs of the RA, to be requestable by the UE without forcing the AMF to reduce the size of the RA

SA1 has agreed a new requirement: For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice. SA2 should investigate mechanisms to support this new requirement.

When the UE performs registration, the UE generates a Requested NSSAI based on the Configured S-NSSAI, Allowed NSSAI and also may take the URSP rules into account. The UE may register S-NSSAIs which are not going to be used at once, and possibly not while registered in this RA. This may be perceived by operator’s customers as an issue due to NSAC procedure introduced by eNS_Ph2. Similar considerations apply to PDU session establishment.This study should investigate the mechanism on how to ensure that the UE can form the Requested NSSAI for registration and Requested S-NSSAI for PDU session based on actual running applications, in addition to the Configured NSSAI and Allowed NSSAI, and how the UE can initial deregistration from the slice or release the PDU session after the last application using the slice stops using the slice or PDU session.

Today the AMF provides the UE with a TAI list but there is no standard way for the AMF and NSSF to be informed of the topology of the TAIs to base this TAI list on. This study should investigate whether and how the AMF can be provisioned with the topology of the TAIs in a more optimal way.

The network at times needs to make a choice among which slices to provide or continue providing for a UE (e.g. the target cell candidates for HO may support different slices). Also, the CN determines the RFSP index for the Allowed NSSAI or RFSP index for target S-NSSAI to redirect the UE to proper target cell. Priority information from the UE regarding some or all of the S-NSSAIs in the Requested NSSAI may be useful for network to determine the set of allowed network slices to select for the UE and possibly to determine the target of a Handover, the target NSSAI and also the target RFSP index. This study should investigate the solutions for providing prioritisation information (if any) regarding S-NSSAIs

In addition, services provided over certain network slices may be limited in Service area (not matching deployed TA boundaries) and/or in time-span of their deployment (e.g. services provided to cover some event). It would be desirable to define mechanisms to support deployments more optimally to minimize operations to enable such scenarios.

In the Rel-17, the UE triggered service request procedure has taken the requested NSSAI into account for the cell reselection, however in Rel-17, the paging procedure does not considered the slice information. This may cause problems that the UE provides improper S-NSSAI in paging response and the NG-RAN made wrong admission control for the network slice.

## 4	Objective

The objective of this study is to investigate the feasibility of further enhancement on network slicing.

For the following objectives, it is expected to study the necessity and potential mechanisms identified as gaps to our specifications:

1.	Study how the deployments can be improved for the following scenario: an existing slice cannot serve the PDU session in current cell (due to OAM reasons) or target cell (due to mobility), or a slice was not allowed due to NSAC, or if the existing slice cannot meet the performance requirements of the applications, If existing mechanisms are concluded to be not sufficient, study whether and how to (continue) support the session continuity and minimize the impact on applications, and study whether and how to expose this capability to third party.

2.	Study how networks deployments can be done using existing mechanisms, in order to avoid situations where not all subscribed and requested slices are supported by a single AMF ("Disjoint Network Slices"). If existing mechanisms are concluded to be not sufficient to achieve the scenarios, study whether and how additional mechanisms can resolve the analysed gap

3.	Study whether and how to enable a UE to initiate a registration for a rejected S-NSSAI that was rejected in a first TA of the RA but may be available in another TA of the RA.

4.	Study whether and how to enhance availability of slices in roaming scenarios to allow the UE to select and obtain services from one or multiple VPLMNs supporting the network slices which UE may wish to use when the UE is roaming according to these requirements in TS 22.261clause 6.1.2.1

5.	Study whether and how to enhance UE behaviour in registering and deregistering with network slices and establishing/releasing PDU sessions taking into account the running applications using the slice.

6.	Study whether and how to allow the AMF to be provisioned with TA topology information in a more optimal way (e.g. by using a control plane approach similarly to gathering the S-NSSAI support per TA)

7.	Study whether and how to support the provisioning of the priority information regarding (some of) the S-NSSAI, e.g. for service continuity decisions or Allowed NSSAI decisions

8.	Study deployment considerations for optimising the temporary deployment of services in an area, and how existing mechanisms including network slicing can help support such scenarios (e.g. the service supported by a network slice may have a limited lifetime or a time of day lifetime). If existing mechanisms are concluded to be not sufficient to achieve the scenarios, study whether and how additional mechanisms can resolve the analysed gap.

9	Study deployment considerations when a service provided by existing network slices has a Service Area that does not overlap with the already deployed Tracking Areas, and how existing mechanisms including network slicing can help support such scenarios. If existing mechanisms are concluded to be not sufficient to achieve the scenarios, study whether and how additional mechanisms can resolve the analysed gap.

10.	Study whether and how to enhance the paging procedure taking the slice information into account in order to ensure that the UE can provide proper Requested S-NSSAI in the Paging Response to the RAN node so the RAN node can enforce proper admission control for the slice.

11. Study how to support the identification by a human user of the slices available to the UE (pending completion of requirements in SA1)

The following bullet is overlapping with another SID proposal and can be considered if the other SID cannot be agreed.

x.	Study how to extend DCAMP mechanism to steer a UE to different radio resources considering ongoing traffic of each network slice.

Existing mechanisms shall be reused to the extent possible to resolve the gaps for the scenarios in the objectives above. No new mechanism will be introduced until it is demonstrated that existing mechanisms cannot fulfil the scenarios described in the objectives.

Depending on the deployment guidelines collected during the study, the study will conclude whether and how to document these deployment guidelines in our specifications.

Interaction with RAN working groups is needed on any RAN impact.

The estimated TU for this study is about 12 TU slots.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xxx | Study on Enhancement of Network Slicing Phase 3 |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

TBD

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

The following aspects may arise related to this WID:

-	Security aspects

-	Charging, OAM aspects

-	RAN aspects

## 9	Supporting Individual Members

Supporting IM name

Alibaba

Apple

AT&T

Broadcom ?

CATT

China Mobile ?

China Telecom

China Unicom

Cisco ?

Convida Wireless LLC

Deutsche Telekom ?

Ericsson?

ETRI ?

Huawei ?

Intel

InterDigital

KDDI

Lenovo

LG Electronics

Matrixx

MITRE

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

KT?

NTT Docomo

OPPO

Orange

Qualcomm

Samsung ?

Sanechips

SK Telecom?

T-Mobile USA

Spreadtrum

Telecom Italia ?

Telefonica S.A. ?

Tencent

Verizon UK Ltd

Vodafone?

Xiaomi

ZTE