3GPP TSG-SA2 Meeting # 147E (e-meeting)	 S2-2107196

# Elbonia, October 18 - 22, 2021											 (revision of S2-2105819)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3092 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add NF services for 5G MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA2#145E, 23.501 CR2858 (S2-2105129, Consistent handling of NF documentation) proposes the following principles: Clause 6.2 – This clause should be complete and include a clause for each 5G NFs that’s defined by SA2. For NFs that are defined in a separate SA2 specification the description may be a simple one line reference. Additional 5G entities defined by other groups are included by referencing the the relevant specifications in clause 6.1. Clause 7 – should include a table of services for each NF described in Clause 6.2. To have a consistent handling on the NF services in clause 6.2, it is proposed to update the services introduced for 5G MBS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new AMF services for broadcast MBS session and multicast MBS Session Adding new NEF services to handle 5G MBS Add new MB-SMF services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete list of NF services |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2, 7.2.8, 7.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

### 7.2.2	AMF Services

The following NF services are specified for AMF:

Table 7.2.2-1: NF Services provided by AMF

| Service Name | Description | Reference in TS 23.502 [3] or indicated other TS |
| --- | --- | --- |
| Namf_Communication | Enables an NF consumer to communicate with the UE and/or the AN through the AMF. This service enables SMF to request EBI allocation to support interworking with EPS. This service also supports PWS functionality as described in TS 23.041 [46]. | 5.2.2.2 |
| Namf_EventExposure | Enables other NF consumers to subscribe or get notified of the mobility related events and statistics. | 5.2.2.3 |
| Namf_MT | Enables an NF consumer to make sure UE is reachable. | 5.2.2.4 |
| Namf_Location | Enables an NF consumer to request location information for a target UE. | 5.2.2.5 |
| Namf_MBSBroadcast | Enables the NF consumer to communicate with the NG-RAN for broadcast communication | TS 23.247 [129] |
| Namf_MBSCommunication | Enables NF consumer to communicate with the NG-RAN for multicast communication | TS 23.247 [129] |

****************** NEXT CHANGES ***************

### 7.2.8	NEF Services

The following NF services are specified for NEF:

Table 7.2.8-1: NF Services provided by NEF

| Service Name | Description | Reference in TS 23.502 [3] or other TS |
| --- | --- | --- |
| Nnef_EventExposure | Provides support for event exposure. | 5.2.6.2 |
| Nnef_PFDManagement | Provides support for PFDs management. | 5.2.6.3 |
| Nnef_ParameterProvision | Provides support to provision information which can be used for the UE in 5GS. | 5.2.6.4 |
| Nnef_Trigger | Provides support for device triggering. | 5.2.6.5 |
| Nnef_BDTPNegotiation | Provides support for background data transfer policy negotiation and optionally notification for the renegotiation. | 5.2.6.6 |
| Nnef_TrafficInfluence | Provide the ability to influence traffic routing. | 5.2.6.7 |
| Nnef_ChargeableParty | Requests to become the chargeable party for a data session for a UE. | 5.2.6.8 |
| Nnef_AFsessionWithQoS | Requests the network to provide a specific QoS for an AS session. | 5.2.6.9 |
| Nnef_MSISDN-less_MO_SMS | Used by the NEF to send MSISDN-less MO SM to the AF. | 5.2.6.10 |
| Nnef_ServiceParameter | Provides support to provision service specific information. | 5.2.6.11 |
| Nnef_APISupportCapability | Provides support for awareness on availability or expected level of a service API. | 5.2.6.12 |
| Nnef_NIDDConfiguration | Used for configuring necessary information for data delivery via the NIDD API. | 5.2.6.13 |
| Nnef_NIDD | Used for NEF anchored MO and MT unstructured data transport. | 5.2.6.14 |
| Nnef_SMContext | Provides the capability to create, update or release the SMF-NEF Connection. | 5.2.6.15 |
| Nnef_AnalyticsExposure | Provides support for exposure of network analytics. | 5.2.6.16 |
| Nnef_UCMFProvisioning | Provides the ability to configure the UCMF with dictionary entries consisting of UE manufacturer-assigned UE Radio Capability IDs, the corresponding UE radio capabilities, the corresponding UE Radio Capability for Paging and the (list of) associated IMEI/TAC value(s) via the NEF. The UE radio capabilities the NEF provides for a UE radio Capability ID can be in TS 36.331 [51] format, TS 38.331 [28] format or both formats. Also used for deletion (e.g. as no longer used) or update (e.g. to add or remove a (list of) IMEI/TAC value(s) associated to an entry) of dictionary entries in the UCMF. | 5.2.6.17 |
| Nnef_ECRestriction | Provides support for queuing status of enhanced coverage restriction, or enable/disable enhanced coverage restriction per individual UEs. | 5.2.6.18 |
| Nnef_ApplyPolicy | Provides the capability to apply a previously negotiated Background Data Transfer Policy to a UE or a group of UEs. | 5.2.6.19 |
| Nnef_Location | Provides the capability to deliver UE location to AF. | 5.2.6.21 |
| Nnef_AMInfluence | Provides the ability to influence access and mobility management related policies for one or multiple UEs. | 5.2.6.22 |
| Nnef_AMPolicyAuthorization | Provides the ability to provide inputs that can be used by the PCF for deciding access and mobility management related policies. | 5.2.6.23 |
| Nnef_AKMA | AKMA Application Key derivation service. | TS 33.535 [124] |
| Nnef_SliceStatus | Provides network slice status information related to the number of the UEs registered with a network slice or the number of PDU Sessions established on a network slice. | 5.2.6.24 |
| Nnef_EASDeployment | EAS deployment service. | 5.2.6.26 |
| Nnef_MBSTMGI | Allows AF to request allocation/deallocation of TMGI(s) for MBS Session | TS 23.247 [129] |
| Nnef_MBSSession | Allows AF to create, update and delete MBS Session | TS 23.247 [129] |

### 7.2.x	MB-SMF Services

The following NF service is specified for MB-SMF.

Table 7.2.26-1: NF Services provided by MB-SMF

| Service Name | Description | Reference in TS 23.247 [129] |
| --- | --- | --- |
| Nmbsmf_TMGI | This service enables NF service consumer to allocate, refresh and deallocate TMGI(s). | 9.1.2 |
| Nmbsmf_MBSSession | This service enables the consumer to create, update and delete the MBS Session. | 9.1.3 |

****************** END CHANGES ***************