# 3GPP TSG-SA WG2 Meeting #147E	S2-2107135

Elbonia, 18 - 22 October 2021	(Revision of S2-210xxxx)

Title:	[Draft] Reply LS on Time Synchronization

Response to:	LS on Time Synchronization (S2-2107007)

Release:	Rel-17

Work Item:	IIoT

Source:	Ericsson [to be SA2]

To:	IEEE 1588 working group, SMPTE

Cc:	-

Contact person:	Shabnam Sultana

Shabnam.sultana@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA2 thanks IEEE 1588 WG for their response.

IEEE feedback:

- Re interoperability between 1588 editions:

The IEEE 1588 working group created IEEE Std 1588-2019 such as to be backward compatible with IEEE 1588-2008.  By that we mean that if you do not use the new optional features in the 2019 edition, then the old ones work the same as in 2008.   However, care should be taken to make sure that the same optional features are implemented to allow equipment to interoperate. See IEEE 1588-2019 clause 19 on compatibility between the 2019 and 2008 editions.

SA2 thanks for the recommendation and agreed to add clarifications in our specification TS 23.501 v17.3.0 and later, regarding the potential interoperability aspect.

IEEE feedback:

- Re interoperability issues with non-synchronizing BC:

The behavior you described is allowed in IEEE 1588-2019, if the other requirements for Boundary Clocks are met.  There could be potential interoperability issues between the SMPTE ST2059-2 profile devices and the Boundary Clock in 5GS as described in the Liaison statement.  We defer to SMPTE to identify interoperability issues with their profile.

SA2 thanks IEEE for the information.

IEEE feedback:

- Re SMPTE profile update:

We defer to SMPTE for any information on updates to their PTP profile.

SA2 requests for feedback on the following issues from SMPTE:

-According to the SMPTE profile the following parameters should be used and set accordingly:

- transparentClockDefaultDS.delayMechanism

- transparentClockDefaultDS.primaryDomain

However, according to IEEE 1588-2019, clause 19.4.5, the deprecated data sets for Transparent Clocks in the 2008 edition have been replaced by attributes in the defaultDS and portDS data sets. And, according to IEEE 1588-2019, clause 8.1.1.3, the transparent clock data sets should be maintained locally.

Q1: Is it still necessary to use and maintain the transparent clock data sets locally in order to support the SMPTE profile, and especially to allow for a common management?

-3GPP does not specify support for PTP management mechanism (Chapter 15, optional), however SMPTE specifies the following:

Q2: Is it possible to support SMPTE profile without implementing the PTP management mechanism?

-SA2 considers the following potential issues concerning support for SMPTE profile vs IEEE 1588 2008/2019:

    - Packet rate: if packet rate is low, synchronization messages may exceed the 1 s limit for difference with the Origin TimeStamp as required by the IEEE1588 standard– note that the 5GS node does not update Origin TimeStamp.

    - Specific rules to send out synchronization messages.

Q3: Does SMPTE have any guidance/feedback on the above concerns?

-Regarding options used in SMPTE profile:

Q4: Has SMPTE introduced any additional option after 2008 edition (which is not included in 2019) in the profile?

-Finally, concerning the end-to-end target accuracy (we understand within 1 microsecond with respect to the master reference):

Q5: What portion of the time error budget can be allocated to the 5GS?

Note, this may require taking actions (e.g., estimate the frequency error of the incoming PTP messages) to limit the impact due to frequency deviation between the SMPTE master (that may be 5ppm compared with the SI second) and the 5GS timing (typically traceable to UTC).

# 2	Actions

To SMPTE

ACTION:

SA2 kindly request feedback on the analysis and follow up questions so we can finalise the work in 3GPP.

# 3	Dates of next TSG SA WG 2 meetings

SA2#148e                   November 15-19, 2021                                    Electronic meeting

SA2#149e		 February 14 -25, 2022			   Electronic meeting