3GPP TSG-SA2 Meeting # 147E e-meeting	S2-2107172

# Elbonia, October 18th – 22nd, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving EN about transmission of Application layer discovery message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the LS R2-2109124, RAN2 answered SA2 question about the PC5 discovery message size: Q1) SA2 has studied the possibility to transmit metadata or application layer discovery information in the PC5 discovery message and realized that it depends on the PC5 discovery message size (as described in clause 5.2.4 of TS 23.304). SA2 would like to ask RAN2 whether there is any limitation on the size of NR PC5 discovery message as similar to LTE PC5 discovery message. [Answer]: For Rel-17 U2N sidelink relay, RAN2 agrees that only RLC UM mode can be used for sidelink discovery message and the maximum size of the discovery message should be 9000 bytes. However, large payload size and/or RLC segmentation may degrade latency and coverage performance of discovery. RAN2 recommend SA2 to assume a reasonable message size for NR PC5 discovery message. From the LS, we can see that the PC5 discovery message can have 9000 bytes, but still it is not recommended to have large discovery messages. Regarding the EN in clause 6.4.2 in TS 23.304 v 17.0.0, putting Application layer discovery message into PC5 direct discovery message may suffer performance degraqdation. Furthermore, application layer messages may need dedicated QoS treatment, using PC5 discovery message to convey the Application layer discovery message cannot fullfill the QoS requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN in clause 6.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN in clause 6.4.2 is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

### 6.4.2	Groupcast mode 5G ProSe Direct Communication

Figure 6.4.2-1 shows the procedure for groupcast mode of 5G ProSe Direct Communication.

Figure 6.4.2-1: Procedure for groupcast mode 5G ProSe Direct communication

Steps 1 to 3 are optional, e.g. applicable for Application Layer managed group. Groupcast mode ProSe Direct communication can be performed without steps 1 to 3 based on the provisioned parameters as defined in clause 5.1.3.1.

1.	ProSe Group management is carried out at ProSe application layer. This may be performed in coordination with ProSe Application Server.

Group Member Discovery parameters as specified in clause 5.1.2.1 can be provisioned in ME from PCF or configured in UICC or provided from ProSe Application Server to the UE.

2.	Each UE may perform a ProSe group member discovery (similar to the procedures in clause 6.3.2.2) that is restricted only to users sharing the same Application Layer Group ID obtained in step 1.

The Application layer discovery messages including the Application Layer Group ID for Application Layer managed group may be exchanged between UEs to discover each other sharing the same Application Layer Group ID. When the group is formed, the Application layer discovery message including group size and member ID may be provided by one UE to all other UEs. The Application layer discovery messages are treated as the groupcast user traffic.

3.	The ProSe Application layer may provide group identifier information (i.e. the Application Layer Group ID discovered based on discovery messages exchanged in step 2) as specified in clause 5.8.2.3.

The ProSe application layer may also provide ProSe Application Requirements for this communication.

If the ProSe application layer does not provide ProSe Application Requirements, the ProSe layer determines the PC5 QoS parameters based on the mapping of ProSe service to PC5 QoS parameters as specified in clause 5.1.3.1.

The ProSe application layer may provide a group size and a member ID (optionally based on discovery messages exchanged in step 2) as specified in clause 5.3.3 for Application Layer managed group.

4.	Transmitting UE self-assigns a source Layer-2 ID and determines a destination Layer-2 ID and Receiving UE(s) determine destination Layer-2 ID as specified in clauses 5.8.2.1 and 5.8.2.3.

Transmitting UE determines the PC5 QoS parameters for this groupcast mode communication as specified in clause 5.6.1.

If the group size and the member ID for Application Layer managed group are provided by the ProSe application layer, the ProSe layer passes them to the AS layer as described in clause 5.3.3.

5.	Transmitting UE sends the ProSe data using the source Layer-2 ID and the destination Layer-2 ID.

NOTE 2:	In step 5, there is only one groupcast message from the transmitting UE, that is, all receiving UEs can receive the groupcast message directly from the transmitting UE.

* * * End of Changes * * * *