SA WG2 Meeting #147E e-meeting	S2-2107300

# 18 – 22 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3275 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time synchronization - description review |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Minor changes to correct time synchronization clauses - Considering the LS response from IEEE in S2-2105362, IEEE Std 1588-2019 should be backward compatible with IEEE 1588-2008. The implementation should make sure that the same optional features between different editions are implemented to allow equipment to interoperate, and the IEEE Std 1588-2019 clause 19 explains the compatibility between IEEE PTP 2019 and 2008 versions. Therefore the open EN regarding the support for external networks operating with IEEE Std 1588-2008 can be closed adding the description that the operation of 5GS PTP instances following different IEEE Std 1588 editions should follow the recommendations of IEEE Std 1588 to ensure interoperation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Figure 4.4.8.3-1 is updated with the TSCTSF reference point numbers - Close open EN in clause 5.27.1.4 adding a note for interoperability between different IEEE Std 1588 editions - Minor text corrections for clauses 5.27.1.8 and 5.27.1.9, to align with TS 23.502 - In 6.3.24 clarify that TSCTSF deletes the TSCTSF NF ID and the Notification Target Address from the UDR upon reception of cancellation for the subscription for UE availability for time synchronization service, or once the PDU Sessions are released, whichever occurs first. - TSC TSF discovery section – to clarify that the GPSI, SUPI, Internal Group ID can also be used for discovery and selection. - Add GPSI – in case the NF consumer uses GPSI for TSC TSF discovery and selection. - Add SUPI or Internal Group ID – in case the NF consumer is only aware of the SUPI or Internal Group ID, it uses SUPI or Internal Group ID for TSC TSF discovery and selection. These parameters are in principle available already with NRF framework thus it should be straight forward to use them also for TSC TSF discovery selection. - Add e2e Transparent Clock to the scope of annex H.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8.3, 5.27.1.4, 5.27.1.8, 6.3.24, H.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

#### 4.4.8.3	Architecture for AF requested support of Time Sensitive Communication and Time Synchronization

This clause describes the architecture to enable Time Sensitive Communication AF requested time sensitive communication and time synchronization services. The Time Sensitive Communication and Time Synchronization related features that are supported based on AF request are described in clauses 5.27.1 and 5.27.2, respectively. Figure 4.4.8.3-1 shows the architecture to support Time Sensitive Communication and Time Synchronization services.

As shown in Figure 4.4.8.3-1, to support Time Synchronization service based on IEEE Std 802.1AS [104] or IEEE Std 1588 [126] for Ethernet or IP type PDU Sessions, the DS-TT, NW-TT and Time Sensitive Communication and Time Synchronization function (TSCTSF) are required in order to support the features in IEEE Std 802.1AS [104] or IEEE Std 1588 [126] as described in clause 5.27. The NEF exposes 5GS capability to support Time Synchronization service as described in clause 5.27.1.8. TSCTSF controls the DS-TT(s) and NW-TT for the (g)PTP based time synchronization service. In addition, TSCTSF supports TSC assistance container related functionalities.

Figure 4.4.8.3-1: Architecture to enable Time Sensitive Communication and Time Synchronization services

## ---Next Change---

#### 5.27.1.4	DS-TT and NW-TT Time Synchronization functionality

This clause describes the support of Time Synchronization functionality supported by the 5G System. Synchronization between UPF/NW-TT and NG-RAN is outside scope of 3GPP.

DS-TT and NW-TT may support the following PTP instance types:

-	Boundary Clock as defined in IEEE Std 1588 [126] as described in clause 5.27.1.1;

-	End-to-End Transparent Clock as defined in IEEE Std 1588 [126] as described in clause 5.27.1.1;

-	Peer-to-Peer Transparent Clock as defined in IEEE Std 1588 [126] as described in clause 5.27.1.1;

-	PTP Relay instance as defined in IEEE Std 802.1AS [104].

NOTE: 	Interoperability between configured 5GS PTP instances conformant to different IEEE Std 1588 editions should follow the recomendations of IEEE Std 1588 [126].

DS-TT and NW-TT may support the following transports for PTP:

-	IPv4 as defined in IEEE Std 1588 [126] Annex C;

-	IPv6 as defined in IEEE Std 1588 [126] Annex D;

IEEE Std 802.3 [131] (Ethernet) as defined in IEEE Std 1588 [126] Annex E.

For operation as a Boundary clock or as a Transparent Clock, DS-TT and NW-TT may support the following path and link delay measurement methods:

-	Delay request-response mechanism as described in clause 11.3 of IEEE Std 1588 [126];

-	Peer-to-peer delay mechanism as defined in clause 11.4 of IEEE Std 1588 [126].

DS-TT and NW-TT may support acting as a PTP grandmaster, i.e. may support generating (g)PTP Announce, Sync and Follow_Up messages. DS-TT and NW-TT supporting (g)PTP shall support one or more PTP profiles as described in clause 20.3 of IEEE Std 1588 [126], i.e.:

-	Default PTP Profiles in IEEE Std 1588 [126], Annex I;

-	IEEE Std 802.1AS [104] PTP profile for transport of timing as defined in IEEE Std 802.1AS [104] Annex F;

-	SMPTE Profile for Use of IEEE Std 1588 [126] Precision Time Protocol in Professional Broadcast Applications ST 2059-2:2015 [127].

TSN AF and TSCTSF may determine the PTP functionalities supported by DS-TT and NW-TT and may configure PTP instances in DS-TT and NW-TT using port and user plane node management information exchange as described in Annex K, clause K.2.

## ---Next Change---

#### 5.27.1.8	Exposure of Time Synchronization

5G System supports time synchronization service that can be activated and deactivated by AF. Exposure of time synchronization comprises the following capabilities:

-	The AF may learn 5GS and/or UE availability and capabilities for time synchronization service.

-	The AF may request to control the time synchronization service with specific service parameters for targeted UE(s). The AF controls activation and deactivation of the time synchronization service for the target UE(s).

-	The AF may request to control the 5G access stratum as a time distribution method for targeted UE(s).

The AF may subscribe for 5GS and/or UE availability and capabilities for time synchronization service. The AF indicates in the request the DNN, S-NSSAI, and in addition the AF may indicate a list of UE identities or group identity to limit the subscription only to corresponding UEs. If the AF does not indicate DNN, S-NSSAI, the NEF determines the DNN, S-NSSAI based on the AF Identifier. The discovery of the TSCTSF is described in clause 6.3.24.

The TSCTSF (directly or via NEF) exposes the 5GS and/or UE availability and capabilities for synchronization service to the AF. The exposed information includes the list of user plane node identities, the list of UE identities and may include the supported   capabilities for (g)PTP time synchronization service per user plane node and UE, as described in clause 4.15.9.2 of TS 23.502 [3].

The AF request to control the (g)PTP time synchronization service is sent to the TSCTSF (directly or via NEF). The request is targeted to a set of AF-sessions that are associated with the exposure of UE availability and capabilities for synchronization service.

The AF request includes requested time synchronization distribution method (IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation (i.e. as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock or as a PTP relay instance)). The request to control the (g)PTP time synchronization service may contain other service parameters as described in clause 4.15.9.3 of TS 23.502 [3].

The AF may request to use the 5G access stratum as a time synchronization distribution method. In this case, the time source is provided by the 5GS. 5G-AN provides the 5GS time to the UE via 3GPP radio access; UE/DS-TT may provide 5G access stratum timing information to end stations using implementation specific means. This method is applicable for both IP and Ethernet PDU Sessions. The request to control the 5G access stratum time distribution is described in clause 4.15.9.4 of TS 23.502 [3].

The AF or NEF selects the TSCTSF as specified in clause 6.3.24.

The AF request may include a time synchronization error budget (see also clause 5.27.1.9). The time synchronization error budget defines an upper bound for time synchronization errors introduced by 5GS when the NG-RAN provides 5G access stratum timing information to a UE.

The TSCTSF uses the Time Synchronization parameters received from AF (directly or via NEF) to control the time synchronization service. When IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation have been selected, the TSCTSF determines the necessary (g)PTP parameters to activate and control the service in DS-TT(s) and NW-TTs. For this purpose, the TSCTSF uses the PMIC or UMIC to manage the IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation in the DS-TT(s) or NW-TTs, respectively (see clause 5.27.1.4).

For handling (g)PTP traffic, the PCF, according to PCC rule authorization, chooses a 5QI and dynamically set the PDB and/or MDBV according to requirements for (g)PTP protocol. The PCF provides the SMF with a PCC rule generated based on the AF request to control the (g)PTP time synchronization service. The SMF may take the information in the PCC rule to modify a PDU Session to create or modify or release a QoS flow for transmitting the (g)PTP messages. The PCF acknowledges the policy request to the TSCTSF. The TSCTSF may report the result of the time synchronization request to the AF (directly or via NEF).

The AF may provide a temporal validity condition to the TSCTSF (directly or via NEF) when the AF activates the time synchronization service. Temporal validity condition contains the start-time and stop-time (in absolute time value) attributes that describe the time period when the time synchronization service is active for the targeted AF sessions. The TSCTSF manages the temporal validity condition as described in clause 4.15.9.3 TS 23.502 [3]

## ---Next Change---

### 6.3.24	TSCTSF Discovery

The NF consumers (e.g. NEF, AF) may utilize the NRF to discover TSCTSF instance(s) unless TSCTSF information is available by other means, e.g. locally configured in NF consumers. The NRF provides NF profile(s) of TSCTSF instance(s) to the NF consumers.

The following factors may be considered for TSCTSF selection:

-	DNN(s) and S-NSSAI(s).

-	GPSI or External Group Identifier;

-	SUPI or Internal Group ID.

If the TSCTSF is locally configured in NF consumers, it shall be ensured that the same TSCTSF is selected in all NF consumers (e.g. NEF, AF) for the given DNN and S-NSSAI. The same TSCTSF shall be configured also to PCF serving PDU Sessions for this DNN and S-NSSAI to notify the TSCTSF for a PDU Session.

If the TSCTSF is discovered from NRF, the discovered and selected TSCTSF shall store the TSCTSF NF ID and Notification Target Address to the UDR for the given UE’s SUPI or DNN and S-NSSAI. The NF consumer shall attempt to retrieve the TSCTSF NF ID for the given UE’s SUPI or DNN/S-NSSAI from the UDR. If the UDR does not contain a TSCTSF NF ID, the NF consumer shall discover the TSCTSF from the NRF. If the UDR contains a TSCTSF NF ID, the NF consumer shall use this TSCTSF NF ID to invoke the TSCTSF for the given DNN and S-NSSAI. The PCF serving the UE shall subscribe for notification of Time-Sync data change with the UDR for the given SUPI. The PCF serving PDU Sessions for the given DNN and S-NSSAI shall subscribe for notification of Time-Sync data change with the UDR, and use the Notification Target Address as received from the UDR to notify the TSCTSF for a PDU Session. The TSCTSF deletes the TSCTSF NF ID and the Notification Target Address from the UDR after all the PDU Session(s) for the given DNN and S-NSSAI are released or TSCTSF receives a cancellation for the subscription for UE availability for time synchronization service, whichever occurs first.

## ---Next Change---

# H.4	Path and Link delay measurements

The procedure described in this clause is applicable if DS-TT and NW-TT support operating as a boundary clock or as a time-aware system or as peer to peer Transparent Clock or end to end Transparent Clock, and when the PTP instance in 5GS is configured to operate as a time-aware system or as a Boundary Clock or as peer to peer Transparent Clock or as end to end Transparent Clock. Whether DS-TT/NW-TT support operating as a boundary clock or peer to peer Transparent Clock or end to end Transparent Clock or as a time-aware system (support of the IEEE Std 802.1AS [104] PTP profile) may be determined as described in clause K.2.1.

PTP ports in DS-TT and NW-TT may support the following delay measurement mechanisms:

-	Delay request-response mechanism as described in clause 11.3 of IEEE Std 1588 [126];

-	Peer-to-peer delay mechanism as defined in clause 11.4 of IEEE Std 1588 [126];

-	Common Mean Link Delay Service.

Depending on the measurement mechanisms supported by DS-TT and NW-TT as well as the configured clock mode of 5GS, the PTP ports in DS-TT and NW-TT are configured as follows:

-	PTP ports configured to operate as a time-aware system according to IEEE Std 802.1AS [104] may be configured to use the peer-to-peer delay mechanism or Common Mean Link Delay Service;

-	PTP ports configured to operate as a Boundary Clock according to IEEE Std 1588 [126] may be configured to use the delay request-response mechanism, the peer-to-peer delay mechanism or Common Mean Link Delay Service.

PTP ports in 5GS configured to operate as a peer-to-peer Transparent Clock according to IEEE Std 1588 [126] shall use the peer-to-peer delay mechanism.

PTP ports in 5GS configured to operate as an end-to-end Transparent Clock according to IEEE Std 1588 [126] do not actively participate in path and link measurements mechanisms but shall calculate and add residence time and delay asymmetry information to PTP messages as defined in clause 10.2.2 of IEEE Std 1588 [126].

If DS-TT and NW-TT support operating as an end-to-end Transparent Clock, then the residence time for one-step operation as an end-to-end Transparent Clock is calculated as follows:

-	Upon reception of a PTP Delay_Req message from the upstream PTP instance, the ingress TT (i.e. NW-TT or DS-TT) makes an ingress timestamping (TSi) for the message.

-	The ingress timestamp is conveyed to the egress TT via the PDU Session as described in clause H.2.

-	The PTP port in the egress TT then creates egress timestamping (TSe) for the PTP message for external PTP network. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this PTP message expressed in 5GS time.

-	The PTP port in the egress TT modifies the payload of the PTP Delay_Req message that it sends towards the downstream PTP instance as follows:

-	Adds the calculated residence time to the correction field.

-	Removes Suffix field that contains TSi.

If DS-TT and NW-TT support operating as an end-to-end Transparent Clock, then the residence time for two-step operation as an end-to-end Transparent Clock is calculated as follows:

-	Upon reception of a PTP Delay_Req message from the upstream PTP instance, the ingress TT (i.e. NW-TT or DS-TT) makes an ingress timestamping (TSi) for the message.

-	The ingress timestamp is conveyed to the egress TT via the PDU Session as described in clause H.2.

-	The PTP port in the egress TT then creates egress timestamping (TSe) for the PTP message for external PTP network. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this PTP message expressed in 5GS time. The egress TT then stores the calculated residence time and removes Suffix field that contains Tsi before sending the PTP Delay_Req message towards the downstream PTP instance.

-	Upon reception of the PTP Delay_Resp message associated with the PTP Delay_Req, the egress TT for the PTP Delay_Req message (i.e. the ingress TT for the PTP Delay_Resp message) modifies the payload of the PTP Delay_Resp message that it sends towards the ingress TT of the PTP Delay_Req message (i.e. egress TT for the PTP Delay_Resp message) as follows:

-	Adds the (previously stored) calculated residence time to the correction field.

## ---End of Changes---