3GPP TSG-SA2 Meeting #147-e	S2-2107325

# Online, , 18th Oct 2021 - 22nd Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0013 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing the EN of policy control for L3 U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current PCC framework allows dynamic policy control for different PDU session types including IPv4, IPv6, Ethernet, unstructure etc. when Remote UEs accessing to 5GC via a L3 U2N Relay, PCF does not require to be aware of the Remote UEs, but it can get the service information with SDF filter specific for a certain Remote UE from the AF and generate the corresponding PCC rule. Since Remote UE uses an address belonging to the Relay UE's PDU session, the session binding could be based on the Relay UE’s address, and different Remote UEs’ traffic could be differentiated by the SDF filters in the PCC rule. thus, the EN in clause 5.4.1.3 can be removed, as the current PCC framework already support the dynamic policy control for L3 U2N relay with IPv4 and IPv6 PDU session type. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN in clause 5.4.1.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s not clear how to support the dynamic policy control for L3 U2N relay with IPv4 and IPv6 PDU session type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

5.4.1.3	Policy control and session binding to support 5G ProSe Layer-3 UE-to-Network Relay without N3IWF

The following functionalities may be provided to enable support for policy control for 5G ProSe Layer-3 Remote UEs accessing to 5GC via a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF:

-	The 5G ProSe Layer-3 Remote UEs may be assigned a /64 IPv6 Prefix from a shorter IPv6 prefix by the 5G ProSe Layer-3 UE-to-Network Relay.

-	The 5G ProSe Layer-3 UE-to-Network Relay, the SMF and PCF may be configured with a dedicated DNN for UE-to-Network Relay connectivity.

NOTE 1:	If Local Breakout configuration is supported for relay connectivity, the dedicated DNN needs to be well-known DNN to allow seamless operation across various operators' networks.

-	The AF discovers the PCF serving the 5G ProSe Layer-3 UE-to-Network Relay PDU Session as specified in clause 6.1.1.2 of TS 23.503 [9].

-	The PCF may validate any 5G ProSe Layer-3 Remote UE related service information from the AF based on roaming agreement and the dedicated DNN for UE-to-Network Relay functionality.

Session binding defined in clause 6.1.3.2.2 of TS 23.503 [9] applies with the following addition to enable session binding for 5G ProSe Layer-3 UE-to-Network Relay without N3IWF:

-	For a PDU Session to the dedicated DNN for 5G ProSe Layer-3 UE-to-Network Relay connectivity and using IPv6 prefix delegation (i.e. the assigned IPv6 network prefix is shorter than 64), the PCF shall perform session binding based on the IPv6 network prefix only. A successful session binding occurs whenever a longer prefix received from an AF matches the prefix value of the PDU Session. PCF shall not use the UE identity for session binding for this PDU Session.

NOTE 2:	For 5G ProSe Layer-3 UE-to-Network Relay connectivity, the UE identity that the SMF has provided (i.e. 5G ProSe Layer-3 UE-to-Network Relay Identity) and a UE identity provided by the AF (i.e. 5G ProSe Layer-3 Remote UE Identity) can be different, while the binding with the PDU Session is valid.

* * * * End of Changes * * * *