SA WG2 Meeting #S2-147E	S2-2107346

18 - 22 October, 2021, Electronic meeting	(revision of S2-21yyxxxx)

# Source:	Sony, Samsung, Convida Wireless, InterDigital, NEC

# Title:	New SID on Study of Enhanced System enablers for Multi-USIM

# Document for:	Approval

# Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Enhanced System enablers for Multi-USIM devices

Acronym: FS_eMUSIM

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x | x |  |
| No | x |  |  |  |  |
| Don't know |  |  |  |  | x |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| x | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820040 | Stage 1 of MUSIM | Rel-17 Work item on Support for Multi-USIM Devices in SA1. |
| 840049 | (Stage 2 of) System enablers for Multi-USIM devices | Rel-17 Work item to enable basic system support for MUSIM operation |
| 860063 | Support for Multi-SIM devices for LTE/NR | Rel-17 Work item on Support for enhanced operation of Multi-SIM devices in RAN |

Dependency on non-3GPP (draft) specification:

# 3	Justification

In Rel-17 system enablers for Multi-USIM (MUSIM) devices were introduced for both 5GS and EPS. The enablers were MUSIM Capability Exchange, Paging Cause, Paging Reception, Reject Paging, UE Based Paging Restriction List and Coordinated Leaving. The work was done for single Rx / single Tx and dual Rx / single Tx UE implementations and focused on situations when the UE was in idle-mode with all USIMs, or when the UE was actively communicating with one USIM and the UE needed to leave this connection to perform idle-mode tasks or start to communicate using another USIM. Efforts focused on reducing service interruption, minimizing signalling and resource utilization, but at the same time securing that the network(s) had control of the UE and could reach it if needed, that is, the UE should not just disappear from the network without notifying the network.

Ideally if the UE is actively communicating using one USIM and the UE for some reason would like to start communication with another USIM, the UE should not need to leave/stop the communication on one USIM in order to start communicating on the other USIM. Simultaneous communication with two or more USIMs was not studied in Rel-17. However, related to the Leaving feature it was discussed that it may not be easy for the UE to decide which USIM/Service should get priority to communicate. If it would be possible to allow more than one USIM to communicate simultaneous the problem with prioritization would be reduced or even removed. This may even reduce the signalling further as the UE does not need to leave the connection and start it again later to resume the service.

Possible further signalling and operation optimizations may be explored, some examples were discussed during the Rel-17 study e.g., intra network UE coordination. The objectives in Rel-17 focused on basic enablers and MUSIM features, implying that some of these optimizations was not pursued during the normative phase.

# 4	Objective

The aim of this study work is to investigate and identify potential system level enablers for 5GS and EPS to support enhanced MUSIM operation. The study only includes Multi-USIM device which has the same UE radio implementation limitation as in the rel-17 MUSIM work item.

Specifically, the objectives include:

- 	[WT#1] Study system enablers that would allow more than one USIM to communicate simultaneous with the same or multiple networks. The simultaneous communication on multiple USIM is expected to use a time division (TD) scheme, due to UE radio limitations.

-	[WT#1.1] Study necessary signalling, e.g. UE to CN, CN to RAN and UE to RAN (out of scope for SA2 but may be important from overall system operation); including support of intra network coordination if Multi-USIM device uses USIMs with same operator.

-	[WT#1.2] Whether and how a network takes the TD scheme in consideration when establishing new QoS Flow(s) or modifies QoS Flow(s).

- 	[WT2] Revisit signalling and operation optimizations discussed during the rel-17 study, but judged to be out of scope for rel-17 normative work:

-	support of intra network UE coordination.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or is depended on completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 | 3 TU | 1.5 TU | Yes | N/A |
| WT#1.1 | 2 | 1 | Yes | Self-contained |
| WT#1.2 | 1 | 0.5 | No | WT1.2 has dependency on WT1.1 |
| WT#2 | 0.5 TU | 1 TU | No | Self-contained |

Total TU estimates for the study phase: 3.5

Total TU estimates for the normative phase: 2.5

Total TU estimates: 3.5 + 2.5 = 6

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XXX | Study on Enhanced Multi-USIM | TSG#96 (June 2022) | TSG#97 (Sep 2022) | Nord Lars, Sony lars.nord@sony.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

Nord Lars, Sony

lars.nord@sony.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

RAN working group needs to be involved to specify the simultaneous communication scheme WT1 (Rel-18 RAN Work package coordination)

No RAN dependency expected for WT2.

SA1: Requirement alignment may be needed.

# 9	Supporting Individual Members

Supporting IM name

Sony

Samsung

Convida Wireless

InterDigital

NEC