3GPP TSG-WG SA2 Meeting #147E e-meeting 	S2-2107348

# Elbonia, Oct. 18– 22, 2021                                     						        (was S2-2105945)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3113 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Principle for Handling of PMIC exchange with UE(s) in idle mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following EN was added during the During the SA2#145E. Editor's note: Handling of PMIC exchange with UE(s) in idle mode is FFS. Normally, PMIC is delivered via NAS to a UE in idle mode with paging. However, all the UEs with a DNN/S-NSSAI may be affected by a Synchronization Activation/Modification/Deactivation request from an AF/TSCTSF. It may cause delays and congestions to the other UEs in the same paging zone, as all the paging resources of the paging zone can be comsumed by the Synchronization Activation/Modification/Deactivation request from the AF/TSCTSF. During tha SA2#146E, the following is added to 5.23 of 23.501. - If the originating NF does not require immediate delivery, it may indicate that the AMF is allowed to use ATC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TSCTSF may indicate that the AMF is allowed to use ATC when it sends PMIC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will have ambiguity handling of PMIC exchange with UE(s) in idle mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

5.27.1.8	Exposure of Time Synchronization

5G System supports time synchronization service that can be activated and deactivated by AF. Exposure of time synchronization comprises the following capabilities:

-	The AF may learn 5GS and/or UE availability and capabilities for time synchronization service.

-	The AF may request to control the time synchronization service with specific service parameters for targeted UE(s). The AF controls activation and deactivation of the time synchronization service for the target UE(s).

-	The AF may request to control the 5G access stratum as a time distribution method for targeted UE(s).

The AF may subscribe for 5GS and/or UE availability and capabilities for time synchronization service. The AF indicates in the request the DNN, S-NSSAI, and in addition the AF may indicate a list of UE identities or group identity to limit the subscription only to corresponding UEs. If the AF does not indicate DNN, S-NSSAI, the NEF determines the DNN, S-NSSAI based on the AF Identifier.

The TSCTSF (directly or via NEF) exposes the 5GS and/or UE availability and capabilities for synchronization service to the AF. The exposed information includes the list of UE identities and may include the supported time synchronization distribution methods (i.e. supported IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation modes or 5G clock sync), (g)PTP grandmaster capable, 5G Clock quality. The TSCTSF (directly or via NEF) exposure of UE availability and capabilities for (g)PTP time synchronization service are described in clause 4.15.9.2 of TS 23.502 [3].

The AF request to control the (g)PTP time synchronization service is sent to the TSCTSF (directly or via NEF) and may target a UE or multiple UEs. The request may be targeted to: Individual UEs identified using GPSI, Group of UEs identified using External Group Identity, or an IP address/Prefix or a MAC address, or any UE accessing the combination of DNN, S-NSSAI.

The AF request includes requested time synchronization distribution method (IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation (i.e. as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock or as a PTP relay instance)). The request to control the (g)PTP time synchronization service may contain other service parameters as described in clause 4.15.9.3 of TS 23.502 [3].

The AF may request to use the 5G access stratum as a time synchronization distribution method. In this case, the time source is provided by the 5GS. 5G-AN provides the 5GS time to the UE via 3GPP radio access; UE/DS-TT may provide 5G access stratum timing information to end stations using implementation specific means. This method is applicable for both IP and Ethernet PDU Sessions. The request to control the 5G access stratum time distribution is described in clause 4.15.9.4 of TS 23.502 [3].

The AF or NEF selects the TSCTSF as specified in clause 6.3.24.

The AF request may include a time synchronization error budget (see also clause 5.28.1.9). The time synchronization error budget defines an upper bound for time synchronization errors introduced by 5GS, when the NG-RAN provides 5G access stratum time to a UE.

The TSCTSF uses the Time Synchronization parameters received from AF (directly or via NEF) to control the time synchronization service. When IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation have been selected, the TSCTSF determines the necessary (g)PTP parameters to activate and control the service in DS-TT(s) and NW-TTs. For this purpose, the TSCTSF uses the PMIC or UMIC to manage the IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation in the DS-TT(s) or NW-TTs, respectively (see clause 5.27.1.4).

The TSCTSF may indicate that the AMF is allowed to use ATC when it sends PMIC to the PCF. In that case, the PCF also includes the indication that the AMF is allowed to use ATC when it sends PMIC to the SMF. Then, the SMF indicate that the AMF is allowed to use ATC when it sends PMIC to the AMF. If the UE CM state in the AMF is CM-IDLE state, the AMF may updates and stores the UE context based on the received message without paging UE immediately. When the UE CM state in the AMF enters CM-CONNECTED state, the AMF forwards N1 and N2 message to synchronize the UE context with the (R)AN and/or the UE (see clause 5.23).

The TSCTSF stores the Notification Target Address in the UDR for the combination of DNN and S-NSSAI until the target UE establishes the PDU Session to this DNN/S-NSSAI. The PCF can retrieve the Notification Target Address from the UDR based on the DNN/S-NSSAI. The PCF notifies the TSCTSF for the PDU session establishment using the Notification Target Address as received from the UDR. The TSCTSF deletes the Notification Target Address from the UDR when it completes a time synchronization deactivation request for this DNN/S-NSSAI.

For handling (g)PTP traffic, the PCF, according to PCC rule authorization, chooses a 5QI and dynamically set the PDB and/or MDBV according to requirements for (g)PTP protocol. The PCF provides the SMF with a PCC rule generated based on the AF request to control the (g)PTP time synchronization service. The SMF may take the information in the PCC rule to modify a PDU Session to create or modify or release a QoS flow for transmitting the (g)PTP messages. The PCF acknowledges the policy request to the TSCTSF. The TSCTSF may report the result of the time synchronization request to the AF (directly or via NEF).

The AF may provide a temporal validity condition to the TSCTSF (directly or via NEF) when the AF activates the time synchronization service. Temporal validity condition contains the start-time and stop-time (in absolute time value) attributes that describe the time period when the time synchronization service is active for the targeted AF sessions. If the start-time and/or the stop-time of the Temporal Validity Condition is in the future, the TSCTSF maintains the start-time and stop-time for the time synchronization service for the corresponding AF Session(s). If the start-time is in the past, the TSCTSF treats the AF request as if the time synchronization service was activated immediately. When the start-time or stop-time is reached, the TSCTSF sends PMIC/UMIC signalling to configure the (g)PTP functionality in DS-TT(s) and NW-TT for activating or deactivating time synchronization service to the corresponding AF Session(s), and enables or disables the 5GS reference time distribution to the respective UEs (as AF requested time synchronization distribution method described in this clause).

***** END OF CHANGES *****