3GPP TSG-SA2 Meeting #147E (e-meeting)	S2-2107392

# Elbonia, October 18 - 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0020 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifiers for Layer-2 UE-to-Network Relay discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are two Editor's notes left for Layer-2 UE-to-Network Relay discovery identifiers in clause 5.8.3.3. 1. EN on additional identifiers for Layer-2 relay Editor's note: It is FFS what identifiers are used in addition to the identifiers specified in clause 5.8.3.1 for Layer-2 UE-to-Network Relay use case. RAN2 discussed AS criteria for relay (re)selection and made the following agreements at RAN2#113bis-e and RAN2#114-e meetings Agreements: Proposal 2-2: For L2 relay, PLMN ID is supported as additional AS criteria for relay (re)selection. Whether cell ID is used can be further discussed by RAN2. Proposal 16: Include the information required for agreed additional AS criteria in discovery message. Agreements: Proposal 4: For L2 U2N relay, cell ID can be used as additional AS criteria for relay (re)selection… It can be seen from the above agreements that for Layer-2 UE-to-Network Relay, PLMN ID and cell ID are used as additional AS criteria for relay (re)selection and are required to be included in the discovery message, and the EN can be removed. 2. EN on RSC associated information for Layer-2 relay Editor's note: It is FFS with what information the RSC is associated for Layer-2 UE-to-Network Relay use case. The Relay Service Code is defined in clause 5.8.3.1 and used as common identifier for both Layer-2 and Layer-2 relays. Besides this there is no further information needed to be associated with the RSC, and the EN can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PLMN ID and NCI into Layer-2 UE-to-Network Relay discovery message and remove the EN. Remove the EN about RSC associated information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The additional AS criteria for relay (re)selection using PLMN ID and NCI not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.8.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE ***************

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5G DDNMF	5G Direct Discovery Name Management Function

AS layer	Access Stratum layer

NCI	NR Cell Identity

PDUID	ProSe Discovery UE ID

PFI	PC5 QoS Flow Identifier

PQI	PC5 5QI

ProSe	Proximity based Services

RPAUID	Restricted ProSe Application User ID

RSC	Relay Service Code

RSD	Rule Selection Descriptor

****************** SECOND CHANGE ***************

5.8.3.3	Identifiers for 5G ProSe Layer-2 UE-to-Network Relay

The following parameters may be used in Announcement message (Model A) or Response message (Model B) in addition to the parameters as specified in clause 5.8.3.1 for 5G ProSe Layer-2 UE-to-Network Relay (re)selection:

-	PLMN ID(s): indicates the PLMN ID(s) corresponding to the serving cell of the 5G ProSe Layer-2 UE-to-Network Relay.

-	NCI: indicates the NCI of the serving cell of the 5G ProSe Layer-2 UE-to-Network Relay.

****************** END OF CHANGES ***************