3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107415

Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

Title:	[DRAFT] Reply LS on network slice admission control for SNPN onboarding

Response to:	LS (S2-2107028/C1-215159) on network slice admission control for SNPN onboarding from CT1

Release:	Release 17

Work Item:	eNPN

Source:	[Huawei to be] SA2

To:	CT1

Cc:

Contact Person:

Name:	zhuqianghua

Tel. Number:

E-mail Address:	zhuqianghua AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	501CR#3297

1. Overall Description:

SA2 thanks CT1 for the LS (C1-215159) on network slice admission control for SNPN onboarding.

CT1 Question: When the UE is registering or registered for onboarding services in SNPN or PLMN, whether the network slice admission control needs to be performed for slice(s) used for onboarding or not?

SA2 Answer:

-	In the case that onboarding network is an SNPN, the AMF Onboarding Configuration Data is used by AMF to apply S-NSSAI and DNN used for Onboarding for the PDU Session Establishment request from the UE, as well as selection of SMF supporting UE onboarding, hence to restrict UE network usage to only onboarding. The ON-SNPN does not need to allocate any network slicing resources for the S-NSSAI used for onboarding for the purpose of restricting UE network usage to only onboarding. Therefore SA2 doesn’t see a need to support the network slice admission control for S-NSSAI used for onboarding configured at the ON-SNPN. However, it is up to ON-SNPN operator to choose whether to apply network slice admission control for S-NSSAI used for onboarding.

-	In the case that onboarding network is a PLMN, the UE subscription that contains DNN and S-NSSAI used for onboarding is used by AMF to check whether the UE is allowed to request for PDU Session used for remote provisioning of UE via User Plane. And S-NSSAI used for onboarding is also used for selection of SMF or UPF. For UEs that access to onboarding services, they can also access to normal services if allowed. Since the PLMN should offer normal services other than onboarding services, so probably the PLMN uses dedicated network slicing resources for the S-NSSAI used for onboarding, so as to avoid impacts to the normal services. Therefore SA2 sees the possibility to support the network slice admission control for S-NSSAI used for onboarding within PLMN acting as onboarding network. However, it is up to PLMN operator to choose whether to apply network slice admission control for S-NSSAI used for onboarding.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks the CT1 group to consider the above feedbacks and update their specifications if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia

TSG-SA2 Meeting #149E   	February 14 – 25, 2022 			Elbonia