3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107441

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0010 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and Correction on AF and NEF authentication service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the last meeting, the DNN/S-NSSAI has been added as an input paramater of Nnef_Authentication_Authenticate service operation in the UUAA-SM procedure. Thus, it needs to update the NEF service in clause 4.4.1.1 to cover the DNN/S-NSSAI. - As the Nnef/Naf_authentication_notification servie operation is used for revoke, re-authentication, re-authorization or authorization data update, a “notify reason” is needed as an input parameter to indicate the purpose of this notification. - Notification endpoint is not needed as the input of Nnef/Naf_Authentication_notification service operation, as this was provided in Nnef_Authentication_authenticate service operation so that the Nnef knows the Notification endpoint to invoke the Nnef_Authentication_notification service operation |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Addition of DNN,S-NSSAI as input of Nnef_Authentication_Authenticate service operation. 2. Addition of Notify reason as input of Nnef/Naf_Authentication_Notification servie operation. 3. Remove notification endpoint from Nnef/Naf_Authentication_Notification service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong NEF/AF implementation and incomplete specification to support UAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.2, 4.4.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.4.1.1.2	Nnef_Authentication service

4.4.1.1.2.1	General

Service Description: This service enables the consumer to either authenticate and authorise, or just authorize, the Service Level Device Identity. In case of UAS, the service is used to authenticate and/or authorize the UAV identified by a CAA-Level UAV ID.

4.4.1.1.2.2	Nnef_Authentication_Authenticate service operation

Service operation name: Nnef_Authentication_AuthenticateAuthorize

Description: Provides the authentication and authorization result of the Service Level device Identity.

Input, Required: Notification endpoint, Service Level Device Identity (i.e. CAA-Level UAV ID), GPSI, DNN, S-NSSAI.

Input, Optional: Authorization Server Address (i.e. USS Address), PEI, UE IP address (in case the consumer NF is SMF), authentication/authorization container provided by UE.

Output, Required: Success/Failure indication, Authorization Data container, Authentication Session Correlation Id.

Output, Optional: None.

4.4.1.1.2.3	Nnef_Authentication_Notification service operation

Service operation name: Nnef_Authentication_Notification

Description: Re-authenticate, re-authorize, update authorization data or revoke the UUAA authorization of a UAV.

Input, Required: Authentication Session Correlation Id, Service Level Device Identity, 3GPP UAV ID, Notify reason (revoke, re-authentication, re-authorization or authorization data update).

Input, Optional: Authorization Data container, cause of revocation.

Output, Required: Acknowledge indication.

Output, Optional: None.

# * * * * Second change * * * *

4.4.1.2.2	Naf_Authentication service

4.4.1.2.2.1	General

Service Description: This service enables the consumer to authenticate and authorize the Service Level Device Identity. In case of UAS, the service is used to authenticate and authorize the UAV identified by a CAA-Level UAV ID.

4.4.1.2.2.2	Naf_Authentication_Authenticate service operation

Service operation name: Naf_Authentication_AuthenticateAuthorize

Description: Provides the Authentication and Authorization result of the Service Level Device Identity (i.e. CAA-Level UAV ID for UAS).

Input, Required: Notification endpoint, Service Level Device Identity, GPSI.

Input, Optional: PEI, UE IP address, authentication container provided by UE, UAV location.

Output, Required: Success/Failure indication, GPSI, Authorization Data container, Authentication Session Correlation Id.

Output, Optional: None.

4.4.1.2.2.3	Naf_Authentication_Notification service operation

Service operation name: Naf_Authentication_Notification

Description: Re-authenticate, re-authorize, update authorization data or revoke the UUAA authorization of a UAV.

Input, Required: Authentication Session Correlation Id, Service Level Device Identity, GPSI, Notify reason (revoke, re-authentication, re-authorization or authorization data update).

Input, Optional: PDU Session IP address, Authorization Data container, cause of revocation.

Output, Required: Acknowledge indication.

Output, Optional: None.

# * * * * End of changes * * * *