3GPP TSG-SA2 Meeting # 147E (e-meeting)	S2-2107476

# Elbonia, October 18 - 22, 2021													   (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0023 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Lossless handover for MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN#93e meeting #93e, the fllowing agreement has been achieved: “There is no consensus to make any changes to the WID on the issue of lossless HO for Rel-17 MBS raised in. ” So it means the lossless HO in Rel-17 should be supported. In RAN3 #112e, the fllowing agreement has been achieved: “Standards support data forwarding to minimize data loss during handover from MBS-supporting nodes to non-MBS supporting nodes. If data forwarding is used from MBS-supporting nodes to non-MBS supporting nodes, the source NG-RAN node should include in forwarded packets the unicast (flow) QFI mapped from the received MBS (flow) QFI.” Based on above agreement, it is proposed add some basic statement for lossless handover procedure to align with the RAN conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The sequence number is added in each MBS session data sent by MB-UPF. - How to perform the MBS session data forwarding in non-homegeneous scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lossless handover for MBS session is not supported, which is not aligned with the RAN conclusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.2.3.5	Minimization of data loss

A sequence number is added in each data packet of the multicast MBS session sent by MB-UPF and forwarded to all related NG-RAN node and UPF.

To minimize the data loss of the UE during the handover procedure:

- If source NG-RAN supports MBS and target NG-RAN does not support MBS, the multicast MBS session data is forwarded from source NG-RAN node to target NG-RAN node via the associated QoS flow within the associated PDU session.

- If source NG-RAN does not support MBS and target NG-RAN supports MBS, for Xn/N2 handover, the multicast MBS session data is forwarded to the target NG-RAN node as the data forwarding mechanism defined in TS 23.502.

NOTE: 	For N2 handover, the detail mechanism of how to transmit the received forwarded multicast MBS session data to UE is defined by RAN WGs.

# * * * * End of change * * * *