3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107545

Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

Title:	[DRAFT] LS on MBS broadcast service continuity and MBS session identification

Response to:	LS (R2-2108914) on the MBS broadcast service continuity and MBS session identification from RAN2

Release:	Release 17

Work Item:	NR_MBS-Core, 5MBS

Source:	[Huawei to be] SA2

To:	RAN2

Cc:	RAN3, SA4, SA6

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

SA2 would like to thank RAN2 for the LS. SA2 has discussed the three questions in the LS and provides the feedback of the answers as follows:

Question 1: Can an “MBS ID” (e.g. SAI) be defined for NR MBS for use in SIB and the upper layer signalling (e.g. USD), to avoid too many TMGIs from being broadcast in System Information?

SA2 answer: SA2 agrees to use MBS SAI (i.e., MBS Service Area ID) for reducing the volume broadcasted in SIB. The MBS SAI is with the similar functionalities as MBMS SAI in TS 23.003. A cell shall be able to belong to one or more MBM SAs, and therefore is addressable by one or more MBS SAIs.

Question 2: Can the mapping between frequency and MBS service/session be provided in the upper layer signalling (e.g. USD), as in LTE SC-PTM?

SA2 answer: SA2 confirms that the mapping between frequency and MBS SAI can be provided to the UEs in the upper layer signalling, e.g., via Service announcement. Details will be determined by SA4 and SA6.

Question 3: For both broadcast and multicast session, is sessionID parameter or alike required in NR or is TMGI sufficient to identify the MBS session?

SA2 answer: In MBMS, sessionID is used for distinguishing the MBMS session retransmissions, and labeled by BM-SC. Similarly, SA2 assumes allocating session ID is part of the service layer functionalities of MBSF, and will be determined by SA4.

2. Actions:

To RAN2 group.

ACTION: 	SA2 respectfully asks RAN2 to take SA2 answers into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia