3GPP TSG-SA2 Meeting #147-e	S2-2107607

# Online, , 18th-22nd Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 034 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on MBS Session update procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communications Co. LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [7.2.6] 1. The procedure is only for update of QoS parameter. 2. Step 8 is not always needed. 3. Other QoS parameters than ARP for associated QoS flow also can be updated by RAN node if associated QoS flow is known. 4. Notification in step 7 uses correlation info instead of TMGI. 5. Step 8 only for individual delivery. [7.2.7] 6. Duplicated description with 7.2.6 7. AF does not know the UE connection state, AF informs area update is not a good practice. 8. Use different method for UE location query. 9. PDU session update is needed when previously outside but now inside. [7.2.X] 10. Current specification does not specify when the session is updated to add new QoS flows or remove QoS flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [7.2.6] 1. Change title for QoS parameters update. 2. Make step 8 optional in figure 3. Specify that RAN update associated QoS flows with the updated QoS parameters. 4. Change TMGI to correlation info in step 7. 5. Change step 8 only for individual delivery. [7.2.7] 6. Reuse 7.2.6 as possible 7. Remove the AF informs area update to UE in figure. 8. Reuse UE location event subscription for UE location query. 9. Specify that PDU session update is also needed when previously outside but now inside. [7.2.X] 10. Add new clause for QoS flow update, and the PDU session modification is triggered after HO to avoid massive PDU session modification.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not clear and complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.6, 7.2.7, 7.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.2.6	Multicast MBS session QoS parameters update procedure

Multicast MBS session QoS parameters update procedure is invoked by the AF to update the ARP and other QoS parameters for an ongoing multicast session. For the interaction between AF and MB-SMF, see clause 7.1.1.6 and 7.1.1.7.

Figure 7.2.6-1: Multicast MBS session QoS parameters update procedure.

1.	This procedure is triggered by the MB-SMF receiving the updated policy for a multicast MBS session, see clauses 7.1.1.6 and 7.1.1.7.

2.	The MB-SMF generates the QoS profile for the multicast MBS session, and invokes Namf_MBSCommunication_N2MessageTransfer service operation (MBS Session ID, N2 SM information (TMGI, QoS profiles for multicast MBS session)) to the AMF(s).

3.	The involved AMF sends N2 MBS session request (N2 SM information) to the NG-RAN nodes based on the MBS Session ID and RAN node IDs stored in the AMF for the multicast MBS session.

4.	The NG-RAN node updates the multicast MBS session context, and also updates the QoS parameters of the associated QoS flows in the associated PDU Sessions.

5.	The NG-RAN node(s) acknowledges N2 MBS session request by sending an N2 MBS session Response message to the AMF.

6.	The AMF invokes the Nmbsmf_MBSSession_ContextUpdate () to the MB-SMF.

7.	The MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify request (subscription correlation info, Event ID, QoS profiles for multicast MBS session) to the SMFs.

8.	[Conditional] The SMF determines the UE list with individual delivery regarding the MBS Session ID related to the subscription correlation info. The SMF triggers PDU Session Modification procedure for each UE in the UE list as defined in TS 23.502 [6].

When delivery method switching from shared to individual after handover procedure, if associated QoS flow is not configured, the SMF triggers PDU Session Modification procedure as defined in TS 23.502 [6] for the UE

# * * * * Second change * * * *

### 7.2.7	Multicast MBS session service area update procedure

9. Determine affected UEs

Figure 7.2.7-1: MBS service area update procedure for multicast

This procedure is triggered by the MB-SMF receiving the updated service area for multicast MBS session, see clauses 7.1.1.6 and 7.1.1.7.

1.	The AF providing the updated service area may also inform UEs at application level about the new service area via a service announcement. If a UE is located in a radio cell which was previously outside the service area and is now inside the updated service area, the UE may join the multicast service.

2.	[Conditional] This step is optional and can be omitted. However, it allows to terminate data transmission towards cells served by the RAN node previously in the multicast service area and now outside the updated multicast service area as quickly as possible.

Same as described in steps 2-6 with the difference that the MB-SMF includes the new MBS service area in the N2 SM information. The AMF sends the N2 SM information to those NG-RAN nodes serving the multicast MBS session.

3.	The MB-SMF invokes Nmbsmf_MBSSession_ContextStatusNotify (subscription correlation info, Event ID, MBS service area, [Area Session ID]) to each SMF with a related subscription. If Area Session ID exists, the MB-SMF notifies the MBS service area corresponding to the Area Session ID to the SMFs involved in the multicast MBS session identified by the MBS Session ID and Area Session ID.

4.	[Conditional] The SMF determines the affected UEs it serves based on the subscription correlation identifying the multicast MBS session and Area Session ID (if exists) included in the step 3.

If the SMF decides to subscribe at the AMF to notify about the UE moving in or out of a subscribed "Area Of Interest" event, the SMF re-subscribes UE location to the AMF with the new MBS service area.

5.	[Conditional] If the UE is previously inside the old MBS service area but now outside the new MBS service area, or previously outside the old MBS service area but now inside the new MBS service area, the SMF performs PDU Session Modification for the UE with the new MBS service area included in the N1 SM container. During the PDU Session Modification, based on the UE location report, the SMF instructs the NG-RAN to remove or create the associated resources related to the multicast MBS session for the UE, which is used for individual delivery or shared delivery depends on NG-RAN capability.

# * * * * Third change (new text) * * * *

### 7.2.X	Multicast MBS session QoS flow update procedure

Multicast MBS session QoS flow update procedure is invoked by the AF to create or remove QoS flow(s) for an ongoing multicast session. For the interaction between AF and MB-SMF, see clause 7.1.1.6 and 7.1.1.7.

Figure 7.2.X-1: Multicast MBS session QoS flow update procedure.

1.	This procedure is triggered by the MB-SMF receiving the updated policy for MBS, see clauses 7.1.1.6 and 7.1.1.7.

2.	Same as described in step 2-8 of clause 7.2.6 with the difference that the NG-RAN does not update the parameters of associated PDU Sessions.

3.	[Conditional] The UE involved in the multicast MBS session performs handover procedure.

4.	[Conditional] Based on the policy, if the associated PDU Session of the UE for the multicast MBS session has not been updated, the SMF performs PDU Session Modification.

If the UE is served by shared delivery after the handover, the SMF includes the MBS related information in N2 SM Information sent to the NG-RAN node, which includes the MBS session ID and Area Session ID (if exists), the associated QoS flow(s) information, and the mapping information between the associated QoS flow and the multicast QoS flow.

# * * * * End of changes * * * *