3GPP TSG-WG SA2 Meeting #147E e-Meeting	S2-2107634

# Elbonia,  Oct 18 - Oct 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3202 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF Service Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UPF has service based interface, so it is efficient to subscribe to the UPF directly. Therefore, it proposes the UPF subscription and unsubscription service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the UPF subscription and unsubscription service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inefficient to subscribe to the UPF exposure service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 (all new text) * * *

### 5.2.x	UPF Services

#### 5.2.x.1	General

The following table shows the UPF Services and Service Operations:

Table 5.2.x.1-1: NF Services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | NEF |
|  | Unsubscribe |  | NEF |
|  | Notify |  | NEF |

#### 5.2.x.2	Nupf_EventExposure service

##### 5.2.x.2.1	General

See clause 4.15.3.2.x.

##### 5.2.x.2.2	Nupf_EventExposure_Subscribe operation

Service operation name: Nupf_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in UPF, then the subscription is updated.

Inputs, Required: (Set of) Event ID(s) as specified in clause 4.15.3.1, Target of Event Reporting defined in clause 5.2.6.2.2, Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Event Filter, (set of) Application Identifier(s), Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1).

##### 5.2.x.2.3	Nupf_EventExposure_Unsubscribe service operation

Service operation name: Nupf_EventExposure_Unsubscribe

Description: the NF consumer deletes an event if already defined in UPF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: None.

* * * End of changes* * *