3GPP TSG-SA2 Meeting #147-e	S2-2107943

# Online, ,18-22  October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3345 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications of NSAC and NSAC for roaming cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | # eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces a number of editorial changes to improve the readability of sections 5.15.11.0, 5.15.11.1, 5.15.11.2., 5.15.11.3 In addition, the CR adds the possibility of HPLMN based admission control for roaming UEs and both for the count of registrations and for the count of PDU connections per network slice. - VPLMN based control of the number of roaming UEs registering to an S-NSSAI is not possible in the case where the roaming UE is registering to the S-NSSAI via the EPC in order to use home routed services. This means that the SLA based local control in the VPLMN is applicable only when 4G-5G interworking is not deployed or when only LBO services are supported. These are not acceptable restrictions for the rel-17 timeframe. - The control of the number of PDU sessions for roaming UEs is currently split into two separate parts: LBO sessions are controlled by an NSACF in the VPLMN and Home routed sessions are controlled by an NSACF in the HPLMN. But, in a very typical case, a network slice may have a mix of PDU Sessions where some locally routed PDU Sessions are active in parallel to home routed PDU Sessions. The splitting of the admission control of the PDU Sessions of one S-NSSAI into multiple NSACFs. This leads to complex situations where some services in a network slice work and others don’t, leading to inconsistency in end user experience. Furthermore, the GSMA requirements make no difference between LBO and home routed sessions and this is why it is a natural extension to point (1) to allow HPLMN control of the complete set of PDU Sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces a number of editorial changes to improve the readability of sections 5.15.11.0, 5.15.11.1, 5.15.11.2., 5.15.11.3 In addition, the CR adds the possibility of HPLMN based admission control for roaming UEs and both for the count of registrations and for the count of PDU connections per network slice. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | frequent and serious mistakes in the admission control of roaming UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.0, 5.15.11.1, 5.15.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.15.11	Network Slice Admission Control

#### 5.15.11.0	General

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice and the number of PDU Sessions per network slice for the network slices that are subject to Network Slice Admission Control (NSAC). The NSACF is also configured with the information indicating which access type is specified for the S-NSSAI subject to NSAC (i.e. 3GPP Access Type, Non-3GPP Access Type, or both).

The NSACF also provides event-based Network Slice status notifications and reports to the consumer NF (e.g. AF).

Multiple NSACFs may be deployed in a network. If so, one NSACF may be responsible for one or more S-NSSAIs in a service area. Within a service area, one S-NSSAI is handled by one NSACF, which can be one NSACF instance or one NSACF Set. A PLMN may have one or more service areas.

A NSACF is configured with the maximum number of UEs per network slice and/or the maximum number of PDU Sessions per network slice which are allowed to be served by each network slice that is subject to NSAC. If multiple NSACFs are configured in a PLMN for a network slice, a NSACF is configured with a maximum number of UEs and/or a maximum number of PDU Sessions specific for the service area and the Network Slice.

Subject to operator policy and national/regional regulations, network slices which are used for Emergency service or for  Critical and Priority services (e.g. MCS, MPS) may be exempted from NSAC.

When the AMF receives a Registration Request for an Emergency Registration or with a Registration Request with an Establishment Cause indicating a priority service (e.g. MPS, MCS), or when the AMF subscription data includes priority subscription (e.g. MCS, MPS), the AMF may accept the registration request without applying NSAC, i.e., the AMF triggers NSAC procedure, but the NSAC exempted the UE response from the NSACF is ignored at the AMF.

When the SMF receives a PDU Session Establishment Request for an emergency PDU Session or a PDU Session Establishment Request with a priority header, the SMF may accept the PDU Session Establishment Request without applying NSAC, i.e., the SMF triggers NSAC procedure, but the response from the NSACF is ignored at the SMF.

Alternative, when NSAC is exempted for the S-NSSAI, the AMF and the SMF skip the corresponding NSAC procedure for the S-NSSAI, i.e. this UE (respectively PDU Session) is not counted towards the maximum number of UEs (respectively PDU Sessions).

#### 5.15.11.1	Maximum number of UEs per network slice admission control

The NSACF keeps  track of the current number of UEs registered for a network slice so that it can ensure it does not exceed the maximum number of UEs allowed to register with the network slice. The NSACF also maintains a list of UE IDs registered with a network slice that is subject to NSAC. When an event related to a UE causes the current number of UEs registered with a network slice to increase, the NSACF first checks whether the UE Identity is already in the list of UEs registered with that network slice.  If not, the NSACF checks whether the maximum number of UEs per network slice for that network slice has already been reached and if it has, the NSACF applies admission control policies.

The AMF triggers a request to NSACF for maximum number of UEs per network slice admission control when the UE's registration status for a network slice subject to NSAC is changing, i.e. during the UE Registration procedure in clause 4.2.2.2.2 of TS 23.502 [3], UE Deregistration procedure in clause 4.2.2.3 of TS 23.502 [3], Network Slice-Specific Authentication and Authorisation procedure in clause 4.2.9.2 of TS 23.502 [3], AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure in clause 4.2.9.3 of TS 23.502 [3], and AAA Server triggered Slice-Specific Authorization Revocation in clause 4.2.9.4 of TS 23.502 [3].

Since the UE may register or deregister for an S-NSSAI via 3GPP access and/or non-3GPP access as described in clause 5.15.5.2.1, the AMF provides the Access Type to the NSACF when triggering a request to increase or decrease the current number of UEs registered with a S-NSSAI . The NSACF may take the Access Type into account for increasing and decreasing the number of UEs per network slice bystoring the UE ID with the associated one or more Access Type(s), i.e. the NSACF is able to add or remove a registration for the UE ID for each Access Type and trigger the increase or decrease of the current number of UEs registered with a S-NSSAI based on a policy that takes the access type into account.

NOTE:	For example, if the NSACF is configured to apply NSAC for 3GPP Access Type only, the NSACF counts registration via 3GPP access type only. If the NSACF is configured to apply NSAC for both Access Types, and the UE newly registers via 3GPP access while the UE is already registered via non-3GPP access (or vice versa), the NSACF updates the UE ID entry with both 3GPP Access Type and non-3GPP Access Type and the NSACF may count the UE once or twice based on its policy.

#### 5.15.11.2	Maximum number of PDU sessions per network slice admission control

The NSACF keeps track of the current number of PDU Sessions per network slice so that it can ensure it does not exceed the maximum number of PDU session allowed to be served by the network slice. When an event related to a UE causes the current number of PDU sessions established within the network slice is to increase, the NSACF checks whether the maximum number of PDU sessions per network slice for that network slice has already been reached, and if it has, the NSACF applies admission control policies.

The anchor SMF triggers a request to NSACF for maximum number of PDU sessions per network slice control during PDU session establishment/release procedures in clauses 4.3.2 and 4.3.4 of TS 23.502 [3].

The SMF provides the Access Type to the NSACF when triggering a request to increase or decrease the number of PDU Sessions. The NSACF takes Access Type into account for increasing and decreasing the current number of PDU Sessions depending on the applicability of the Access Type for the NSAC for maximum number of PDU Sessions for the S-NSSAI.

NOTE:	I-SMF does not interact with NSCAF.