3GPP TSG-SA WG2 Meeting #148E (e-meeting)	S2-2108495

15 - 22 November 2021, Elbonia	(revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3380 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Service Areas and Forbidden Areas for NR Satellite Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A solution for supporting Service Areas and Forbidden Areas for NR Satellite Access has been defined and evaluated in S2-210xxxx. This solution adds 2 new rules to those already supported for TN: Rule 1 Both an AMF and a UE shall consider that a radio cell is allowed for the UE as long as at least one TAC supported by the radio cell for the RPLMN or equivalent to the RPLMN is not subject to a known restriction for a forbidden area, service area or RAT type. Rule 2 When receiving a NAS Reject message from an AMF indicating a forbidden area, and where the serving radio cell indicates support for more than one TAC for the RPLMN or equivalent to the RPLMN, a UE shall consider that all TACs indicated by the radio cell for the RPLMN or equivalent to the RPLMN are forbidden. Changes to TS 23.501 are needed to support this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add impacts to support the new rules for SAs and FAs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to support Service Areas and Forbidden Areas for NR Satellite access. This will mean that UEs would always be allowed NR satellite access over the whole coverage area of any PLMN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 5.4.11.Y	Service Areas and Forbidden Areas

Both an AMF and a UE shall consider that a radio cell is allowed for the UE as long as at least one TAC supported  by the radio cell for the RPLMN or equivalent to the RPLMN is not subject to a known restriction for a forbidden area, service area or RAT type.

When receiving a NAS Reject message from an AMF indicating a forbidden area, and where the serving radio cell indicates support for more than one TAC for the RPLMN or equivalent to the RPLMN, a UE shall consider that all TACs indicated by the radio cell for the RPLMN or equivalent to the RPLMN are forbidden.

**** End of Changes ****