3GPP TSG-WG SA2 Meeting #148E e-meeting 	S2-2108537

Elbonia, November 15 – 19, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on ML Model Filter Information during the NWDAF containing MTLF registration

Document for:	Discussion

Agenda Item:	8.1

Work Item / Release:	eNA_Ph2 / Rel-17

Abstract: This contribution proposes to discussion on the ML Model Filter Information during the NWDAF containing MTLF registration.

1. Discussion

During the SA2#147E meeting, the 23.288 CR S2-2107250 proposes to remove the ML Model Filter Information during the NWDAF containing MTLF registers its ML model provisioning services into the NRF.

From Huawei’s understanding, the ML Model Filter Information is needed anyway.

In Rel-16, the NWDAF only supports analytics services, so the Analytics ID, S-NSSAI and the NWDAF Serving Area information in the NWDAF profile in Rel-16 are only applied for the analytics services.

While in Rel-17, the NWDAF is decomposed into AnLF and MTLF, one or both of which can be included in an NWDAF. For an NWDAF contains both AnLF and MTLF, the parameter type or the value of the parameter type in the Analytics Filter Information for the AnLF and the ML Model Filter Information for the MTLF can be different. If the NWDAF profile for analytics exposure and the ML model provisioning are mixed up, it may not possible to discovery a suitable NWDAF for the consumer NF.

For example:

-	For an NWDAF X who has both AnLF and MTLF, the AnLF can support analytics inference for S-NSSAI A and S-NSSAI B, but the MTLF can only support ML model training for S-NSSAI A.

-	When no ML Model Filter (including only S-NSSAI A) is provided during the NWDAF containing MTLF registration, the NWDAF X may be provided by the NRF to the consumer who want to discover an NWDAF who support for the ML model training for S-NSSAI B.

So it is better to distinguish the NWDAF profile for the analytics services and the ML model provisioning.

2. Proposal

Proposal. It proposes to not remove the ML Model Filter Information during the NWDAF containing MTLF registration.