3GPP TSG-SA WG2 Meeting #148E (e-meeting)	S2-2108553

# November 15 – November 22, 2021, Elbonia	(revision of S2-2107150)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0002 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and EN resolution for SMF Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 23.256 specifies as below and EN need to be resolved “In addition, when AMF invokes Nsmf_PDUSession_CreateSMContext service operation, AMF may provide an indication to indicate "UAV has been authenticated by the USS", if the UUAA has taken place during registration procedure by the AMF. Editor's Note: Whether an indication is needed or if UAS NF or NEF can decide since these entities will maintain the UUAA operation status is FFS.” The intention of the indication is to inform the SMF that there is no need for SMF to trigger the UUAA-SM again if there was successful UUAA-MM. However, to give a reference to trigger USS to subscribe PDU Session Status Event, SMF need to use Naf_Authentication_Authenticate service operation including DNN/S-NSSAI although UUAA is performed by AMF. Also, UAS NF/NEF can decide whether the UAV is authenticated by UUAA context. Therefore, the indication is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the description about the need of indication to indicate “UAV has been authenticated by the USS” in SMF services and Editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary descrption and unresolved Editor's note exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.4.1.4	SMF Services

SMF services related to UAS are defined in TS 23.502 [3] clause 5.2.8.

* * * * End of Changes * * * *