3GPP TSG-SA2 Meeting # 148E (e-meeting)	S2-2108680

# Elbonia, November 15 – 19, 2021											       (revision of S2-2108014)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0021 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the local MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, [LG Electronics, Nokia, Nokia Shanghai-Bell, Samsung, ZTE,vivo] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For location dependent MBS session, the SMF get the MB-SMF profile include all related area session ID and MBS service area it support. Similar the UE also get the whole MBS servcie area for the MBS session based on the service announcment. However for the UE joining procedure only one area session ID and its related MBS service area need be included and sent to NG-RAN node. It is not need to send other Area Session ID and MBS service Area to the NG-RAN. Thus some unnecessary plural description need be removed. - To support the individual delivery, the SMF need subscribe the UE mobility event from AMF. - For the location dependent MBS session, the Area Session ID and MBS service area are also need be registered to NRF. So it is unsuitable to assume if the MBS session ID is TMGI, the registration can be skipped. - Add new clause about the local MBS session activation and termination of the local MBS session. - For the broadcast MBS service, there are no SMF need be involved. So the NOTE about the MB-SMF need be same for the one SMF service area is unclear on how these two are related. - In clause 7.2.4.2.2 (see below), the following text “for the UE” is incorrect and it is proposed to say “…procedure is performed as defined….” For local MBS, the configuration procedure for the UE is optional and performed as defined in clause 7.1.1.2 with the following additions Rev 2: - Clause 6.2.1 the subscription also applies to the shared delivery to let the SMF be aware that the UE need be removed from the MBS session. - Adrress the Editor’s Note related to mobility procedure, which is also analized in S2-2108679. . - Add two new clause related to the UE is removed from the MBS session. - Add a new clause related to the Inactivate MBS session state mobility handling. All the change introduced in the Rev2 are in yellow highlight in the change part or clause title(If whole clause change is new). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove unnecessary plural description - Add the UE mobility event subscritption description - Remove the MB-SMF can skip the registration part description. - Add new clause about the local MBS session activation and termination of the local MBS session. - Remove unclear NOTE description. Rev 2: - Clause 6.2.1 the subscription also applies to the shared delivery to let the SMF be aware that the UE need be removed from the MBS session. - Adrress the Editor’s Note related to mobility procedure. - Add two new clause related to the UE is removed from the MBS session. - Add a new clause related to the Inactivate MBS session state mobility handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local MBS for individual delivery will not be supported for the Activtion, procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.3, 7.2.4.1, 7.2.4.2.1, 7.2.4.2.2, 7.2.4.2.3, 7.2.4.2.4, 7.2.4.2.5, 7.2.4.3.1, 7.2.4.3.2, 7.2.4.3.3, 7.2.4.3.4, 7.2.4.3.5, 7.2.4.x, 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

6.2	Local MBS service and Location dependent MBS service

6.2.1	General

A Local MBS service is an MBS service provided in one MBS service area. A location dependent MBS service is an MBS service provided in several MBS service area(s). An MBS service area is identified by a cell list or a tracking area list. The MBS service area could be geographical area information or civic address information, and NEF/MBSF translates the location information to Cell ID list or TAI list as MBS service area, see clause 7.1.1.2.

If the multicast session relates to a Local MBS service or a location dependent MBS service, the SMF subscribes at the AMF to notifications about the "UE location" or UE moving in or out of a subscribed "Area Of Interest"" event using the Namf_EventExposure service.

For multicast communication, local MBS and location dependent MBS services may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS. If the SMF obtains a notification about the UE location, it checks whether the UE is still in the MBS service area of the multicast session. For a local MBS service, if the UE is no longer in the MBS service area, the SMF terminates the 5GC Individual MBS traffic delivery towards the UE. For a location dependent MBS service, if the UE is no longer in the current MBS service area, the SMF determines whether the UE is in another MBS service area of the multicast session; if so, the SMF configures the UPF to send multicast data relating to the new MBS service area towards the UE.

If the SMF removes the UE from the MBS session, it unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

For more details, refer to clause 7.2.4 for multicast MBS Session and refer to clause 7.3.4 for broadcast MBS Session.

6.2.2	Local MBS service

For a local MBS service, only UEs within the MBS service area may receive content data, while UEs outside the MBS service area are not allowed to receive location specific content. For multicast MBS service, UEs outside the MBS service area are not allowed to join the MBS service, and the network shall not deliver location specific content anymore to the UEs moved out of the MBS service area. Depending on policy, for the multicast MBS service the network may remove UEs outside the MBS service area of the MBS session from the MBS session context after a grace period (e.g. by using a local configured timer).

The UE shall be able to obtain service area information of the local multicast service via MBS service announcement or via NAS signalling (UE Session Join Accept/Reject including Cell ID list or TAI list). If the UE Session Join procedure fails due to the UE being outside the MBS service area, the UE does not attempt to join the multicast session again until the UE moves inside the MBS service area. When the UE Session Join succeeds and if the multicast session is deactivated, the UE does not perform monitoring the session activation notification and any other information related to the multicast session identified by an MBS Session ID over the radio if outside the MBS service area.

NOTE:	Broadcast communication service is the service provided simultaneously to all UEs in a geographical area, therefore for broadcast it is naturally a local MBS service.

For more details, refer to clause 7.2.4 for multicast MBS Session and refer to clause 7.3.4 for broadcast MBS Session.

6.2.3	Location dependent MBS service

A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location-dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE. For multicast MBS service, UEs outside all MBS service area of the MBS Session are not allowed to join the MBS service. When UE moves out of an MBS service area and there is no other MBS service area for the MBS session, the network ceases to deliver the content data to the UE. Depending on policy, for the multicast MBS service the network may remove UEs outside all MBS service areas of the MBS Session from the multicast MBS session context after a grace period (e.g. by using a local configured timer).

Information about different MBS service areas for a location dependent MBS service may be provided by one or several AFs or may be configured. Different ingress points for location dependent points for the MBS session are supported for different MBS service area dependent content of the MBS session; different MB-SMFs and/or MB-UPF may be assigned for different MBS service areas in an MBS session. When the different MB-SMFs are assigned for different MBS service areas in an MBS session, the same TMGI is allocated for this MBS session.

The Area Session ID is allocated by MB-SMF in MBS Session configuration procedure. MB-SMF allocates Area Session ID for each MBS services area which is unique within the MBS session. MB-SMF needs to further ensure there is no MBS service area overlapping with other MBS service areas that share the same MBS session ID.

NOTE 1:	In this release, deployments topologies with specific SMF Service Areas are not supported, as a result, location dependent service using multicast communication is not supported when a UE moves outside its SMF service area.

NOTE 2:	For location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

NOTE 3:	An example of Location-dependent MBS is a nationwide weather forecast service with local weather reports.

NOTE 4:	Area Session ID is equivalent to Flow ID as specified in TS 23.246 [8].

# * * * * Next change * * * *

7.2.4	Support of Local multicast service and Location dependent multicast service

7.2.4.1	General

The clause captures the procedural enhancement to supportLocation-dependent MBS service and the Local MBS service described in clause 6.2.

7.2.4.2	Support of location-dependent multicast service

7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The local multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The local multicast session is configured as described in clause 7.2.4.2.2.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	The SMF check whether the UE is inside the MBS service area by comparing the User Location Information of the UE with the MBS service area received from the MB-SMF.If the SMF has no UE current location information, the SMF queries the AMF. If the UE is out of the MBS service area of the whole MBS session, the SMF reject the multicast session join request.

-	If the Join Request from the UE is accepted, the SMF subscribes at the AMF using the Namf_EventExposure service to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. The SMF supplies the whole MBS service area of the MBS session as Area Of Interest.

-	The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID and MBS service areaassociated with the cell where UE camps. .

-	For the case that the NG-RAN node supports MBS, the NG-RAN uses the received MBS Session ID and Area Session ID to determine the local multicast session context and whether the user plane for the local multicast session is already established. If the target RAN determines the shared delivery is not established for the MBS session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides all MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN.

-	For the case that the NG-RAN node serving the UE does not support MBS and UE is in the MBS service area, the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via individual delivery within a PDU session of the UE. The SMF additionally subscribes at the AMF using the Namf_EventExposure service to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. The SMF supplies the service area associated with the multicast session and area session ID as Area Of Interest.

7.2.4.2.2	Configuration for local MBS

For local MBS, the configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, MBS service area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

-	The policy of Multicast session is determined based on the service requirements per MBSSession.MB-SMF associate the same service requirement QoS flow in different Area Session with the same QFI.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The whole MBS service area(s) for the MBS session are indicated to the UE in the Service Announcement as defined in clause 6.11.

7.2.4.2.3	Handover procedure

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-

If the  UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Shared MBS traffic delivery before the handover, the handover procedure is executed as defined in clauses 7.2.3.2 and 7.2.3.3 with the following difference.

-	For the Xn Handover:

-	The Source RAN node includes MBS Session ID, Area Session ID and MBS service area to the Target RAN node.

-	For the case that the Target RAN node supports MBS, it determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by Source RAN node, based on MBS Session ID, Area Session ID and MBS service area.

Editor’s note: Data forwarding issue needs the feedback of RAN WGs

-	For the case that the Target RAN node support MBS, it respondes to Source RAN node, with the accepted MBS Session ID, Area Session ID. When Target RAN supports MBS, but the UE is no longer in the MBS service area provided by the source RAN, Target RAN does not allocate RAN resource for the multicast MBS session to the UE.

-	For the case that the Target RAN node supports MBS, when it determines the shared delivery is not established for the MBS Session ID and Area Session ID, the Target RAN node initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides all MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN.

-	For the N2 Handover:

-	The source RAN node includes all MBS session area information (MBS Session ID, Area Session ID and MBS service area) to the Target RAN node in Handover Required message.The SMF includes all MBS session area information (MBS Session ID, Area Session ID and MBS service area) to the Target RAN in Handover request.

NOTE:  	Due to the content within the same MBS session can be switched at the RAN node autonomously, the Area Session ID provided by SMF can be different than the one provided by RAN node. In that case the Area Session ID from RAN node take precedence.

-	For the case that the Target RAN node support MBS, it determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided bysource RAN, based on MBS Session ID, Area Session ID and location area. If UE is not in the in the MBS service area provided by the source RAN, the Target RAN does not allocate RAN resources for the multicast MBS Session to the UE.

-	For the case that the target RAN node support MBS, when it determines the shared delivery is not established for the multicast session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides all MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN.

- For the Xn/N2 Handover,

- For the case that the target RAN not support MBS, the SMF applies individual delivery towards the UE.The associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the original area session ID or not.

If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Individual MBS traffic delivery before the Handover, the handover procedure is executed as defined in clause 7.2.3.4 with the following difference:

- For Xn handover, the associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the original area session ID or not.

- For the N2 handover:

- The SMF includes all MBS session area information (MBS Session ID, Area Session ID, and MBS service area) in N2 SM information to the Target RAN node in Handover Request message.

- For the case that the Target RAN node supports MBS, target RAN determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID provided by SMF, based on MBS Session ID, Area Session ID, MBS service area and UE location area. If UE is not in the in the MBS service area provided by the SMF, the Target RAN does not allocate RAN resources for the multicast MBS Session to the UE.

- For the case that the Target RAN node supports MBS, when it determines the shared delivery is not established for the multicast session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides all MBS service area information (Area session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN.

When the AMF receives the User Location Information from target RAN node via the the Path Switch Request message or Handover Notify message, the AMF provide it to the SMF. The AMF can also provide the User Location Information to the SMF after the location query from SMF, e.g. due to UE out of the Area of Interest. When the SMF get the User Location Information, the SMF checks the MBS service area of the UE camping cell by comparing the User Location Information received received from AMF with the MBS service areas received from the MB-SMF.

- The SMF update the UE MBS session context with the corresponding area session ID if the area session ID is changed.

- The SMF subscription at the AMF to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event is handling as following,

  - If the UE is still in the MBS session, the event related to UE in or out of the MBS service area associated with the MBS session is subscribed at the AMF

  - If the target NG-RAN node does not support MBS, the addional event related to UE in or out of the MBS service area associated with MBS session and Area Session ID is subscribed at the AMF.

- If the UE is out of the original MBS service area but in another MBS service area of the MBS session,

- For the case that the target NG-RAN node support MBS and RAN resource has not been allocated, the SMF provides the MBS session information related to the new Area session ID to NG-RAN via AMF using the Namf_Communication_N1N2MessageTransfer service operation. Per the received the MBS session information, the 5GC shared delivery is established.

- For the case that the target NG-RAN node does not support MBS, the SMF updates the UPF to forward the MBS data packet from the tunnel associated with the old Area session ID to the new Area session ID. If the SMF does not configure the UPF receiving the MBS data packet from the tunnel associated with the new Area session ID before, the SMF informs the MB-SMF of the new Area session ID and UPF DL N19mb Tunnel information. MB-SMF configure the MB-UPF to transmit the multicast session data towards UPF using the received downlink tunnel ID.

- If the UE is out of the whole service area of the MBS session,

- For the case that target NG-RAN node does not support MBS, the SMF delete the associated QoS flow from NG-RAN and UPF.

- Per operator’s policy (e.g. when a local configured timer expires since the UE left the whole MBS service area), the SMF may remove the UE from the MBS session as defined in clause 7.2.4.2.5.

# * * * * Next change * * * *

7.2.4.2.4	Activation of location dependent MBS session

When the location dependent MBS session is activated, the MBS session is activated in the whole MBS service area of the MBS session. It is not supported that the same MBS session is in active state in one MBS service area but in inactive state in another MBS service area.

For the location dependent MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which includes the whole MBS service area associated with the MBS session, i.e. the sum of all MBS service area associated with the MBS session ID regardless of the Area session ID.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request or Namf_MT_UEReachabilityInfoNotify message. Based on that information, the SMF determines how to activate the MBS session same as the handling when the SMF get the User Location Information defined in clause 7.2.4.2.3

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area, and Area Session ID, in the N2 SM information.

# * * * * Next change * * * *

7.2.4.2.5	SMF removing joined UEs from location dependent MBS session

Besides the condition as defined in clause 7.2.2.3, per operator's policy (e.g. when a local configured timer expires since the UE left the whole MBS service area) the SMF may remove the UE from the location dependent MBS session if the UE is out of the whole service area of the MBS session. When the UE is removed from the location dependent MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed Area Of Interest" event related to that MBS session.

# * * * * Next change * * * *

7.2.4.3	Support of multicast service available within a limited area

7.2.4.3.1	Local MBS service area information provided by AF

For local multicast services, the MBS service area information is provided to the UE and the 5GC as specified in clauses 7.1.1.2 and 7.1.1.3 with the following differences and clarifications:

-	For the Service Announcement, MBS service area information is included unless it is preconfigured.

-	When performing the MBS session request to the 5GC, the MBS service area information for a multicast session is provided by the AF unless it is preconfigured.

7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	The SMF determines whether the UE is inside the MBS service area by comparing the User Location Information of the UE received together with the join request with the MBS service area received from the MB-SMF. If the SMF has no UE current location information, the SMF queries the AMF.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the RAN node serving the UE does not support MBS the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session via individual delivery within a PDU session of the UE.

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6]. The SMF supplies the service area associated with the multicast session as Area Of Interest.

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

7.2.4.3.3	Handover procedure with limited area MBS session

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

If the UE is camping at the Source RAN node and receiving multicast data corresponding to the MBS Session ID via the 5GC Shared MBS traffic delivery before the Handover, the handover procedure is executed as defined in clauses 7.2.3.2 and 7.2.3.3 with the following difference:

-	For Xn based handover in clause 7.2.3.2, the Source RAN node includes MBS Session ID and MBS service area to the Target RAN node during Handover Preparation phase. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request and Handover Required message, respectively.

-	For Xn based handover in clause 7.2.3.2, if the target RAN node support MBS, it responses to Source RAN node with the accepted MBS Session ID based on the MBS service area of the MBS session. When Target RAN supports MBS but the UE is no longer in the MBS service area, the Target RAN node does not allocate RAN resources for the MBS Session to the UE. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request acknowledge message.

-	For Xn handover, if the UE is handed over to a target cell outside the MBS service area, the SMF does not provide the MBS session related information in N2 SM Info to the target RAN.

-	For N2 handover, if the UE is handed over to a target cell outside to the MBS service area, the SMF does not provide the MBS session related information in N2 SM Info to the target RAN.

- For the Xn/N2 Handover, if the target RAN does not support MBS the SMF applies individual delivery towards the UE. The associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the MBS session or not.

If the UE is camping at the Source RAN node and receiving multicast data corresponding to the MBS Session ID via the 5GC individual MBS traffic delivery before the Handover, the handover procedure is executed as defined in clause 7.2.3.4 with the following difference:

- For Xn handover the associated QoS flow is established regardless whether the UE is still in the MBS service area associated with the MBS session or not.

- For the N2 Handover:

- The SMF includes all MBS session area information (MBS Session ID and MBS service area) in N2 SM information to the Target RAN node in Handover request.

- For the case that target RAN node support MBS, target RAN determines whether to establish the resources for multicast distribution for MBS Session ID provided by SMF, based on MBS Session ID, MBS service area and UE location area.

When the AMF receives the User Location Information from target RAN node via the the Path Switch Request message or Handover Notify message, the AMF provide it to the SMF. The AMF can also provide the User Location Information to the SMF after the location query from SMF, e.g. due to UE out of the Area of Interest. When the SMF get the User Location Information, the SMF check the MBS service area of UE camping cell by comparing the User Location Information received received from AMF with the MBS service areas received from the MB-SMF.

- If the UE is out of the service area of the MBS session,

- For the case that target NG-RAN node does not support MBS, the SMF delete the associated QoS flow from NG-RAN node and the UPF.

- Per operator’s policy (e.g. when a local configured timer expires since the UE left the whole MBS service area of the MBS session) the SMF may remove the UE from the MBS session as defined in clause 7.2.4.3.5.

# * * * * Next change * * * *

7.2.4.3.4	Activation of limited area MBS session

For the limited area MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which include the MBS service area associated with the MBS session.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request or Namf_MT_UEReachabilityInfoNotify message.Based on that information, the SMF determines how to activate the MBS session same as the handling when the SMF get the User Location Information defined in clause 7.2.4.3.3.

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area in the N2 SM information.

# * * * * Next change * * * *

##### 7.2.4.3.5	SMF removing joined UEs from limited area MBS session

Besides the condition as defined in clause 7.2.2.3, per operator's policy (e.g. when a local configured timer expires since the UE left the MBS service area) the SMF may remove the UE from the limited areaMBS session if the UE is out of the service area of the MBS session. When the UE is removed from the limited area MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed Area Of Interest" event related to the MBS session.

# * * * * Next change * * * *

7.2.4.X	Inactive mobility procedure

If a UE that has joined into a multicast MBS session is outside the MBS service area of the MBS session, the MBS session state in the UE is Inactive state regardless whether the MBS session is activated in the MBS service area or not.

When the UE comes back to the MBS service area of the MBS session after out of the MBS service area of the MBS session before and the SMF detects that the UE is inside the MBS service area:

-	For location-dependent content, if the multicast MBS session is in the "Active" state, the SMF try to activate the multicast MBS session towards the UE per UE’s current location.

-	For a limited area content, if the multicast MBS session is in the "Active" state, the SMF try to activate the multicast MBS session towards the UE.

# * * * * Next change * * * *

7.3.4	Support for Local Broadcast Service

The procedures for Local broadcast service contains the ones for local broadcast service with the location-dependent content as described in clause 6.2.

The configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same broadcast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the TMGI and Area Session ID.

-	The policy of broadcast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

The MBS session establishment procedure is performed as defined in clause 7.3.1 with the following additions:

-	MB-SMF requests the AMF to transfer an N2 message (i.e. MBS Session Resource Setup Request) to the NG-RAN nodes of the MBS service area with broadcast session information which additionally includes the Area Session ID and MBS service area.

-	The NG-RAN uses the received MBS Session ID and Area Session ID to determine the local broadcast session context.

# * * * * End of change * * * *