SA WG2 Meeting #S2-148E  	S2-2108850

# November 15 – 22, 2021, Elbonia	(revision of S2-xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3297 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on AS time distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In the S2#147E meeting, it is agreed the parameter Time synchronization configuration Id is replaced by PTP instance reference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the Nnef_TimeSynchronization_ASTIDelete service operation used by AF. - Clarify the use of PTP instance reference. - Update the parameter Time synchronization configuration Id to PTP instance reference in Ntsctsf_TimeSynchronization_ASTI service operations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Incomplete specification. 2. Incorrect service operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.9. 4, 5.2.27.2.9, 5.2.27.2.10, 5.2.27.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

4.15.9.4	Procedures for management of 5G access stratum time distribution

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE, group of UEs or any UE using the DNN and S-NSSAI.

The Nnef_TimeSynchronization_ASTICreate and Nnef_TimeSynchronization_ASTIUpdate request may contain the parameters as described in Table 4.15.9.4-1. The AF may invoke a Nnef_TimeSynchronization_ASTIDelete service operation to remove the 5G access stratum time distribution configuration, and deactivate the corresponding 5G access stratum time distribution service.

Table 4.15.9.4-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.8.1 of TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |

Figure 4.15.9.4: Management of 5G access stratum time information

1.	AM Policy Association establishment as described in clause 4.16.1.

2.	The PCF subscribes to policy data related to time synchronization (Data Set = Application Data; Data Subset = Time-Sync data, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

3a.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	To create a new request, the AF provides time synchronization access stratum time information data to the NEF using the Nnef_TimeSynchronizationRTI_Create service operation (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1), including a target (one UE identified by SUPI or GPSI, a group of UEs identified by an External Group Identifier, or all UEs connected to a combination of DNN/S-NSSAI). The request may also contain a temporal validity condition. The NEF maps the External Group Identifier to an Internal Group Identifier and any GPSI to a SUPI.

-	To update or remove an existing request, the AF invokes a Nnef_TimeSynchronizationRTI_Update or Nnef_TimeSynchronizationRTI_Delete service operation via using the PTP instance reference to identify the time synchronization service configuration.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

NOTE 1:	Steps 1, 2, and 3 can occur in any order.

4.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_RTICreate service operation with the corresponding TSCTSF.

5.	(When the procedure is triggered by the AF request to influence the 5G access stratum time distribution):

-	The TSCTSF determines whether the targeted UE is part of a PTP instance in 5GS, if so the TSCTSF rejects the request.

(When the procedure is triggered by PTP instance activation, modification, or deactivation in the TSCTSF):

-	If time synchronization error budget is provided by the AF, the TSCTSF may use the PTP port state of each DS-TT to determine an error budget for corresponding SUPIs that are part of the PTP instance. The TSCTSF associates the 5G access stratum time information parameters with a SUPI (without DNN, S-NSSAI) when storing them to the UDR.

-	If the TSCTSF receives multiple time time synchronization error budgets for a given UE, then the TSCTSF picks the most stringent budget. The TSCTSF calculates the Uu time synchronization error budget as described in clause 5.27.1.9 of TS 23.501 [2]. Then, TSCTSF stores the calculated Uu time synchronization error budget together with the time distribution indication as Time Sync Data Subset in the UDR.

6.	The UDR informs the TSCTSF about the result of the operation of step 5.

7.	The UDR notifies the PCF(s) that have a matching subscription (from step 2) about the data stored, updated, or removed in step 5. If matching entries already existed in the UDR when step 5 is performed, this shall be immediately reported to the PCF.

8.	If the access stratum time information parameters in UDR are associated with a DNN/S-NSSAI for the SUPI (i.e. the parameters are valid only for a DNN/S-NSSAI), the PCF for the UE may discover the PCF for the PDU Session using Nbsf_Management_Subscribe with SUPI and (DNN, S-NSSAI) as parameters.

9.	Upon PDU Session establishment, if the PCF of the UE has subscribed for discovery of the PCF for the PDU Session from the BSF, the BSF provides to the PCF of the UE the identity of the PCF for the PDU Session and the UE address for the requested SUPI and (DNN, S-NSSAI) combination via an Nbsf_Management_Notify operation. If a matching entry already exists in the BSF when step 8 is performed, this shall be immediately reported to the PCF for the UE.

10.	The PCF takes a policy decision and then it may initiate an AM Policy Association Modification procedure initiated by the PCF for the UE may be performed as described in clause 4.16.2.2. As part of this, the time distribution indication and the Uu time synchronization error budget are provided to NG-RAN. Based on this, NG-RAN provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE 2:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [30].

11.	The TSCTSF responds the AF with the Ntsctsf_TimeSynchronization_ASTICreate response, including a reference to the time synchronization service configuration (PTP instance reference).

12.	The NEF informs the AF about the result of the Nnef_TimeSynchronization_ASTI operation performed in step 3.

# * * * * End of 1st change * * * *

# * * * * 2nd change * * * *

5.2.27.2.9	Ntsctsf_TimeSynchronization_ASTICreate operation

Service operation name: Ntsctsf_TimeSynchronization_ASTICreate

Description: Authorize the request, activate the 5G access stratum time distribution.

Inputs, Required: List of UE identities, mandatory service parameters as described in Table 4.15.9.4-1.

Inputs, Optional: DNN, S-NSSAI, optional service parameters as described in Table 4.15.9.4-1.

Outputs, Required: Operation execution result indication, in successful operation the PTP instance reference.

Outputs, Optional: None.

5.2.27.2.10	Ntsctsf_TimeSynchronization_ASTIUpdate operation

Service operation name: Ntsctsf_TimeSynchronization_ASTIUpdate

Description: Authorize the request and forward the request to update the 5G access stratum time distribution configuration.

Inputs, Required: PTP instance reference.

Inputs, Optional: service parameters as in Ntsctsf_TimeSynchronization_ASTICreate input.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

5.2.27.2.11	Ntsctsf_TimeSynchronization_ASTIDelete operation

Service operation name: Ntsctsf_TimeSynchronization_ASTIDelete

Description: Authorize the request, delete the 5G access stratum time distribution configuration, deactivate the corresponding 5G access stratum time distribution service.

Inputs, Required: PTP instance reference.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of 2nd change * * * *