SA WG2 Meeting #148e	S2-2108925

# November 15th – 22nd, 2021; Elbonia            		  (revision of S2-210)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	 Corrections to to procedures for Handover of a PDU Session between 3GPP and untrusted non-3GPP access

Document for: 	discussion

Agenda Item: 	6.10

Work Item / Release:	5GS_Ph1, TEI16

Abstract of the contribution: introduction to the 2 CR (cat F/ cat A) with the same title

# 1 background - Discussion

23.502 § 4.9.2.1, 4.9.2.2, 4.9.2.3.1, 4.9.2.4.1: (for some reason, not on 4.9.2.3.2! 😊) in the same step 3 (release of the resources in the old 5G AN) have some text that requires rewording

All references are within 23.502

Taking an example from 4.9.2.1 (Handover of a PDU Session procedure from untrusted non-3GPP to 3GPP access (non-roaming and roaming with local breakout)) step 3:

the text in revision mark italics has been added by me

3.  If the User Plane of the PDU Session is activated in non-3GPP access, the V-SMF executes the release of resources in non-3GPP access by initiating a Namf_Communication_N1N2MessageTransfer (to send N2 resource release request) which triggers performing steps 4 to 7 specified in clause 4.12.7, followed by step 7a specified in clause 4.3.4.2 in order to release the resources over the source non-3GPP access. Because the PDU Session shall not be released, the SMF shall not send the PDU Session Release Command to the UE. Hence, in steps 4 and 7 of clause 4.12.7 as well as in step 7a of clause 4.3.4.2, the messages do not include the N1 SM container but only the N2 Resource Release Request (resp. Ack). Since the PDU Session is not to be released, the SMF shall not execute step 7b of clause 4.3.4.2 and the SM context between the AMF and the SMF is maintained.

Figure 4.12.7-1: UE Requested PDU Session Release via Untrusted non-3GPP access

Figure 4.3.4.2-1: UE or network requested PDU Session Release for non-roaming and roaming with local breakout

…and why exactly would the SMF not execute step 7b of clause 4.3.4.2 ?

Step 7b of 4.3.4.2 is

7b.                The SMF responds to the AMF with an Nsmf_PDUSession_UpdateSMContext response.

It looks strange that SMF does not answer to AMF request in step 7a

The description should say “the SMF shall not execute step 11 of clause 4.3.4.2.” (The SMF invokes Nsmf_PDUSession_SMContextStatusNotify to notify AMF that the SM context for this PDU Session is released.) instead of “the SMF shall not execute step 7b of clause 4.3.4.2.”

# 3 Proposal

It is proposed to agree on the associated CR(s) with the same title (from R16 on)