3GPP TSG-SA WG2 Meeting #148E	S2-2108946

# E-Meeting, 15th- 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0071 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the support of satellite access for Local MBS service and Location dependent MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Since the beam footprint size of one satellite cell may be equivalent to multiple TAs, especially over the GEO obit, to support a finer granularity MBS service area, TAI can be used to identify a MBS service area instead of cell ID for satellite access. Considering that MBS service area refers to a fixed geographical area, for an earth-moving cell, the coverage is not earth fixed, so cell ID can’t be used to identify the MBS service area. Some clarification need to be made to address the above issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A NOTE is added for the clarification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How MBS service area over satellite access is identified is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2.1	General

A Local MBS service is an MBS service provided in one MBS service area. A location dependent MBS service is an MBS service provided in several MBS service area(s). An MBS service area is identified by a cell list or a tracking area list. The MBS service area could be geographical area information or civic address information, and NEF/MBSF translates the location information to Cell ID list or TAI list as MBS service area, see clause 7.1.1.2.

NOTE 1: A finer granularity of MBS service area over satellite access may be identified by TAI list instead of Cell ID list in case of fix cells on earth.

NOTE 2: For this relese, Local MBS service and Location dependent MBS service is not supported over the earth moving satellite cell.

If the multicast session relates to a Local MBS service or a location dependent MBS service and the SMF configures 5GC Individual MBS traffic delivery, the SMF subscribes at the AMF to notifications about the "UE location" or UE moving in or out of a subscribed "Area Of Interest"" event using the Namf_EventExposure service.

For multicast communication, local MBS and location dependent MBS services may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS. If the SMF obtains a notification about the UE location, it checks whether the UE is still in the MBS service area of the multicast session. For a local MBS service, if the UE is no longer in the MBS service area, the SMF terminates the 5GC Individual MBS traffic delivery towards the UE. For a location dependent MBS service, if the UE is no longer in the current MBS service area, the SMF determines whether the UE is in another MBS service area of the multicast session; if so, the SMF configures the UPF to send multicast data relating to the new MBS service area towards the UE. If the SMF terminates the 5GC Individual MBS traffic delivery towards the UE, it unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

# * * * * End of change(s) * * * *