SA WG2 Meeting #S2-148E	S2-2108992

15 - 22 November, 2021, Electronic meeting

3GPP TSG RAN WG2 Meeting #116-e	R2-2111583

Electronic, 1st - 12th Nov, 2021

Title:	Reply LS on discovery and relay (re)selection

Response to:	R2-2111236 (S2-2107972)

Release:	Release 17

Work Item:     	NR_SL_Relay-Core

Source:         	RAN2

To:             	SA2, RAN3

CC:

Contact Person:

Name:	Hao Xu

E-mail Address:	xuhao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the LS on discovery and relay (re)selection. RAN2 had discussed the LS in RAN2#116-e meeting, and provided RAN2 understanding on these issues as below:

Q1) SA2 has assumed 5G MOCN architecture is supported for 5G ProSe Layer-2 UE-to-Network Relay as described in clause 4.2.7.2 of TS 23.304, and would like to ask RAN2 to confirm this assumption. SA2 has also realized PLMN IDs are required (before Layer-2 link has been established) for the Layer-2 Remote UE to perform PLMN selection as well as Relay selection under 5G MOCN architecture, and would like to know whether PLMN IDs are forwarded by Layer-2 UE-to-Network Relay to Layer-2 Remote UE via the AS layer message.

[Answer]:

For Rel-17 U2N sidelink relay, RAN2 discussed whether RAN sharing can be supported for the NG-RAN node for Rel-17 Layer-2 UE-to-Network Relay. Unfortunately, no consensus was reached. Furthermore, RAN2 has made working assumption:

Agreement: WA: cellAccessRelatedInfo from SIB1 is forwarded before PC5-RRC connection. FFS the exact signalling.

plmn-IdentityList is included in cellAccessRelatedInfo. If SA2 conclude RAN sharing should be supported, RAN2 majority prefer to deliver the non-serving PLMN IDs to remote UE via discovery message and further discussion on whether to include it in a RRC container of discovery message or not will be carried out in RAN2. Other aspects of RAN sharing have not been discussed.

Q2) SA2 has realized TAI is needed for 5G ProSe Layer-2 Remote UE to determine the type of initial access message (Mobility Registration Update or Service Request), and would like to ask whether TAI is forwarded by 5G ProSe Layer-2 UE-to-Network Relay to the 5G ProSe Layer-2 Remote UE via the AS layer message.

[Answer]:

RAN2 confirms that TAI can be forwarded by 5G ProSe Layer-2 UE-to-Network Relay to the 5G ProSe Layer-2 Remote UE after PC5 connection establishment via PC5-RRC message. And RAN2 has made a working assumption that TAI can also be forwarded by 5G ProSe Layer-2 UE-to-Network Relay to the 5G ProSe Layer-2 Remote UE before PC5 connection establishment (FFS the exact signalling).

Q3) SA2 has discussed the Editor’s Note in TS 23.304 clause 6.4.3.6:

Editor’s Note 	Whether the Layer-2 link modification procedure is also applicable to ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay requires cooperation with RAN2.

SA2 understands that during the Layer-2 link establishment procedure the 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-2 Remote UE do not interact with the QoS Info (the information about PC5 QoS Flows), meaning there is no PC5 QoS Flow established in the PC5 unicast link between Layer-2 UE-to-Network Relay and Layer-2 Remote UE and the QoS handing is therefore setup by RAN. SA2 would like to ask as the Layer-2 link modification procedure is used to add/modify/remove PC5 QoS Flow(s) in the PC5 unicast link, whether this procedure is applicable or not to the Layer-2 UE-to-Network Relay?

[Answer]:

RAN2 confirms during the Layer-2 link establishment procedure the Relay UE and Remote UE do not interact with the PC5 QoS Flows Info. Whether the Layer-2 link modification procedure is used should be decided by SA2.

Q4) Per TS 23.304 clause 6.6.2, NG-RAN is provided with 5G ProSe authorised information indicating whether a UE is authorized to use 5G ProSe Direct Discovery, 5G ProSe Direct Communication, to act as a 5G ProSe Layer-2 UE-to-Network Relay, a 5G ProSe Layer-3 UE-to-Network Relay and a 5G ProSe Layer-2 Remote UE. NG-RAN is not provided with authorisation information for whether a UE is authorised to act as a 5G ProSe Layer-3 Remote UE.

Is the authorisation information for whether a UE can act as a 5G ProSe Layer-3 Remote UE needed by NG-RAN to enable configuring the UE with correct discovery configuration information via dedicated signalling?

[Answer]:

Whether authorization information for L3 remote UE is needed for NG-RAN can be decided by RAN3. The full set of relay discovery agreements is listed in Appendix for reference.

2. Actions:

To SA2: RAN2 kindly asks SA2 to take the above information into account for the future work.

To RAN3: RAN2 kindly asks RAN3 to take the above information into account for the future work and feedback on Q4.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 #116bis-e 		Jan 17 – 25 2022	         Electronic Meeting

TSG-RAN WG2 #117-e			Feb 21 – Mar 3 2022	   Electronic Meeting

Appendix- full set of relay discovery agreements

RAN2#113-bis-e

As in LTE, the RRC_IDLE/RRC_INACTIVE relay UE is able to perform discovery message transmission, in case:

-	Uu RSRP is above a configured minimum threshold by a hysteresis and below a configured maximum threshold by a hysteresis, or

-	only minimum threshold is provided and Uu RSRP is above the minimum threshold by a hysteresis, or

-	only maximum threshold is provided and Uu RSRP is below the maximum threshold by a hysteresis

As in LTE, the RRC_IDLE/RRC_INACTIVE remote UE is able to perform discovery message transmission, if and only if Uu RSRP of serving cell is below a configured minimum threshold by a hysteresis.

Define threshHighRelay and threshLowRelay for relay UE and threshHighRemote for remote UE. The value range for the three thresholds can be half of RSRP-Range specified in TS 38.331.

One new SL-SRB4 is used for all discovery messages. Its parameters will be fixed and defined as SCCH configuration in 38.331. (FFS on the LCH priority in Proposal 8b)

No ciphering and integrity protection in PDCP layer is needed for the discovery messages.

Shared resource pool shall be the baseline for discovery message transmission/reception.

For determining whether remote UE and/or relay UE in RRC CONNECTED can trigger discovery message transmission, i.e., the remote UE and relay UE in the RRC_CONNECTED can use the threshold based methods as in IDLE/INACTIVE, to determine whether it is allowed to perform discovery message transmission.

Relay UE and remote UE (IC) in RRC CONNECTED can use the discovery configuration provided via dedicated signalling if available.

Relay UE and remote UE (IC) in RRC IDLE or RRC INACTIVE shall use the discovery configuration provided via SIB if available.

WA: L3 relay UE uses pre-configuration for discovery, only if the discovery SIB configuration is not provided by gNB, in case its serving carrier is not shared with carrier for sidelink operation. Otherwise, L3 relay UE uses the configuration for discovery provided by gNB.

L2 relay UE will always use the discovery configuration provided by gNB (either via SIB or dedicated signalling).

FFS: Remote UE (regardless of L2 relaying or L3 relaying) performs discovery based on pre-configuration, only if the discovery configuration is not provided by gNB (regardless not provided, or not able to provide, or not able to obtain in OOC, etc.). Otherwise, Remote UE uses the configuration for discovery provided by gNB.

RAN2#114-e

RAN2 confirm the SI conclusion that for L2 remote UE which is out-of-coverage, and is neither in RRC_CONNECTED nor RRC_IDLE/INACTIVE, it can rely on pre-configuration.

RAN2 confirm the SI conclusion that for L3 remote UE which is out-of-coverage, and is neither in RRC_CONNECTED nor RRC_IDLE/INACTIVE, it should follow pre-configuration.

RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE (i.e., either in RRC CONNECTED or RRC IDLE/INACTIVE), it should follow network configuration, i.e., SIB or dedicated signalling, if available.

RAN2 agree that for relay/remote UE in RRC IDLE/INACTIVE state, in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency

-	If there is Uu deployedcoverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency.

-	If there is no Uu deployedcoverage at the concerned frequency, UE shall rely on pre-configuration.

RAN2 agree that for relay/remote UE in RRC IDLE/INACTIVE state, in-coverage on the serving frequency，if the serving frequency is shared with concerned SL frequency

-	If there is no discovery related SIB broadcasted on the serving carrier, UE does not perform SL discovery transmission/reception on the concerned frequency.

RAN2 agrees to reuse Rel-16 power control mechanism for transmission of discovery messages.

The same PDCP data PDU format as SL-SRB0 is used for sidelink discovery message (SL-SRB4), and the SDU type field is not used for SL-SRB4.

RAN2 agrees to postpone the discussion related to resource allocation to after RAN#92-e.

RAN2 to postpone the issue on network capability differentiation to stage 3 ASN.1 discussion.

RAN2 rely on SA2 on the L2 ID design for discovery message. No LS is needed.

De-prioritize additional condition for discovery transmission/reception in Rel-17.

NOTE 1: Proposal 1b was edited after agreement to change “deployed” to “coverage”.

NOTE 2: Proposal 9 was edited after agreement to remove the FFS, per the organisational email discussion [AT114-e][600].

RAN2 agrees that for relay/remote UE in RRC IDLE/INACTIVE state, and in-coverage on the serving frequency, if there is discovery related SIB broadcasted on the serving frequency, and if the configuration of concerned SL frequency is included within the SIB of the serving frequency but the Tx resource pool configuration is absent, UE shall enter RRC CONNECTED state to acquire dedicated configuration on Tx resource pool.

RAN2 agree that RRC_CONNECTED relay/remote UE which are in-coverage on the serving frequency, if there is discovery related SIB broadcasted on the serving frequency, and if the configuration of concerned SL frequency is included within the SIB of the serving frequency, it can only use the SL discovery Tx resource configuration provided by dedicated signalling if provided, or not transmit discovery if not provided.

RAN2 agree that RRC_CONNECTED L3 relay/remote UE or layer 2 remote UE which are in-coverage on the serving frequency, and the serving frequency is not shared with concerned frequency, if the configuration of concerned SL frequency is absent within the SIB of the serving frequency or if there is no discovery related SIB on the serving frequency,

-	If there is Uu coverage at the concerned SL frequency, UE shall 1) rely on the discovery related SIB, if any broadcasted in the concerned SL frequency; Or 2) if there is no discovery related SIB on the concerned SL frequency, UE does not perform SL discovery transmission/reception on the concerned frequency.

-	If there is no Uu coverage at the concerned frequency, UE shall rely on pre-configuration.

RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE and in RRC IDLE/INACTIVE state, if the network configuration is not available, i.e., SIB, remote UE shall rely on pre-configuration to perform discovery.

RAN2 agrees to down-prioritize discovery specific resource allocation optimization in this release.

RAN2 agrees to down-prioritize the support of discovery gaps in this release.

RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE and in RRC CONNECTED state, if the network configuration is not available, i.e., SIB or dedicated signalling, remote UE shall rely on pre-configuration to perform discovery.

RAN2 agrees dedicated discovery resource pool is supported besides shared resource pool configuration, whether it is configured is based on network implementation. And PHY layer parameters and design shall reuse the Rel-16 legacy resource pool design (including resource allocation design).

RAN2 agree that the UE selection between dedicated and shared pool can be discussed as a stage 3 issue after RAN#92-e.

RAN2 agrees to fix the priority value as 1 of sidelink discovery message in the specification.

RAN2#115-e

FFS if the network can configure shared and dedicated pool simultaneously.

For mode 1, if agreed that both shared and dedicated resource pools can be configured, it is up to gNB which one the UE should use to transmit discovery message.

For mode 2, if agreed that both shared and dedicated resource pools can be configured, downselect from the following options:

- Left to UE implementation

- Dedicated pool should be prioritised

- Shared pool should be prioritised

Discovery for IC relay UE, for IC remote UE which has not been connected to network via a relay UE, and for OOC remote UE which has not been connected to network via a relay UE, relay discovery reuses the Rel-16 V2X resource allocation principles.

For UE (including IC remote UE and OOC remote UE) which has been connected to network via a relay UE, only resource allocation mode 2 can be used.

RAN2#116-e

If only shared TX pools are configured in SIB/RRC/Pre-config, all the configured TX pools can be used for discovery and SL communication, without extra indication required.

Deprioritize the discussion on UE which is only interested in relay discovery rather than SL communication.

For relay discovery, dedicated pools can be configured simultaneously with TX shared pool in SIB/RRC/Pre-configuration.

As baseline, TX shared pool can only be used for SL communication in case dedicated and shared pools are configured simultaneously.  FFS if network can also configure a setting where both shared and dedicated pools can be used for SL discovery.

The discovery dedicated exceptional resource pool is not introduced.

The exceptional pool usage condition for discovery can follow the legacy Rel-16 mechanism, i.e., UE can use the exceptional resource pool to transmit discovery message when T301, T304, T310 or T311 is running for mode 1, or when there is no available sensing result for mode 2.

RLC UM mode is used for SL-SRB4.

The transmitting PDCP/RLC entity establishment for SL-SRB4 is requested by upper layer, e.g., if the transmission of PC5 discovery message for a specific destination is requested by upper layers, establish the corresponding PDCP/RLC entity for PC5 discovery message.

PDCP entity re-establishment for SL-SRB4 is not supported.

The PDCP entity release for a SLRB of sidelink discovery can be requested by the upper layers.

Reuse SIB12 to carry the relay/discovery related configuration.