3GPP TSG-WG SA2 Meeting #148E e-meeting 	S2-2109176

Elbonia, November 15 – 22, 2021	(Revision of S2-2108651 )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0002 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS Session Management vs Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 7.1.1, there is following EN: Editor's note: The term "configuration" can be revisited in the next meeting in clauses 7.1.1.2~7.1.1.7 Currently clause 7.1.1 has the following structure: 7.1 Common procedure for Multicast and Broadcast 7.1.1 MBS Session Management 7.1.1.1 General 7.1.1.2 Initial MBS session configuration without PCC 7.1.1.3 Initial MBS session configuration with PCC 7.1.1.4 Removal of MBS session configuration without PCC 7.1.1.5 Removal of MBS session configuration with PCC 7.1.1.6 MBS Session Update without PCC 7.1.1.7 MBS session configuration Update with PCC In our view, #1 “Configuration” is a well-established term/concept intended for O&M or O&M-like operations in Telecom, therefore extending “configuration” to describe MBS Session management procedures is both misleading and confusing. Particularly for broadcast MBS Session, Using “configuration” to represent AF initiating MBS Session creation towards the MB-SMF and MB-SMF requesting to establish NG-RAN resource via AMF is not appropriate. #2 Using “configuration” to represent MBS Session management (with/wo PCC) is inconsistent with the common understanding of session management procedures. In previous meetings, there was comment that wording “SMF configures UPF” is used in 23.501/23.502, which is true, however, TS 23.501 has also occurrences of “The SMF is responsible for instructing the UP function…” or similar wording, which in our view is more proper wording. There was also comment that “URSP configuration” is used in 23.501/23.502. In our view, “configuration” is used properly in URSP because it is related to UE’s configuration. Rev02: 1. There’s no “Release and deletion” operation, we only have “Release” and “Deletion” operation separatly, so “Release and deletion” should be “Deletion”. 2. There’s no “NULL” state, and “NULL” cannot distinguish “Start” and “End”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use “MBS Session Creation/Update/Deletion” instead of wording with “MBS Session Configuration” Rev02: 1. Change 4.3-4 to 4.3-3, 4.3-5 to 4.3-4 2. Change figure 4.3-(1-4) 3. Change “NULL” to “Start (NULL)” or “End (NULL)” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misuse of a well-established term/concept leading to inconsistent specification as well as causing confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.2, 4.3, 6.2.3, 6.2.4, 6.2.5, 6.9.1, 7.1.1.1, 7.1.1.2, 7.1.1.3, 7.1.1.4, 7.1.1.5, 7.1.1.6, 7.1.1.7, 7.1.2, 7.2.1.3, 7.2.4.2.1, 7.2.4.2.2, 7.2.4.3.2, 7.3.2, 7.3.4, 9.1.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START OF CHANGES ***************

4.2	MB service provisioning

4.2.1	Multicast data provisioning

An example for the sequence of phases for multicast data provisioning is described in the figure below:

Figure 4.2.1-1: Phases of Multicast data provisioning example.

The following phases are performed for a specific UE:

-	UE Session Join: UE Session Join is the process by which a UE joins an MBS Session, i.e. the UE indicates to 5GC that such UE wants to receive Multicast data identified by a specific MBS Session ID.

-	UE Session Leave: UE Session Leave is the process by which a UE leaves a MBS Session, i.e. the UE no longer wants to receive Multicast data identified by a specific MBS Session ID.

The following phases are performed for a specific service:

-	MBS Session Creation: It is the phase that the information of multicast session is created as described in clause 4.3. This step is optional.

-	Service announcement: Service announcement is used to distribute information toward UEs about the service required for service reception (e.g., IP multicast address(es)) and possibly other service related parameters (e.g. service start time). This step is optional.

-	Session Establishment: It is the phase that multicast session is established as described in clause 4.3.

-	No data receiving: It is the phase when no multicast data is received by 5GC. This step is optional.

-	Data transfer: It is the phase when Multicast data are transferred to the UEs.

-	Session Release: It is the phase that the resources for multicast session is released as described in clause 4.3.

-	Session Deletion: It is the phase that multicast session is deleted as described in clause 4.3.

NOTE:	After session establishment, multicast session state could be switched between Active and Inactive several times, triggered by AF or User Plane event, see clause 7.2.5. 5GC further updates multicast session state towards NG-RAN nodes after Session Establishment.

The phase of Multicast data provisioning is illustrated with the following example of timeline:

Figure 4.2.1-2: Multicast service timeline example

4.2.2	Broadcast data provisioning

An example for the phases of broadcast data provisioning is described in the figure below:

Figure 4.2.2-1: Phases of Broadcast data provisioning

The following phases are performed for a specific service:

-	MBS Session Creation: MBS Session Creation is used by the AF to create the MBS Session towards 5GC. MBS session creation can occur in several steps (e.g. TMGI allocation, provisioning information about MBS session, request to activate the MBS session). The last step of the MBS session creation triggers resource establishment for transmitting the DL Broadcast data between 5GC and NG-RAN.

NOTE:	For broadcast communication, after MBS Session Creation and Session Establishment, the established resources are not only between 5GC and NG-RAN, but also between the AF to 5GC.

-	Service announcement: Service announcement is used to distribute information towards UEs about the service required for service reception (e.g., IP multicast address(es)) and possibly other service related parameters (e.g. service start time). This step can occur in parallel or after the MBS session configuration. However, TMGI allocation is required before. The information of the service announcement could be pre-configured at the UE side, see clause 7.3.1.

-	Data transfer: It is the phase when broadcast data are transferred in the air interface.

-	Session Release and Deletion: It is the point at which there will be no more need to transmit Broadcast data. At Session Release, the resources in 5GS are released and the broadcast session is deleted.

The phase of Broadcast data provisioning is illustrated with the following example of timeline:

Figure 4.2.2-2: Broadcast service timeline

****************** NEXT CHANGES ***************

## 4.3	Multicast session state model

Editor's note:	It is FFS whether to use “Configured” or “Created” as the state name.

The following illustrate the states for the multicast session:

-	Configured state: Information about the multicast session (e.g. QoS information) is available in 5GC NFs (e.g. MB-SMF) serving the multicast session, but no User Plane resources towards NG-RAN are reserved and no MBS data can be transmitted. Only resources at MB-SMF, NEF and MB-UPF are reserved and no multicast data are transmitted. A TMGI can be allocated for the multicast session. UEs may be allowed to join (subject to authorization check and configuration), but the first accepted UE join request will trigger the multicast session establishment towards the NG-RAN and the UE, see clause 7.2.1.

NOTE 1:	The SMF is not involved in the multicast session while the multicast session is in created state.

NOTE 2:	There may be several interim states in the created state, e.g. TMGI requested, or information about the multicast session provided, but these interim states will not be specified in this release.

-	Active state: Multicast session is established and MBS data can be transmitted to the UEs that have joined the multicast session. Radio resources for the multicast session are established. To receive multicast MBS session data, UEs that joined the multicast session shall be in CM-CONNECTED state for receiving data of the multicast session. UEs are allowed to join the multicast session (subject to authorization check). 5GC resources and radio resources for the multicast session are reserved for UEs that joined the multicast session.

-	Inactive state: Multicast session is established but no MBS data is transmitted to the UEs that have joined the multicast session. Radio resources for the multicast session are released, and the UEs that joined the multicast session may be in CM-CONNECTED or CM-IDLE state. UEs are allowed to join the multicast session (subject to authorization check).

The following procedures are defined which result in transition of the multicast session state:

Multicast Session Creation: The AF provides information about the multicast session and optionally request the allocation of a TMGI, see clause 7.1.1.2 and 7.1.1.3. Alternatively, the information about the multicast session can be pre-configured in the network. The creation may indicate whether the multicast session may be established in active or inactive state and when a multicast session can become active. The AF may perform creation in several steps, e.g. to first request TMGI and then provide full information about the multicast session and allow it to be established, or to update the information whether the multicast session is to be in active or inactive state after establishment. Multicast session state transitions from "Start (NULL)" to Configured state.

NOTE 3:	A multicast session can also be created by the operator via OAM configuration or be established without prior creation.

-	Multicast Session Establishment: When the join request of the first UE for the multicast session is accepted, the multicast session is established towards the NG-RAN node and the UE, see clause 7.2.1. Multicast session state transitions from "Start (NULL)" or Configured state to either Inactive or Active state.

-	Multicast Session Activation: See clause 7.2.5.2, Triggered by the 5GC, the radio resources for the multicast session are established and multicast session data starts to be transmitted to the UE. UEs in CM-IDLE state and CM-CONNECTED with RRC Inactive state that joined the multicast session are notified. Activation can be triggered by AF request or data notification from the MB-UPF. Multicast session state transitions from Inactive state to Active state.

NOTE 4:	The AF could not be aware, and the NEF will not be aware, whether a session is in created or established state. An AF may therefore update the session state to request the activation of a session prior to the establishment of the session, and this will determine that the session is subsequently established in active state when the first UE joins, but will not trigger the Multicast Session Activation state transition.

-	Multicast Session Deactivation: See clause 7.2.5.3. Triggered by the 5GC, the radio resources for the multicast session are released and multicast session data stops to be transmitted to the UE. Deactivation can be triggered by AF request or no reception of multicast data by the MB-UPF. Multicast session state transitions from Active to Inactive state.

-	Multicast Session Release: Triggered by the last UE leaving the multicast session (see clause 7.2.2.2), or Multicast Session Deletion procedure (7.1.1.6 or 7.1.1.7), the resources for the multicast session are released in both 5GC nodes and RAN nodes, see clause 7.2.2. Multicast session state transitions from Active or Inactive state to Configured.

-	Multicast Session Deletion: All information about the multicast session is removed from the 5GC, and the TMGI for the multicast session (if allocated during Multicast Session Configuration) is deallocated, see clause 7.1.1.4 or 7.1.1.5. The deletion may be triggered by an AF request. Multicast session state transitions from Configured, Active or Inactive state to "End (NULL)".

Figure 4.3-1: Multicast session states and state transitions

Figure 4.3-2: Multicast session states and state transitions in MB-SMF

Figure 4.3-3: Multicast session states and state transitions in NG-RAN

NOTE 5:	Multicast session states and state transitions in NG-RAN is for illustration purpose, the final decision will be made by RAN WGs.

Figure 4.3-4: Multicast session states and state transitions in SMF

****************** NEXT CHANGES ***************

6.2.3	Location dependent MBS service

A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location-dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE. When UE moves out of an MBS service area and there is no other MBS service area for the MBS session, the network ceases to deliver the content data to the UE.

Information about different MBS service areas for a location dependent MBS service may be provided by one or several AFs or may be configured. Different ingress points for location dependent points for the MBS session are supported for different MBS service area dependent content of the MBS session; different MB-SMFs and/or MB-UPF may be assigned for different MBS service areas in an MBS session. When the different MB-SMFs are assigned for different MBS service areas in an MBS session, the same TMGI is allocated for this MBS session.

The Area Session ID is allocated by MB-SMF in MBS Session creation procedure. MB-SMF allocates Area Session ID for each MBS services area which is unique within the MBS session. MB-SMF needs to further ensure there is no MBS service area overlapping with other MBS service areas that share the same MBS session ID.

NOTE 1:	In this release, deployments topologies with specific SMF Service Areas are not supported, as a result, location dependent service using multicast communication is not supported when a UE moves outside its SMF service area.

NOTE 2:	For location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

NOTE 3:	An example of Location-dependent MBS is a nationwide weather forecast service with local weather reports.

NOTE 4:	Area Session ID is equivalent to Flow ID as specified in TS 23.246 [8].

****************** NEXT CHANGES ***************

6.2.4	Update of broadcast local MBS service and location dependent MBS service

If an MBS service area is updated for broadcast communication, the MB-SMF provides the updated MBS service area to AMFs, and the AMFs provide it to NG-RAN node(s) affected by the previous or updated service area. The RAN nodes terminate transmission of MBS data towards cells that were in the previous service area but are outside the new service area. The NG-RAN nodes start transmission of MBS data towards cells that were outside the previous service area but are inside the new service area.

****************** NEXT CHANGES ***************

6.2.5	Update of multicast local MBS service and location dependent MBS service

If an MBS service area is updated for multicast communication, the MB-SMF provides the updated MBS service area to:

- SMFs,

- and AMF(s), and the AMF(s) forward this information to the relevant NG-RAN(s) that update the MBS service area.

NG-RAN node configures the UE not to receive the MBS data over air interface if it detects the UE was in the previous service area but is outside the updated MBS service area NG-RAN node may release the tunnel for the shared delivery if none of the cells/TAs of the NG-RAN node belongs to the MBS service area any more. NG-RAN node configures the UE to receive the MBS data over air interface if it detects the UE was outside the previous service area but is inside the updated MBS service area, if part of cells/TAs of the NG-RAN node belongs to MBS service area and others outside the MBS service area.

****************** NEXT CHANGES ***************

6.9	MBS Session Context

6.9.1	MBS Session Context

The MBS Session Context contains all information describing a particular MBS session in the 5GS and is created in each node involved in the delivery of the MBS data.

The content of the Multicast MBS Session Context is described in Table 6.9.1-1.

Table 6.9.1-1: Multicast MBS Session context

| Parameter | Description | NG-RAN | MB-SMF | SMF | AMF |
| --- | --- | --- | --- | --- | --- |
| State | State of MBS session ('Active' or 'Inactive ' or 'Configured') | X (note 2) | X | X (note 2) |  |
| SSM (source specific IP multicast address) | IP multicast address identifying the MBS session. |  | X (note 1) | X (note 1) |  |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | x | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) | X |
| MB-SMF | The MB-SMF that handles the MBS session. | X |  | X | X |
| QoS information | QoS information of the MBS session. | X | X | X |  |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X (note 1) | X (note 1) | X (note 1) | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are involved in the multicast session |  |  |  | X |
| AMF | The AMF(s) which are selected for the MBS session | X | X |  |  |
| IP multicast and source address for data distribution | IP addresses identifying the SSM user plane transport for shared delivery between MB-UPF and NG-RAN and for individual delivery between MB-UPF and UPF when the IP multicast transport is used. | X (note 1) | X (note 1) | X (note 1) |  |
| SMF | The SMF(s) that manages the associated PDU session. |  | X |  |  |
| UE ID | ID identifying the UE that successfully join the Multicast MBS Session. For NG-RAN it is NGAP UE ID and for SMF it is SUPI. | X (note 3) |  | X (note 3) |  |
| IP address for distribution | The IP addresses and TEID of NG-RAN used for the user plane between NG-RAN and MB-UPF and between MB-UPF and UPF when Point to Point tunnel is used. | X (note 1) | X (note 1) | X (note 1) |  |
| TEID for data distribution | The tunnel ID used for receiving the multicast data for shared delivery by NG-RAN and for individual delivery by UPF | X | X | X |  |
| MB-PCF | The MB-PCF that provides policy control for the MBS session. |  | X (note 1) |  |  |
| NOTE 1: It is an optional parameter. NOTE 2: The value 'Configured' is not applicable for NG-RAN and SMF. NOTE 3: the UE ID is available within the UE Context which contains the MBS information. |  |  |  |  |  |

In Broadcast MBMS mode, an MBS Session Context is created in the NG-RAN, AMF, MB-SMF and MBSF as a result of the MBS Session Start procedure.

The content of the Broadcast MBS Session Context is described in Table 6.9.1-2.

Table 6.9.1-2: Broadcast MBS Session context

| Parameter | Description | NG-RAN | AMF | MB-SMF |
| --- | --- | --- | --- | --- |
| TMGI | Temporary Mobile Group Identity allocated to the MBS Session. | X | X | X |
| Area Session Identifier | Used for MBS session with location dependent content. When present, the Area Session Identifier together with the TMGI uniquely identify the MBS Session in a specific MBS service area. | X (note 1) | X (note 1) | X (note 1) |
| AMF | The AMF(s) which are selected for the MBS session | X |  | X |
| QoS information | QoS information for the MBS Session, including the QoS parameters of QoS flows. | X |  | X |
| MBS Service Area | Area over which the MBS session data is distributed (i.e. Cell ID list or TAI list). | X | X | X |
| NG-RAN Node ID(s) | NG-RAN nodes which are selected for the broadcast session |  | X |  |
| IP multicast address for data distribution | IP addresses identifying the user plane transport used for shared delivery between MB-UPF and NG-RAN when the IP multicast transport is used. | X (note 1) |  | X (note 1) |
| NG-RAN IP Address for data distribution | The IP address of NG-RAN used for the user plane between NG-RAN and MB-UPF when Point to Point tunnel is used. | X (note 1) |  | X (note 1) |
| TEID for data distribution | The tunnel ID used for receiving the broadcast data for shared delivery by NG-RAN | X |  | X |
| MB-PCF | The MB-PCF that provides policy control for the MBS session. |  |  | X (note 1) |
| NOTE 1: It is an optional parameter. |  |  |  |  |

****************** NEXT CHANGES ***************

7.1.1	MBS Session Management

7.1.1.1	General

The call flows in Clause 7.1.1 and clause 7.3 show a "NEF/MBSF", but as detailed in Annex A, there can be different related network deployment involving either only NEF, or MBSF, or both.

The interactions between "NEF/MBSF" and MB-SMF, PCF, BSF and NRF depicted in the call flows apply for NEF, MBSF or a combined NEF and MBSF, depending on network deployment. They may also apply for AF in trusted domain where NEF is not mandated.

However, the interactions between AF and "NEF/MBSF" depicted in the call flows only apply for the NEF.

Interactions between AF and MBSF based on the MB2 interface follow TS 23.468 [10].

Interactions between AF and MBSF based on the xMB interface follow TS 26.348 [11].

Services offered by the MBSF and related interactions based on that service between MBSF and AF or NEF (if MBSF and NEF are split as shown in configuration 2) are specified in TS 26.502 [18].

Detailed interactions between the MBSF or NEF and the MBSTF are specified in TS 26.502 [18].

#### 7.1.1.2	MBS Session Creation without PCC

This procedure is used by the AF to start the MBS Session towards 5GC and consist of TMGI allocation, and MBS session creation, and they apply to both multicast and broadcast communications unless otherwise stated.

For multicast, MBS session establishment procedure triggered by UE join requests may follow the MBS session creation procedure to reserve resources towards NG-RAN. For broadcast, the MBS session start procedure to reserve resources towards NG-RAN is triggered by the MBS session creation procedure.

For both broadcast and multicast communication, the TMGI allocation may be separated from the MBS Session creation request.

For multicast communication, TMGI allocation procedure is applicable if TMGI is used as MBS Session ID.

Figure 7.1.1.2-1: MBS Session Creation without PCC

Steps 1 to 6 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1.	AF sends Nnef_TMGI_Allocate Request (TMGI number) message to NEF/MBSF to request allocation of a TMGI(s) to identify new MBS session(s).

NOTE 1:	Depending on the network deployment and use case, MB-SMF may receive requests from AF directly, or via NEF, or via MBSF, or via NEF and MBSF.

2.	NEF checks authorization of AF.

NOTE 2:	NEF is not required if AF is in trusted domain.

3.	NEF/MBSF discovers and selects an MB-SMF using NRF or based on local configuration,

4.	NEF/MBSF sends an Nmbsmf_TMGI_Allocate Request (TMGI number) message to the MB-SMF.

5.	MB-SMF allocates TMGI(s) and returns the TMGI(s) to the NEF/MBSF via the Nmbsmf_TMGI_Allocate response (TMGI(s), expiration time).

6.	The NEF or MBSF responds to the AF by sending an Nnef_TMGI_Allocate Response (TMGI(s), expiration time).

7.	The AF may perform a Service Announcement towards UEs. The AF informs UEs about MBS Session information with MBS Session ID, e.g., TMGI, source specific multicast address, and possibly other information e.g., MBS service area, session description information, etc.

The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information. Amongst them, Cell ID list and TAI list shall only be used by AFs who reside in trust domain, and when the AFs are aware of such information.

The UE needs to be aware if the service is broadcast or multicast to decide if JOIN is to be performed.

Editor's note:	How to do service announcements requires SA WG4 /WG6 coordination.

8.	AF of content provider may provide description of an for an MBS session (possibly providing information for a previously allocated TMGI to NEF via a Nnef_MBSSession_Create request (([MBS Session ID], service type, MBS information, [TMGI allocation indication]). If step 1-6 has not been executed before, the AF may provide a source specific multicast address or it may request that the network allocates an identifier for the MBS session (i.e., TMGI). The AF provides the service type (i.e. either multicast service or broadcast service). MBS information may further include QoS requirements, for a multicast session MBS authorization information (e.g., or an indication that any UE may join), MBS service area, start and end time of the MBS session and MBS session state (active/inactive). In addition, MBS information may also indicate whether the allocation of an ingress transport address is requested.

If geographical area information or civic address information was provided by the AF as MBS service area, NEF/MBSF translates the MBS service area to Cell ID list or TAI list.

Editor's note:	What other information is to be sent by AF is FFS.

9	NEF/MBSF checks authorization of content provider.

10.	NEF/MBSF discovers MB-SMF candidates and selects MB-SMF as ingress control node, possibly based on MBS service area. If a TMGI is included in step 8, NEF/MBSF finds MB-SMF based on that TMGI.

11.	NEF/MBSF sends Nmbsmf_MBSSession_Create Request (MBS Session ID, service type, TMGI allocation indication, MBS service area information, ingress transport address request indication) to MB-SMF, to request MB-SMF to reserve ingress resources for a MBS distribution session, The NEF/MBSF provides MBS Session ID or request allocation of a TMGI, and indicate the requested service type (either multicast service or broadcast service) and MBS session state (active/inactive). It also indicates that the allocation of an ingress transport address is requested if this was requested in step 8, or if the MBSF decides to insert an MBSTF into the user plane for the MBS session. If provided in step 8, the request includes the indication that any UE may join.

The MBS service area is provided by NEF/MBSF to the MB-SMF if provided by the AF in step 8.

12.	If requested to do so, or if a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF allocates a TMGI.

If a source specific multicast is provided as MBS Session ID in step 11, the MB-SMF updates its NF profile at the NRF with the serving MBS Session ID. If an MBS service area information was received in step 11, the MB-SMF updates its NF profile at the NRF with that information.

NOTE 3:	If TMGI is used to represent an MBS Session, MB-SMF does not need to update NRF if the TMGI range(s) supported by an MB-SMF is already included in the MB-SMF profile when MB-SMF register itself into NRF.

13.	The MB-SMF derives the required QoS parameters locally.

14	MB-SMF selects the MB-UPF. If the allocation of an ingress transport address was requested in step 11, the MB-SMF requests the MB-UPF to reserve user plane ingress resources. If multicast transport of the MBS data towards RAN nodes is to be used, the MB-SMF also request the MB-UPF to reserve for the outgoing data a tunnel endpoint and the related identifiers (source IP address, source specific multicast address and GTP Tunnel ID) and to forward data received at the user plane ingress resource using that tunnel endpoint.

If the allocation of an ingress transport address was not requested in step 11, the MB-SMF provides the source specific multicast address received as MBS session ID to the MB-UPF and requests the MB-UPF to join the corresponding multicast tree from the content provider. The MB-SMF may also defer the configuration to join the corresponding multicast tree e.g. based on information that the session is inactive, QoS requirements and MBS start/end time until receiving the first query for the MBS session as part of the establishment procedure in clause 7.2.1.3, or until receiving a request to activate the MBS session via the MBS Session Update procedure in clause 7.1.1.6 or 7.1.1.7..

15.	If requested, MB-UPF selects an ingress address (IP address and port) and a tunnel endpoint for the outgoing data and provides it to MB-SMF.

16.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1.

17.	MB-SMF indicates the possibly allocated ingress address to the NEF/MBSF. MB-SMF may include TMGI if it is allocated in step 9. It also indicates the success or failure of reserving transmission resources.

18.	[Optional] If the MBSF decides to use an MBSTF, the NEF/MBSF provides the ingress address received in step 18 towards the MBSTF as DL destination. If the allocation of an ingress transport address was requested in step 8, the MBSF requests the MBSTF to allocate the user plane ingress resources. If the allocation of an ingress transport address was not requested in step 8, the MBSF provides the source specific multicast address received as Multicast session ID in step 8 and requests the MBSTF to join the corresponding multicast tree from the content provider.

19.	[Conditional on step 19]	If requested, the MBSTF selects an ingress address (IP address and port) and provides it to NEF/MBSF.

20.	The NEF/MBSF-C indicates the possibly allocated ingress address and other parameters (e.g. TMGI) to the AF via an Nnef_MBSSession_Create response ([TMGI], [Allocated ingress address])). If MBS Session ID is not provided in step 8, or the MBS Session ID is source specific multicast address, the NEF/MBSF provides the allocated TMGI. If AF requests the allocation of an ingress transport address, the message also includes the allocated ingress address

21.	Same as step 7. The AF may also perform a service announcement at this stage.

22.	For multicast communication, depending on configuration UEs can join the MBS Session as specified in clause 7.2.1.

#### 7.1.1.3	MBS Session Creation with PCC

Figure 7.1.1.3-1: MBS Session Creation with PCC

Steps 1 to 7 are optional and only applicable if TMGI is used as MBS Session ID and required to be pre-allocated.

1. to 10:	Same as in Figure 7.1.1.2-1.

11. Same as step 11 in Figure 7.1.1.2-1. In addition, the NEF/MBSF decides based on local configuration or based on parameters received in step 8 (e.g. whether the session comprises several data flows) whether it will invoke the Npcf_MBSPolicy Authorization service for the MBS session. If so, the NEF/MBSF indicates to the MB-SMF that it will also provide a policy authorization for the MBS session to the PCF.

12. Same as step 12 in Figure 7.1.1.2-1.

13.	[Optional] If the NEF/MBSF indicated in step 11 that it will also provide a policy authorization for the broadcast session to the PCF, the MB-SMF selects a PCF and sends an Npcf_MBSPolicyControl_Create Request (MBS session ID) for the MBS session towards the PCF, and defers step 25 until receiving an Npcf_MBSPolicyControl_UpdateNotify for the MBS session. Otherwise the MB-SMF decides based on local configuration whether to invoke the Npcf_MBSPolicyControl service.

14.	[Conditional on step 13] The PCF registers at the BSF that it handles the MBS session by using Nbsf_management_Register Request (MBS Session ID, PCF ID).. It provides an identifier that the policy association is for MBS and the MBS Session ID, its own PCF ID and optionally its PCF set ID.

15.	[Optional] The PCF may retrieve preconfigured policy information for the MBS session (e.g. applicable QoS, the MBS Session-AMBR and/or default 5QI) from the UDR.

16.	[Conditional on step 13] The PCF responds with Npcf_MBSPolicyControl_Create Request (MBS Policy, see clause 6.10) with policies for the MBS Session ID. The MBS Policy may include the Session-AMBR for the MBS session and 5QI for the MBS QoS Flow.

Editor's Note: How PCF determines the MBS policy for MBS QoS Flow without service requirement in this case is FFS.

17.-18	Same as steps 14-15 in Figure 7.1.1.2-1.

19	Same as step 17 in Figure 7.1.1.2-1.

20-21.	[Optional] The NEF/MBSF uses the BSF Discovery service to discover the PCF serving the MBS session with the MBS session ID by using Nbsf_management_Discovery operation.

22.	[Optional] The NEF/MBSF sends an Npcf_MBSPolicy Authorization_Create Request to PCF with the MBS session ID and MBS information:

The PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (e.g. according to the policy input configuration in the UDR).

If the request is not authorized or the required QoS is not allowed, the PCF indicates so in the response to the NEF

23.	[Conditional] If the PCF determined updated policies for the MBS session in step 21, it update the policy information at the MB-SMF. When obtaining a request for the creation of a policy association (signal 21) for a broadcast session, for which it already performs policy control towards an MB-SMF, the PCF always provides a policy update to the MB-SMF; if no real policy update is required, the PCF repeats previous policies or sends an empty update message.

24.	[Conditional] If required by the updated policies, the MB-SMF updates the MB-UPF accordingly.

25.	When obtaining an MBS policy control update from the PCF (signal 23) for a broadcast session, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.1 to request the allocation of resources to for the transmission of the broadcast session.

26.-30	Same as steps 18-22 in Figure 7.1.1.2-1.

Editor's note:	AF sends a create message in step 8 and get a response in step 26. How to avoid the potential procedure handling failure is FFS.

NOTE:	Steps 26-27 can be executed in parallel to steps 20-25.

#### 7.1.1.4	MBS Session Deletion without PCC

This procedure is used by the AF to delete the MBS Session. This procedure may also include TMGI de-allocation. The procedures apply to both multicast and broadcast communications unless otherwise stated. This procedure releases the reserved resources in both 5GC and NG-RAN.

Figure 7.1.1.4-1: MBS Session Deletion without PCC

1.	AF of content provider may request to delete the MBS session (MBS Session ID).

2/3.	If an MBSTF was inserted into the user plane, the MBSF request the MBSTF to release user plane resources.

4.	NEF/MBSF requests MB-SMF to delete resources for the MBS session.

5.	For broadcast session, the MB-SMF triggers resource release towards the AMFs as specified in clause 7.3.2. For multicast session, the MB-SMF triggers resource release towards the SMFs as specified in clause 7.2.2.3.

6/7.	MB-SMF requests the MB-UPF to release user plane resources.

8.	[Conditional] If MB-SMF configured the profile with an MBS session ID when the MBS session was created, the MB-SMF updates its NF profile at NRF to release the MBS Session ID.

9.	MB-SMF responds to the NEF/MBSF.

10.	The NEF/MBSF responds to the AF.

11.	[Optional] AF requests NEF/MBSF to de-allocate TMGI(s),

12.	[Conditional on step 11] NEF/MBSF forwards request to de-allocate TMGI(s) to MB-SMF.

13.	[Conditional on step 12] The MB-SMF responds to the NEF or MBSF by sending a de-allocate TMGI Response message.

14.	[Conditional on step 13] NEF or MBSF forwards de-allocate TMGI Response message to AF.

#### 7.1.1.5	MBS Session Deletion with PCC

This procedure is used by the AF to release the MBS Session. This procedure may also include TMGI de-allocation. The procedures apply to both multicast and broadcast communications unless otherwise stated. This procedure releases the reserved resources in both 5GC and NG-RAN.

Figure 7.1.1.5-1: MBS Session Deletion with PCC

1-3	Same as in Figure 7.1.1.4-1

4.	The NEF/MBSF sends an NMBSPolicyAuthorization_Delete Request to the PCF that handles the Policy of the MBS Session.

5.	The PCF sends an Npcf_MBSPolicyControl_UpdateNotify Request to MB-SMF to release the MBS Policy Control Association.

6-9.	Same as steps 5-8 in Figure 7.1.1.4-1

10.	The MB-SMF sends the Npcf_MBSPolicyControl_UpdateNotify Response to the PCF.

11.	The PCF de-registers at the BSF that it handles the MBS session.

12.	The PCF sends an NpcfMBSPolicyAuthorization_Delete Response to the NEF/MBSF.

13-17.	Same as steps 11-15 in Figure 7.1.1.4-1.

#### 7.1.1.6	MBS Session Update without PCC

This procedure is used by the AF to update the MB service area and/or update QoS of an MBS Session. Updating QoS of an MBS Session may lead to addition of new MBS QoS Flow(s). The procedure applies to both multicast and broadcast communications unless otherwise stated.

Figure 7.1.1.6-1: MBS Session Update without PCC

1.	AF of content provider initiates MBS Session Update to a NEF/MBSF, e.g. to update MBS service area and/or update service requirement, or to activate or deactivate an MBS session. AF may provide updated information for an MBS session (identified by MBS session ID) by sending MBS Session update request ([MBS Session ID], MBS information, AF Identifier). MBS information may include service requirements, MBS service area information, and media information.  The service requirements adjustment may lead to addition of new MBS QoS Flow(s), removal of existing MBS QoS Flow(s) or update of existing MBS QoS Flow(s).

2.	NEF checks authorization of AF.

3.	NEF/MBSF forward the MBS Session Update request to MB-SMF.

4.	The MB-SMF derives any updated QoS parameters locally.

5-6. MB-SMF may need to update MB-UPF, e.g. if new MBS QoS Flow is to be created, or existing MBS QoS Flow is to be deleted.

7.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation), 7.2.6 (for QoS updates) and 7.2.x (for service area updates).

8.	If an MBS service area is being updated, the MB-SMF stores the new service area in its profile at the NRF.

9.	MB-SMF responds to the MBS Session Update.

10.	NEF/MBSF responds to the MBS Session Update.

#### 7.1.1.7	MBS Session Update with PCC

Figure 7.1.1.7-1: MBS Session Update with PCC

1-2.	Same as in Figure 7.1.1.6-1.

For updates of MBS service area and/or MBS session activation/deactivation steps 3 to 6 apply

3.	NEF/MBSF forward the MBS Session Update request to MB-SMF, removing any updates not related to MBS service area and/or MBS session activation/deactivation

4.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.5 (for service activation/deactivation), and 7.2.x (for service area updates).

5.	If an MBS service area is being updated, the MB-SMF stores the new service area in its profile at the NRF.

6.	MB-SMF responds to the MBS Session Update.

For other updates of the service description and QoS related updates, steps 7 to 12 apply.

7.	NEF/MBSF updates the MBS policy Authorization for the MBS session at the PCF and provides the input received from the AF, removing any updates related to MBS service area and/or MBS session activation/deactivation, by sending Npcf_MBSPolicyAuthorization_Update Request message (MBS Session ID, service requirement).

8.	Based on the input received in step 7, the PCF may provide updated policy rules to the MB-SMF by issuing Npcf_MBSPolicyControl_UpdateNotify request message including the updated policy information about the MBS Session.

9-10.	Same as steps 5-6 in Figure 7.1.1.6-1.

11.	For broadcast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.3.3. For multicast communication, the MB-SMF continues the procedure towards the AMF and NG-RAN as specified in clause 7.2.6 (for QoS updates).

12.	The PCF sends the Npcf_MBSPolicyAuthorization_Update response to the NEF/MBSF.

13.	Same as step 10 in Figure 7.1.1.6-1

****************** NEXT CHANGES ***************

### 7.1.2	MB-SMF discovery and selection for multicast/broadcast session

To facilitate the MB-SMF discovery/selection for one multicast/broadcast MBS session, the following mechanism is used:

-	The MB-SMF registers its capability related to multicast/broadcast session management (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), TMGI range, service area) as part of its profile to the NRF by invoking Nnrf_NFManagement_NFRegister. In addition, when an multicast session is created, and the MBS session ID is not yet included in the MB-SMF profile, the MB-SMF updates its profile towards the NRF with the MB Session ID (i.e. TMGI or source specific IP multicast address).

NOTE:	The operator can preconfigure MB-SMF for specific source IP multicast address range or TMGI range.

-	When the UE joins the multicast MBS session via PDU session modification procedures, the SMF serving the PDU session invokes the Nnrf_NFDiscovery_Request Request including the MBS session ID for multicast provided by the UE and optionally other information, i.e. the S-NSSAI and the associated NSI ID (if available), DNN, etc., to query the NRF for MB-SMF information. Based on the MBS session ID and other information for query, the NRF decides whether an MB-SMF serving the MBS session exists. If so, the NRF provides in Nnrf_NFDiscovery_Request Response the information of the MB-SMF currently serving the MBS session. The SMF serving the PDU session selects the MB-SMF currently serving the multicast MBS session, based on the MB-SMF information provided by the NRF. For local MBS services, the SMF takes MB-SMF service area, UE location into account when selecting the MB-SMF. If no MB-SMF serving the multicast session exists, the NRF provides MB-SMF profiles based on the other query information.

-	When the multicast MBS session context is deleted from the MB-SMF, e.g. due to MBS session release, the MB-SMF updates its profile towards the NRF, i.e., removing the MBS session ID which is no longer served by the MB-SMF.

-	During MBS session information provisioning procedures defined in clause 7.1.1.2, unless the MB-SMF information is available by other means, e.g. locally configured in the NEF/MBSF/AF, the NEF/MBSF/AF queries the NRF with information of the multicast/broadcast session (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s)), and selects the MB-SMF(s) based on the MB-SMF information provided by the NRF. For local MBS services, the NEF/MBSF/AF takes MB-SMF service area into account when selecting the MB-SMF(s).

****************** NEXT CHANGES ***************

7.2.1.3	Multicast session join and session establishment procedure

The following steps are executed before the UE requests to join the MBS session:

-	The MBS Session may have has been created in the 5GC (see clause 7.1.1 for details).

-	The UE registers in the PLMN and establishes a PDU session.

-	The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via service announcement.

Figure 7.2.1.3-1: PDU Session modification for UE joining multicast session

1.	To join the multicast group, the UE sends a PDU Session Modification Request which additionally contains one or several MBS Session ID(s) and join request. The MBS Session ID(s) indicate the multicast group(s) that UE wants to join.

2.	[Conditional] Based on the received MBS Session ID and join request, the SMF determines this is MBS Session join request. The SMF checks whether the user is authorized to use the required multicast service.

3.	[Conditional] If SMF has no information about MBS Session context for the indicated MBS Session ID(s), SMF discovers and selects an MB-SMF for the MBS Session via the NRF as described in clause 7.1.2. If no MB-SMF is assigned for the MBS session ID, the SMF may select an MB-SMF and request it configure the multicast session or the SMF may reject the join request.

NOTE 1:	Details how the SMF select an MB-SMF and request it configure the multicast session are left to SMF implementation.

4.	Nmbsmf_ MBSSession_ContextStatusSubscribe request indicates the SMF want to subscribe the MBS session context. For each MBS session in step 1, by using Nmbsmf_ MBSSession_ContextStatusSubscribe request (MBS Session ID) with the immediately reporting flag. SMF interacts with the MB-SMF to retrieve information about the indicated multicast session context information (multicast QoS flow information (e.g., QoS profile(s) for multicast MBS session), [start time], [session status indication (active/inactive)], [MBS session authorization information (MBS session open for any user)], LL MC address]) and to subscribe to events notifications related to the multicast session.

If it is the first time for the MB-SMF to receive Nmbsmf_ MBSSession_ContextStatusSubscribe request of the indicated MBS Session from SMF, MB-SMF learns it is the first UE joining the multicast group. For multicast transport between MB-UPF and content provider, if it is the first UE joining the multicast group, and MB-UPF has not joined the multicast tree in the MBS session creation procedure, the MB-SMF requests the MB-UPF to join the multicast tree towards the AF/MBSF, otherwise MB-SMF will not send the request to the MB-UPF. The MB-SMF determines whether the user is authorized to join the multicast session as follows: The MB-SMF checks the user subscription data received from the UDM to determine whether the user is allowed to use any multicast service. If so, the MB-SMF checks the received indication whether the multicast session is open for any user. If the multicast session is not open to any user, the MB-SMF checks the user subscription data received from the UDM to determine whether the MBS session ID is included. If a UE joins prior to the start time of the multicast session, the SMF may accept the join request in step 8 and indicate to the UE the start time, or it may reject the join request with appropriate error cause and possible back-off time. If a UE joins while the multicast session is inactive, the SMF accepts the join request in step 8. If authorization check fails, the SMF indicates cause value in the PDU Session Modification Reject sent to the UE and proceeds with step 8.

NOTE 2:	The MB-SMF can answer the Nmbsmf_ MBSSession_ContextStatusSubscribe request either based on information received in the MBS session creation procedures in Clause 7.1.1 or based on preconfigured information. The pre-configuration also includes information about the MBS session stored in the NRF. If the MB-SMF uses preconfigured information, the pre-configuration also include MB-UPF configuration.

7.	SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response (N2 SM information (PDU Session ID, MBS Session ID, [updated PDU Session information], [mapping information between unicast QoS flow(s) and multicast QoS flow (s)]), N1 SM container (PDU Session Modification Command)) to:

-	create an MBS session context for the indicated MBS session in the RAN, if it does not exist in the RAN already; and

-	inform the NG-RAN about the relation between the multicast context and the UE's PDU Session context by including the MBS session ID and the mapping between the multicast QoS flow(s) and associated QoS flow(s).

Based on operator policy, the SMF may prepare for 5GC individual MBS traffic delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session's unicast QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

NOTE 3:	Detailed information included in N2 SM information will be aligned with by RAN WG3.

NOTE 4:	A PDU Session UP activation is not triggered by message 7 if it only includes information related to the multicast session and associated QoS flows and is received by an MBS capable NG RAN node.

Editor's Note: The implication of not triggering PDU Session UP activation in NG-RAN when SMF informs the NG-RAN of UE join requires RAN collaboration.

Editor's note:	Possible PCF interactions related to the multicast QoS flows are FFS.

8.	The N2 message, which includes the multicast session information and PDU session modification information is sent to the NG-RAN.

If the 5G MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise if the MBS is supported by NG-RAN, 5GC shared MBS traffic delivery is adopted.

If the NG-RAN supports 5G MBS, the NG-RAN uses the MBS Session ID to determine that the PDU Session identified by the PDU Session ID is associated with the indicated multicast session.

If the multicast QoS information is received and the NG-RAN supports MBS, the associated unicast QoS flow information is not used to allocate the radio resource and CN resource.

NOTE 5:	It is NG-RAN that decides whether radio resource is allocated or not, and it is NG-RAN/UPF that decides whether multicast transport or unicast transport is used between the NG-RAN/UPF and the MB-UPF.

9. [Conditional]	If shared tunnel has not been established for the MBS session towards the NG-RAN node, the procedures in clause 7.2.1.4 for the Establishment of shared delivery toward RAN node are executed. Step 9 is executed separately for each MBS session.

10.	The NG-RAN node performs AN specific signalling exchange with the UE to establish radio resource for the MBS session if not established yet. If the NG-RAN does not support MBS, radio resource are reconfigured for unicast transmission of the MBS data over the associated PDU session. As part of the AN specific signalling exchange, the N1 SM container (PDU Session Modification Command) is provided to the UE.

11.	The NG-RAN node sends the PDU session modification response.

If the MBS is not supported by NG-RAN, the accepted unicast QoS flow is included in the N2 SM response container. If the MBS is supported by NG-RAN, the N2 SM response container includes the accepted multicast QoS flow.

12.	The AMF invokes Nsmf_PDUSession_UpdateSMContext request ([N2 SM container]) to the SMF.

Per the accepted unicast QoS flow information, the SMF determines the delivery mode, i.e. whether 5GC individual MBS traffic delivery is used for multicast data transmission.

NOTE 6:	If the shared tunnel is used, no interaction with UPF is needed for the indicated MBS session

[Conditional] Step 13 is used for 5GC Individual MBS traffic delivery, if the related NG-RAN does not support multicast .If a shared tunnel between the UPF (PSA) and MB-UPF for 5GC individual MBS traffic delivery has not yet been established by the SMF for the multicast session, steps 13a to 13e are executed. Step 13f is executed irrespective of that.

13a.	The SMF contacts the UPF to request the creation of a tunnel and provides the MBS session ID. The UPF indicates to the SMF whether the tunnel for this MBS session is newly allocated (as there can be multiple SMFs interacting with the same UPF for the same MBS Session). If the UPF determines to use unicast transport over N19mb, the UPF allocates a DL N19mb Tunnel endpoint for the MBS session if the SMF request is the first one to allocate DL N19mb Tunnel endpoint for the MBS Session in the UPF. The UPF includes the DL Tunnel Info in the response to the SMF. The DL tunnel info includes the downlink tunnel ID and the UPF address. The UPF is now ready for receiving the MBS data from the MB-UPF and forwarding the data to the PDU session.

Editor's Notes: whether 13b to 13e can be skipped for multicast N10mb is FFS

13b.	If the UPF indicates the DL N19mb Tunnel is newly allocated, the SMF invokes Nmbsmf_MBSsession_ ContextUpdate request (MBS Session ID, [DL tunnel info]) towards MB-SMF that includes MBS Session ID and downlink tunnel info of UPF, for establishing the multicast session transport between MB-UPF and UPF.

13c.	If the DL tunnel info of the UPF is received, MB-SMF configures MB-UPF to transmit the multicast session data towards UPF using the possibly received downlink tunnel ID.

13d.	MB-SMF responds to SMF through Nmbsmf_MBSSession_ ContextUpdate response (MBS Session ID, [multicast DL tunnel info]). If the UPF DL tunnel info for unicast transport is not received by the MB-SMF, multicast transport between MB-UPF and UPF is to be used, and the SMF includes the downlink tunnel information with the transport multicast address for the multicast session.

13e.	For multicast transport between MB-UPF and UPF, SMF configures UPF to receive the multicast session data. If multicast transport over N19mb is used, the UPF joins the source specific multicast group of MB-UPF. The UPF forwards the data to the PDU session.

13f.	The MB-SMF configures the MB-UPF to forward the received multicast session data within the PDU session. (This step may be combined with step 13a or 13e).

14.	The SMF invokes Nsmf_PDUSession_UpdateSMContext response to the AMF.

15.	MB-UPF receives multicast PDUs, either directly from the content provider or via the MBSTF that can manipulate the data.

Steps 16 to 18 are for 5GC shared MBS traffic delivery:

16.	MB-UPF sends multicast PDUs in the N3mb tunnel associated to the multicast session to the NG-RAN. There is only one tunnel per multicast session and NG-RAN node, i.e., all the UEs which have joined the multicast session via the NG-RAN node share this tunnel for reception of the multicast session data.

17.	The NG-RAN selects PTM or PTP radio bearers to deliver the multicast PDUs to the UE(s) that have joined the multicast session.

18.	The NG-RAN transmits the multicast session data to the UE(s)  using the selected PTM or PTP radio bearer(s).

Steps 19 to 21 are for 5GC individual MBS traffic delivery:

19.	MB-UPF sends multicast PDUs in the N19mb tunnel associated to the multicast session to UPF. There is only one tunnel per multicast session and destination UPF, i.e., all associated PDU sessions share this tunnel.

20.	UPF forwards the multicast data towards the NG-RAN via unicast (i.e. in the N3 tunnel of the associated PDU Session).

21.	The NG-RAN forwards the multicast session data to the UE via unicast (i.e. over the radio bearer(s) corresponding to the mapped unicast QoS flow(s) of the associated PDU Session).

NOTE 7:	Details of the DL MBS data transmission are described in clause 6.7.

NOTE 8:	When the MBSF is involved in the multicast MBS session, the tunnel between MBSTF and MB-UPF has been established in the MBS session creation procedure.

****************** NEXT CHANGES ***************

#### 7.2.2.4	Release of shared delivery toward RAN node

In the following case, the shared delivery tunnel may be released between NG-RAN and MB-UPF:

-	The last UE is excluded from the context of the MBS session in the NG-RAN;

-	Handover to the target NG-RAN when the UE is the last UE for this MBS session in the source NG-RAN node during handover preparation phase known by the source NG-RAN node;

-	Handover to the target E-UTRAN when the UE is the last UE for this MBS session in the source NG-RAN;

-	MBS session deletion.

NOTE:	When the MBS session is deactivated, the shared delivery is not release if there is at least one UE is in connected mode for this MBS session.

Figure 7.2.2.4-1: Release of shared delivery toward RAN node

1.	A RAN node decides to release shared delivery for an MBS session, e.g. because it no longer serves at least one UE within the MBS session. For location dependent services, the NG-RAN node may release shared delivery for the location dependent contents of an MBS session if it no longer serves at least one UE assigned to an MBS session ID and area session ID.

2.	The NG-RAN node sends N2 MBS Session release request (MBS Session ID, [unicast GTP TEID], [Area Session ID]) to the AMF. For location dependent services, the NG-RAN node also provides the Area Session ID. The RAN node includes the unicast GTP TEID if unicast transport is used for the shared delivery.

3.	The AMF invokes Nmbsmf_MBSSession_ReceptionEnd request (MBS Session ID, N2 SM information) to the MB-SMF corresponding to the MB-SMF ID stored in the AMF for the MBS Session ID.

4.	If unicast transport was used towards the NG-RAN node, the MB-SMF sends N4mb Session Modification to the MB-UPF to release the N3mb tunnel used for the multicast session (or location dependent content of the multicast session if an area session ID was received) towards that RAN node using the received GTP tunnel endpoint information.

5.	The MB-SMF responses to the AMF. If there's no RAN node controlled by the AMF serving the multicast session, the MB-SMF also removes the AMF ID from the context of the multicast session.

6.	The AMF removes the RAN node ID from storage in the context of the multicast session (or location dependent part of the multicast session if an area session ID was received).

7.	The AMF sends an N2 MBS Session release response (MBS Session ID) to the RAN node. The NG-RAN node deletes the unicast GTP TEID (if unicast transport is used for the shared delivery) or sends IGMP/MLD leave message to leave the multicast transport if multicast transport is used for the shared delivery). The NG-RAN node releases local resources for  the multicast session.

****************** NEXT CHANGES ***************

7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The local multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The local multicast session is created as described in clause 7.2.4.2.2.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile(Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

-	SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID(s) and MBS service area(s) that the NG-RAN node belongs to. The SMF provides all MBS service areas information (Area session ID, MBS service area) to NG-RAN.

-	The RAN uses the received MBS Session ID(s) and Area Session ID(s) to determine the local multicast session context and whether the user plane for the local multicast session is already established.

****************** NEXT CHANGES ***************

7.2.4.2.2	Creation for local MBS

For local MBS, the MBS session creation procedure for the UE is optional and performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, session area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session. If the MBS Session ID is TMGI, the MB-SMF updating NF profile can be skipped.

-	The policy of Multicast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

****************** NEXT CHANGES ***************

7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been created where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted and 5GC individual MBS traffic delivery is used, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6].

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

****************** NEXT CHANGES ***************

7.3.2	MBS Session Release for Broadcast

The MBS Session Release for broadcast follows the MBS Session Deletion (e.g. TMGI De-allocation and MBS Session Deletion) so that resource for shared MBS delivery is released. It is possible for AF to stop MBS Session but keep TMGI allocated.

Figure 7.3.2-1: MBS Session Release for Broadcast

1.	The AF/AS may stop the media stream before sending the MBS Session Release Request (TMGI) message to the 3GPP network.

2.	The AF/AS performs MBS Session Stop procedure to request release of MBS Session (step 1 ~7 in the figure 7.1.1.4-1).

3.	MB-SMF sends Namf_MBSBroadcast_ContextRelease request (TMGI) to the AMF(s) that has been involved in the MBS Session.

4.	The AMF sends an MBS Session Resource Release Request (TMGI) message to all RAN nodes that have been involved. If a NG-RAN node receives multiple MBS Session Resource Release Request messages for the same TMGI (e.g. from several AMFs the NG-RAN is connected to), NG-RAN only performs step 5 and step 6 once.

5.	The NG-RAN stops the PTM transmission.

6.	If N3mb multicast transport has been used, the NG-RAN sends a Leave message (LL MC Address) to stop the media stream to this NG-RAN node. If N3mb point-to-point transport has been used, the NG-RAN release its N3mb Tunnel Info. NG-RAN deletes its MBS Session Context.

7.	The NG-RAN reports successful release of resources for the MBS Session by sending MBS Session Resource Release Response (TMGI) message(s) to the AMF(s).

8.	The AMF sends Namf_MBSBroadcast_ContextRelease response (TMGI) to the MB-SMF.

9.	The AF may start a TMGI de-allocation procedure (step 8~11 in the figure 7.1.1.4-1).

****************** NEXT CHANGES ***************

7.3.4	Support for Local Broadcast Service

The procedures for Local broadcast service contains the ones for local broadcast service with the location-dependent content as described in clause 6.2.

The MBS session creation procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same broadcast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the TMGI and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

-	The policy of broadcast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

The MBS session establishment procedure is performed as defined in clause 7.3.1 with the following additions:

-	MB-SMF requests the AMF to transfer an N2 message (i.e. MBS Session Resource Setup Request) to the NG-RAN nodes of the MBS service area with broadcast session information which additionally includes the Area Session ID and MBS service area.

-	The NG-RAN uses the received MBS Session ID and Area Session ID to determine the local broadcast session context.

****************** NEXT CHANGES ***************

9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS session creation. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (source specific multicast address or TMGI) or TMGI request.

Input, Optional: DNN, S-NSSAI, MBS service area, MBS activation time, MBS termination time, service description, QoS flow information, Input Transport Address Request, session activity status (active/inactive). For a multicast session, indication that any UE may join. For subscription to notifications event ID(s), notification target address.

Output, Required: Result Indication.

Output, Optional: TMGI, Cause, MB-UPF tunnel info.

****************** END OF CHANGES ***************