3GPP TSG SA WG2 Meeting SA2#148E 		S2-2108368r06

November 15 – 22, 2021, Electronic, Elbonia

Title:	[DRAFT] Reply LS on mandatory SSC modes supported by UE

Response to:	LS to SA2 on mandatory SSC modes supported by UE (C6-210258 / S2-2108269)

Release:	Rel-15 / Rel-16

Work Item:	5GS_Ph1_UEConTest

Source:	Ericsson [to be: SA2]

To:	CT6

Cc:	CT1

Contact person: 	Stefan Rommer

stefan.rommer@ericsson.com

Attachment: 	none

# 1	Overall description

SA2 would like to thank CT6 for the LS on mandatory SSC modes supported by UE.

The current SA2 5GS specifications (TS 23.501, TS 23.502 & TS 23.503) in releases up to rel.17 do not specify whether any of SSC modes (1, 2 or 3) are mandatory or optional for the UE to support and use, however a (1, 2 or 3) SSC mode shall be selected for a PDU session. In TS 23.228 clause Y.1.0 the following is stated:

“To support IMS, the UE shall establish a PDU session with PDU session type set to IP for the corresponding DNN (see TS 23.501 [93], clause 5.6.1), and acquire an IP address according to TS 23.501 [93], clause 5.8.2.2. If IMS Multimedia Telephony Service as specified in TS 22.173 [53] is to be used within the PDU session, SSC mode 1 shall be set for the PDU session. In this release of the specification, it is assumed that single PDU session with multiple PDU session anchors defined in TS 23.501 [93] clause 5.6.4 is not applicable for PDU sessions dedicated to IMS”.

From URSP rules perspective, the following is SA2's reply for Rel-15 and Rel-16:

Q1)	Is it correct, if we handle SSC modes 2 and 3 as optional?

SA2 reply: A UE may support one or more than one SSC modes (at least SSC mode 1). At PDU session establishment a SSC mode is selected by UE or Network. If UE does not provide a SSC mode, the network will select a SSC mode for the PDU session and if the UE does not support the SSC mode, the UE behaviour is unspecified and based on UE implementation the PDU session may be released by the UE. If the the UE provides a SSC mode in the PDU session establishment and the network cannot accept the SSC mode for this DNN and S-NSSAI, the network will reject the PDU session and indicating supported SSC modes. For a UE to use IMS Multimedia Telephony Service within the PDU session, the SSC mode 1 is mandatory. Not supporting a SSC mode provided in URSP means that the UE will not establish a PDU session to the indicated DNN/S-NSSAI. .

Q2)	Is it possible to modify 3GPP TS 23.501 to explicitly state which SSC modes have to be supported?

SA2 reply: . A UE may support one or more than one SSC modes (at least SSC mode 1). A UE to use IMS Multimedia Telephony Service within the PDU sessionshall support SSC mode 1

Q3)	If the support of SSC mode 2 and/or 3 is not optional but conditional, can you please specify the conditions?

SA2 reply: Depends on network and UE implementation

# 2	Actions

To CT6

ACTION: 	SA2 kindly requests CT6 to take the above information into account

# 3	Dates of next TSG SA WG2 meetings

SA2 WG Meeting #149 E		February 14 – 25, 2022		Elbonia

SA2 WG Meeting #150		April 4 – 8, 2022 		TBD