3GPP TSG-WG SA2 Meeting #148E 	S2-2108427r02

# Electronic Meeting, November 15–22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0674 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameter handling for an AF requesting a specific QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During SA2#147E, CRs handling clauses 4.15.6.6 and 4.15.6.6a of TS 23.502, specifying the procedures for setting up and updating the AF session with required QoS, triggered discussions about what QoS parameters (QoS Reference, individual QoS parameters, Alternative Service Requirements) and their combinations might be used by the AF in its request. The proposed changes for the clauses above are included in CR (S2-2108426) submitted for this meeting. Since the reason of the discussions are partly related to the different interpretation of clause 6.1.3.22 of TS 23.503, we submit this dependent CR in order to clarify the clause, which is required to avoid any potential misinterpretation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) In its request, the AF may provide either QoS Reference or Individual QoS parameters (i.e., exclusive options) 2) Alternative Service Requirements can be provided only when the AF request a specific QoS via a QoS Reference 3) Alternative Service Requirements contain one or more QoS Rreference parameters in a prioritized order indicating the preference of the QoS requirements with which the service can operate. (They do not include a subset of individual QoS parameters (in addition or separately) to QoS Reference parameters) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3131 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

#### 6.1.3.22	AF session with required QoS

The AF may request that a data session to a UE is set up with a specific QoS (e.g. low latency or jitter) and priority handling. The AF can request the network to provide QoS for the AF session based on the service requirements with the help of a QoS Reference  that refers to pre-defined QoS information. Instead of  QoS Reference, the AF may provide individual QoS parameters associated to the Flow Description.

a)	When the AF provides only a QoS Reference to determine the QoS parameters but no QoS parameters:

-	When the PCF authorizes the service information from the AF, it generates a PCC rule by deriving the QoS parameters of the PCC rule based on the service information and the indicated QoS Reference.

NOTE 1:	A SLA has to be in place between the operator and the ASP defining the possible QoS levels and their charging rates. For each of the possible pre-defined QoS information sets, the PCF needs to be configured with the corresponding QoS parameters and their values as well as the appropriate Charging key (or receive this information from the UDR).

-	The AF may change the QoS by providing a different QoS Reference while the AF session is ongoing. If this happens, the PCF shall update the related QoS parameter sets in the PCC rule accordingly.

b)	When the AF provides individual QoS parameters instead of QoS Reference:

-	The AF provides the following QoS parameters associated with the Flow Description. One of the following:

a)	In the case that the AF takes the role of TSN AF, which is a 5GC NF, TSC Assistance Container describing the traffic characteristics  and needed for TSCAI determination (as described in clause 5.27.2.4. of TS 23.501 [2]); or

b)	In the case that the AF is not the TSN AF: a subset of individual QoS parameters describing individual traffic characteristics and subsequently used for the determination of TSC Assistance Container as described in clause 5.27.2.3 of TS 23.501 [2].

-	The AF provides the QoS information, i.e. priority, maximum Burst Size, Requested 5GS delay and Maximum Flow Bit Rate.

The PCF performs Session binding using the DS-TT port MAC address or UE IP address.

The PCF generates a PCC Rule with service data flow filter (including IP Packet Filter set as in clause 5.7.6.2 of TS 23.501 [2] or Ethernet Packet Filter set as in clause 5.7.6.3 of TS 23.501 [2]) derived from the Flow Descriptions provided by the AF, the mapped 5QI, ARP, GBR and MBR, and the associated TSC Assistance Container as received from the TSN AF or TSCTSF.

The PCF derives the 5QI value as defined in clause 5.27.3 of TS 23.501 [2], the PCF derives the GBR using the Maximum Flow Bit Rate provided by the AF. The SMF binds the PCC Rule to a QoS Flow as defined in clause 6.1.3.2.4.

If the PCF gets informed about Policy Control Request Triggers relevant for the AF session, the PCF shall inform the AF about it as defined in clause 6.1.3.18.

If an AF session can adjust to different QoS parameter combinations, the AF may provide Alternative Service Requirements in addition to the QoS Reference. Alternative Service Requirements contain one or more QoS Reference parameters in a prioritized order, indicating the preference of the QoS requirements with which the service can operate. If so, the AF shall also subscribe to receive notifications from the PCF for successful resource allocation and when the QoS targets can no longer (or can again) be fulfilled as described in clause 6.1.3.18.

When the PCF authorizes the service information from the AF and generates a PCC rule, it shall also derive Alternative QoS parameter sets for this PCC rule based on the QoS Reference parameters in the Alternative Service Requirements.

The PCF shall enable QoS Notification Control and include the derived Alternative QoS parameter sets (in the same prioritized order indicated by the AF) in the PCC rule sent to the SMF. When the PCF notifies the AF that QoS targets can no longer be fulfilled, the PCF shall include the QoS Reference parameter corresponding to the Alternative QoS parameter set referenced by the SMF or an indication that the lowest priority QoS Reference of the Alternative Service Requirements cannot be fulfilled (as described in clause 6.1.3.18).

NOTE 2:	The AF behaviour is out of the scope of this TS but can include adaptation to the change of QoS (e.g. rate adaptation) as well as application layer signalling with the UE.

The AF may change the Alternative Service Requirements while the AF session is ongoing. If this happens, the PCF shall update the Alternative QoS parameter sets in the PCC rule accordingly.

The AF may indicate to the PCF that the UE does not need to be informed about changes related to Alternative QoS Profiles. With this indication received from the AF, the PCF decides whether to disable the notifications to the UE when changes related to the Alternative QoS Profiles occur and sets the Disable UE notifications at changes related to Alternative QoS Profiles parameter in the PCC rule accordingly.

For TSC, the AF may additionally provide a subset of individual QoS parameters describing individual traffic characteristics and subsequently used for the determination of TSC Assistance Container as described in clause 5.27.2.3 of TS 23.501 [2].

>>>> End of Changes <<<<