3GPP TSG-WG SA2 Meeting #148E e-meeting	S2-2108436r04

# Elbonia, November 15 – 22, 2021												(revision of S2-2107321)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0448 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to resolve conflicts in ML model transferring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. In this release of the specification, we make consensus on the principle that AnLF only retrieves the ML model information from the set of configured MTLFs in the AnLF, however, Nnwdaf_AnalyticsSubscription_Transfer and Nnwdaf_AnalyticsInfo_ContextTransfer service are in conflict with that principle. Specifically, Nnwdaf_AnalyticsSubscription_Transfer allows source NWDAF to transfer ML model information (i.e., file address of ML model) to target NWDAF without any confirmation of the set of configured MTLFs in target NWDAF, and then, target NWDAF directly accesses the ML model while exploiting the transferred ML model information, even if the ML model comes from unconfigured MTLF in the target NWDAF. This breaks the principle. 2. As proposed in CR0414, "File address of the ML model(s) that the NWDAF is currently using for the analytics" is removed from Analytics Context Information, to avoid duplication with model information in Nnwdaf_AnalyticsSubscription_Transfer. However, when NWDAF service consumer performs NWDAF reselection and requests for analytics subscription to the target NWDAF via Nnwdaf_AnalyticsSubscription_Subscribe, target NWDAF cannot get the "File address of the ML model(s)” if needed unless using analytics context information transfer procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that target NWDAF subscribes to ML model(s) from the locally configured NWDAF(s) containing MTLF based on the model information or the MTLF IDs provided from source NWDAF. Add back "File address of the ML model(s) that the NWDAF is currently using for the analytics" to the content of Analytics Context Information, with the clarification that the file address of ML Model is provided in case the source NWDAF contains MTLF and the ML Model is produced by itself. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AnLF can use ML model(s) from the unconfigured MTLFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.2.2, 6.1B.3, 6.1B.4, 7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of the first change * * * *

6.1B.2.2	Analytics Subscription Transfer initiated by source NWDAF

The procedure in Figure 6.1B.2.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

If the source NWDAF discovers that the analytics consumer may change concurrently to this procedure, the source NWDAF should not perform the procedure. In such a case, the source NWDAF may send a message to indicate to the analytics consumer that it will not serve this subscription anymore.

NOTE 1:	To discover if the analytics consumer may change, the source NWDAF can, for example, check if the serving area of target NWDAF is within the serving area of the analytics consumer, if the serving area information is available.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.2.2-1 is applicable for analytics subscription transfer across NF Sets or if the NWDAF is not deployed in a Set.

Figure 6.1B.2.2-1: Analytics subscription transfer initiated by source NWDAF

0.	Source analytics consumer subscribes to Analytics from source NWDAF. The Analytics Consumer may send its NF ID or serving area, enabling NWDAF to determine whether this procedure is applicable. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in clause6.1B.2.3.

2.	Source NWDAF determines, e.g. based on the UE location information received and the NF consumer's serving area either directly received in step 0 or indirectly received via NRF, to perform an analytics subscription transfer to target NWDAF(s). Therefore, the source NWDAF determines the analytics subscription(s) to be transferred to a target NWDAF.

3.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics (see clause 6.1.3) to preferably select a target NWDAF that is already serving the consumer.

4.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF. The request contains a callback URI of the analytics consumer. The request also contains active data source ID(s), ML Model related information, which are related to the analytics subscription(s) requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 1). The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

If the analytics subscription being transferred requires analytics aggregation in the Source NWDAF (i.e. the Source NWDAF as an Aggregator NWDAF produces analytics information for the analytics subscription being transferred by aggregating analytics outputs from other NWDAFs and possibly itself as specified in clause 6.1A), the request message may also include the analytics subscription aggregation information, i.e. information about the analytics subscriptions that the source NWDAF has with these NWDAFs that collectively serve the transferred analytics subscription for specific analytics ID(s), which includes IDs of these NWDAFs for specific analytics ID(s) and optionally input parameters of analytics exposure as defined in clause 6.1.3.

5.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation based on the information received from the source NWDAF.

Target NWDAF may use the ML Model related information, if provided in the Nnwdaf_AnalyticsSubscription_Transfer request, to retrieve the ML Model(s) and use it for the transferred analytics subscription(s). If ML model provider NWDAF ID(s) is provided in the Nnwdaf_AnalyticsSubscription_Transfer request, target NWDAF may subscribe to the ML Model(s) from the indicated ML Model provider NWDAF(s) as specified in clause 6.2A, if the ID(s) of  NWDAF(s) containing MTLF is configured to the list of (set of) locally configured NWDAF ID(s) containing MTLF (see clause 6.2A.0) of the target NWDAF, and use the ML model(s) for the transferred analytics subscription. If ML Model file address(es) is provided in addition to the ID(s) of NWDAF(s) containing MTLF, target NWDAF may retrieve the ML Model from the ML Model file address only when the ID(s) of NWDAF containing MTLF is configured to the list of (set of) locally configured NWDAF ID(s) containing MTLF of the target NWDAF. In case the target NWDAF retrieves ML Model from the ML Model file address(es), the target NWDAF neither indicates ML Model Filter Information and Target of ML Model for the analytics nor checks the validity condition of ML Model(s).

NOTE 4:	In case the target NWDAF retrieves ML Model from the ML Model file address(es), it is needed to align with SA3 about the potential security issues on ML Model stealing by unconfigured target NWDAF.

Otherwise when ML Model related information is not provided, target NWDAF may use the ML model trained by itself (if the target NWDAF contains MTLF), or may discover NWDAF containing MTLF tha can provide a traind ML Model(s) for the transferred analytics and subscribe to ML model(s) from the NWDAF containing MTLF with ID being among the list of locally configured NWDAF ID(s) containing MTLF.

If the analytics subscription being transferred requires analytics aggregation and the Nnwdaf_AnalyticsSubscription_Transfer request contains the analytics subscription aggregation information as described in step 3a, the target NWDAF, if supporting analytics aggregation, may use the analytics subscription aggregation information as indicated in the request to acquire analytics outputs and perform analytics aggregation for the transferred analytics subscription.

NOTE 5:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in TS 29.518 [25] clause 6.2.6.2.2 and UEContextTransfer as specified in TS 29.518 [25] clause 5.2.2.2.1.1.

NOTE 6:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

6.	Target NWDAF informs the analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message and providing the new subscription Id, that had been assigned by the target NWDAF, in the Subscription Change Notification Correlation Id parameter of this message as specified in clause 7.2.4.

NOTE 7:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

NOTE 8:	If this subscription is used as input for analytics aggregation by the analytics consumer, the analytics consumer may inform the other NWDAFs instance participating in this analytics aggregation that the Set of NWDAF identifiers of NWDAF instances used by the NWDAF service consumer for this analytics aggregating (see clause 6.1.3) has changed using the Nnwdaf_AnalyticsSubscription_Subscribe service operation.

7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the target NWDAF requests the "analytics context". The analytics context information transfer procedure is specified in clause 6.1B.3.

8.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

9.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

10.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions. In addition, Source NWDAF unsubscribes with the ML model provider NWDAF(s), if exist, which are no longer needed for the remaining analytics subscriptions.

NOTE 9:	At this point, the analytics subscription transfer is deemed completed, i.e. the source NWDAF can delete all information related to the successfully transferred analytics subscription.

11-12.	Target NWDAF at some point derives new output analytics based on new input data and notifies the analytics consumer about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

NOTE 10:	Notification correlation information allows the analytics consumer to correlate the notifications to the initial subscription request made with the source NWDAF in step 0.

# * * * * Start of the second change * * * *

6.1B.3	Analytics Context Information Transfer

The procedure depicted in Figure 6.1B.3-1 is used by an NWDAF instance to request analytics context information from another NWDAF instance, using the Nnwdaf_AnalyticsInfo_ContextTransfer service operation as defined in clause 7.3.3. This procedure, for example, can be invoked in the procedures described in clause 6.1B.2 to request the transfer of relevant analytics context information.

Figure 6.1B.3-1: Analytics Context Information Transfer

The procedure of analytics context information transfer comprises the following steps:

1.	The consumer NWDAF requests analytics context information by invoking Nnwdaf_AnalyticsInfo_ContextTransfer request service operation. The parameters that can be provided in the request are listed in clause 6.1B.4.

2.	The provider NWDAF responds with analytics context information to the consumer NWDAF. The analytics context information that can be provided in the response is listed in clause 6.1B.4.

If the provider NWDAF stores analytics context information (e.g. historical analytics, historical data) in ADRF, the provider NWDAF may include in the response the ADRF ID together with an indication of the type of data stored in the ADRF.

Upon receiving the analytics context information, the consumer NWDAF may:

-	provide the pending output analytics or historical analytics information to the analytics consumer per the subscription/request;

-	use the historical data and analytics metadata in the analytics context information to generate analytics;

-	subscribe to data collected for analytics with the data sources indicated in the analytics context information;

-	retrieve the ML model(s) from the ML model file address(es) indicated in the analytics context information and use the ML model(s) for analytics only when ML Model file address(es) is provided in addition to the ID(s) of NWDAF(s) containing MTLF and the ID(s) of NWDAF(s) containing MTLF being among the locally configured list of NWDAF ID(s) containing MTLF (see clause 6.2A.0);

NOTE:	It is needed to align with SA3 about the potential security issues on ML Model stealing by unconfigured target NWDAF when ML Model file address(es) is given.

-	subscribe to trained ML model(s) from the indicated model provider NWDAF(s) and use for analytics for analytics for those indicated model provider NWDAF with ID(s) being among the locally configured list of NWDAF ID(s) containing MTLF (see clause 6.2A.0);

-	use an ML Model trained by the consumer NWDAF itself (if the NWDAF contains MTLF) for the analytics generation;

-	discover NWDAF containing MTLF that can provide a trained ML Model(s) for the transferred analytics and subscribe to ML Model(s) from the NWDAF containing MTLF with ID(s) being among the locally configured list of NWDAF ID(s) containing MTLF (see clause 6.2A.0);

and/or

-	subscribe to output analytics from the indicated NWDAFs that collectively serve the transferred analytics subscription and perform analytics aggregation on the output analytics using the analytics metadata information, based on the analytics subscription aggregation information.

# * * * * Start of the third change * * * *

6.1B.4	Contents of Analytics Context Information

The consumers of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3) provide the following input parameters:

-	A list of analytics context identifier(s): identify a set of analytics context information that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance. The analytics context identifier is provided as the following:

-	Subscription Correlation ID: identifies the analytics subscription for which the related analytics context information is requested; or

-	A set of SUPI and associated Analytics ID for UE related Analytics; or

-	An Analytics ID for NF related Analytics.

-	[OPTIONAL] Requested Analytics Context Information per analytics context identifier: indicates which part of the analytics context information the consumer wishes to receive. Following values are specified:

-	Pending output Analytics;

-	Historical output Analytics;

-	Analytics subscription aggregation information;

-	Data related to Analytics;

-	Aggregation related information;

-	ML Model related information.

NOTE 1:	A list of "analytics context identifier(s)" can be provided by the source NWDAF to the target NWDAF in an analytics subscriptions transfer request as described in clause 6.1B.2; information allowing to identify an analytics context can also be provided by the NWDAF consumer to the target NWDAF in the Nnwdaf_AnalyticsSubscription_Subscribe request as defined in clause 7.2.2.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3), the output information listed below:

-	(Set of) Analytics context information matching the input parameters of the Nnwdaf_AnalyticsInfo_ContextTransfer request. If no Requested Analytics Context Information parameters are available in the request, all available analytics context information is sent. Analytics context information includes the following information parts, if available:

-	Analytics related:

-	Pending output analytics (i.e. not yet notified to the consumer);

Historical output analytics information. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

-	Timestamp(s) of the last output analytics provided to the analytics consumer(s). Value is set to 0 if no output analytics had been sent yet.

-	Analytics subscription aggregation information (only provided when analytics context is related to analytics aggregation): information about the analytics subscriptions that the source NWDAF has with the NWDAFs that collectively serve the transferred analytics subscription, which includes IDs and analytics metadata information of these NWDAFs for specific analytics ID(s), and optionally input parameters of analytics exposure as defined in clause 6.1.3.

-	Data related to Analytics:

-	Historical data that is available at the source NWDAF and that is related to the analytics to be handed over to the target NWDAF. If available, the time period of the collected data, NF ID(s) of the data source(s) and information about subscriptions with the data sources that are related to the analytics is included.

-	Aggregation related information: Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	(Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytic subscriptions.

-	ML Model related information:

-	Model provider NWDAF ID: Instance ID of the ML model provider NWDAF from which the NWDAF currently subscribes to the ML model information used for the analytics.

-	[OPTIONAL] File address(es) of the ML Model that the NWDAF is currently using for the analytics only if the ML Model used by the analytics subscription being transferred and the ML Model file is produced by the NWDAF itself.

NOTE 2:	If file address(es) of ML Model is included Model provider NWDAF ID is the NWDAF ID itself.

NOTE 3:	It is needed to align with SA3 about the potential security issues on ML Model stealing by unconfigured target NWDAF when ML Model file address(es) is given.

# * * * * Start of the third change * * * *

7.2.5	Nnwdaf_AnalyticsSubscription_Transfer service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Transfer.

Description: Requests to transfer analytics subscription(s) from the consumer NWDAF to the service provider NWDAF.

Inputs, Required:

-	Transfer type: indicates the type of the transfer request. The following values are supported:

-	Analytics subscription transfer preparation: requests the NWDAF to prepare for taking over the analytics subscription(s) and/or prepare for collecting Analytics Context(s).

-	Analytics subscription transfer: requests the NWDAF to take over the analytics subscription(s).

-	Analytics subscription transfer cancel: cancels a prepared analytics subscription request.

Inputs, Optional:

-	If this service operation is for "analytics subscription transfer preparation", the following parameter shall be provided:

-	(Set of) analytics subscription information with the following parameters:

-	All input parameters for the analytics exposure as specified in clause 6.1.3.

-	[OPTIONAL] Active data source ID(s): Instance Id or Set Id of the active data source(s) the consumer NWDAF is currently using for the analytics of this analytics subscription.

-	[OPTIONAL] ML Model related information that the NWDAF is currently using for the analytics:

-	[OPTIONAL] File address(es) of the ML model(s).

-	[OPTIONAL] Model provider NWDAF ID(s): Instance ID(s) of the ML model provider NWDAF(s) from which the consumer NWDAF currently subscribes to the ML model information used for the analytics.

NOTE 1:	Target NWDAF containing AnLF is only allowed to use the locally configured (set of) NWDAF(s) containing MTLF (see clause 6.2A.0).

-	[OPTIONAL] File address(es) of the ML Model(s) only if the ML Model used by the analytics subscription being transferred and the ML Model file is produced by the NWDAF itself.

NOTE 2:	If file address(es) of ML Model is included Model provider NWDAF ID is the NWDAF ID itself.

NOTE 3:	It is needed to align with SA3 about the potential security issues on ML Model stealing by unconfigured target NWDAF when ML Model file address(es) is given.

-	Subscription callback URI: indicates the callback URI of the analytics consumer of this subscription, i.e. the URI the analytics notifications are to be sent to.

-	Id of analytics consumer that is subscribed to receive analytics.

-	(Set of) analytics context identifier(s): identifies analytics context information available at the consumer NWDAF as defined in clause 6.1B.4.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is not performed, the same parameters as those for transfer type "Analytics subscription transfer preparation" shall be provided.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is performed, the same parameters as those for transfer type "Analytics subscription transfer preparation" can be provided with updated parameter values.

-	If this service operation is to request analytics subscriptions transfer cancel, the following parameter shall be provided:

-	Subscription Correlation Id.

Outputs Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *