3GPP TSG-WG SA2 Meeting #148E e-meeting   	S2-2108571r02

Elbonia, November 15 – 22, 2021	(revision of S2-2107415r06)

Title:	[DRAFT] Reply LS on network slice admission control for SNPN onboarding

Response to:	LS (S2-2108265/C1-215159) on network slice admission control for SNPN onboarding from CT1

Release:	Release 17

Work Item:	eNPN

Source:	[Huawei to be] SA2

To:	CT1

Cc:

Contact Person:

Name:	zhuqianghua

Tel. Number:

E-mail Address:	zhuqianghua AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	501CR# 3241

1. Overall Description:

SA2 thanks CT1 for the LS (C1-215159) on network slice admission control for SNPN onboarding.

CT1 Question: When the UE is registering or registered for onboarding services in SNPN or PLMN, whether the network slice admission control needs to be performed for slice(s) used for onboarding or not?

SA2 Answer:

SA2 assumes that in case UE is registering for onboarding service in PLMN, network slice admission control is applied for slice(s) used for onboarding depending on operator policies. In case of registration for onboarding services in SNPN, the UE does not provide the S-NSSAI in the Registration request to the onboarding SNPN. In SA2 understanding the UE is aware of the S-NSSAI that is rejected due to network slice admission control as it is signalled to the UE using the cause “S-NSSAI not available due to maximum number of UEs reached” in Extended rejected NSSAI.

SA2 Question:

When the UE does not provide any Requested NSSAI (as in case of UE onboarding within a SNPN) can the UE associate the “S-NSSAI not available due to maximum number of UEs reached” with the Onboarding Registration attempt in order to avoid subsequent Onboarding Registration attempts?

If the answer to above question is that the UE is not able to associate the “S-NSSAI not available due to maximum number of UEs reached” with the Onboarding Registration attempt, then SA2 agrees that network slice admission control cannot be used on network slices used for onboarding.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to consider the above feedbacks and answer the above question.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #149E   	February 14 – 25, 2022 			Elbonia

TSG-SA2 Meeting #150   	April 4 – 8, 2022 				TBD