SA WG2 Meeting #S2-148E	S2-2108575r02

15 - 22 November, 2021, Electronic meeting

Source:	Saankhya Labs, Ligado Networks, ONE Media 3.0, Fraunhofer IIS, CEWiT, Tejas Networks, IIT Bombay, IIT Kanpur, IIT Madras, IIT Hyderabad, IIT Kharagpur

Title:	Study on Interworking of Non-3GPP Multicast and Broadcast Networks with 3GPP 5G System

Document for:	Approval

Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Interworking of Non-3GPP Multicast and Broadcast Networks with 3GPP 5G System

Acronym:

Unique identifier:

Potential target Release:	Rel-18

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

Dependency on non-3GPP (draft) specification:

# 3	Justification

The proliferation of mobile broadband networks is increasingly making the devices like smartphones as the preferred choice for content consumption. However, with the growth in content consumption, especially video consumption, via cellular networks, the load on the available cellular spectrum has also increased, leading to congestion in cellular networks. The increased usage of these video/content consumption along with a host of new services such as software delivery over wireless networks, group communications and broadcast/multicast IoT applications have created a need for their delivery via multicast/broadcast services in 5G system [1].

Satellite access networks, which include both satellite NG-RAN and non-3GPP satellite access network [1], may be used to support delivery of multicast/broadcast services in 5GS. Satellite NG-RAN is already being considered for multicast/broadcast service delivery in 5GS. In this study item we propose to use non-3GPP satellite access network (non-3GPP NTN) for multicast/broadcast delivery in 5GS.

Some of the advantages of using non-3GPP NTN for Multicast/Broadcast delivery in 5GS are as follows:

- In 3GPP, the usage of NTN is already being considered as a means of providing coverage enhancement for terrestrial networks. They are being proposed to be used both for access as well as backhaul connectivity.

- Services such as Firmware-Over-The-Air (FOTA) might be delivered very efficiently via satellite broadcast due to the pervasive coverage provided by them. The satellite-based delivery may be cost-effective too.  Another service that may be considered part of critical infrastructure is satellite delivery of augmentation services for high precision GNSS; this is already part of the planning for automated driving in the future. Multimode, satellite-terrestrial delivery of such services is likely to optimize both the cost and the geographic coverage of such services. Apparently, there is no plan in 3GPP as yet to issue standards for satellite multicast/broadcast over NBIoT and eMTC air interfaces until Release 3GPP 18. Until such services are launched, FOTA & GNSS augmentation (mentioned as examples) could be carried over a multimode 3GPP-cellular/non-3GPP-satellite broadcast network, at least as a gap filler. Therefore, it would be helpful for 3GPP to enable the proposed work.

- Another example that can benefit from broadcasting/multicasting via Satellite access is content delivery from a content caching application, as has been highlighted in TS 22.261 [1].

- Non-3GPP satellite multicast/broadcast systems (both open-standard and proprietary) have existed for some time, and it may not be necessary to wait for a 3GPP satellite broadcast system to be standardized in NTN before other 5G 3GPP systems can roam to them. 3GPP can play an enabling role in the process (of standardization) by standardizing the roaming interfaces while remaining agnostic towards the air interface.

In addition to the non-3GPP NTN, usage of other broadcast technologies such as non-3GPP Digital Terrestrial Broadcasting Technology networks (non-3GPP DTT) may also help in reducing the load on the cellular spectrum/network if a portion of the content delivery (video data) is offloaded onto them. A few of the advantages of using Non-3GPP Digital Terrestrial Broadcasting Technologies are as follows:

- They also provide support for large coverage areas, which might be helpful for service delivery in rural areas and sparsely populated areas.

- Most Non-3GPP DTTs provide high spectral efficiency supporting higher-order modulation and coding schemes, e.g., Advanced Television Systems Committee 3.0 (ATSC3) standard supports 1024-QAM.

- Improved mobility management due to support for large coverage areas and SFNs

Therefore, the convergence or integration of 5GS and Non-3GPP Digital Terrestrial Broadcast Network /Non-3GPP Satellite Broadcasting Technologies would

- Reduce load on Cellular Network through efficient offloading

- Enable creation of ‘best connected’ hybrid broadcast network. Higher throughput payload could be broadcast over terrestrial networks, and lower throughput, but critical payloads broadcast over the satellite network. The Converged Core architecture of the 5G system can be used for simpler dynamic interworking with non-3GPP NTN/DTT networks just as it has been achieved with Wi-Fi and Wireline access networks (via functions like N3IWF and W-AGF). Extending the interworking to non-3GPP NTN/DTT is likely to unleash benefits through newer services which have not been realized so far, similar to the converged services such as ATSSS or multiple access support achieved with Wi-Fi and Wireline access in the unicast domain.

- Enhance the device ecosystems of both cellular and broadcast networks by leveraging the scales of cellular devices

- Improve resource utilization across non-3GPP NTN/DTT and 3GPP 5G NG-RAN

- Open up new business opportunities through the interfacing of 5GS with the legacy and new broadcasting networks. For example, ATSC3 is deployed in the USA, Korea & DVB-T2 is deployed in 144 countries worldwide. There can be improved monetization of these networks through convergence with 5GS.

- The 5G system architecture has already been defined with a converged core network integrating non-3GPP access networks such as WLANs and wireline access for unicast communication. Therefore, it may be appropriate to extend this capability to wide-area multicast/broadcast communication, including satellite networks, and integrate the non-3GPP multicast and broadcast networks within the 5G system.

The 5GC Integration with non-3GPP DTT/NTN for Broadcast will help in providing improved services via the following representative scenarios.

- Efficient offload of several unicast sessions in 3GPP NG-RAN to Broadcast session in non-3GPP DTT/NTN network based on the “best connected” hybrid network.

- Enables Broadcast Capacity on-demand update as per the traffic needs and Flexible capacity allocation by making use of non-3GPP DTT/NTN network.

- Enables on-demand Coverage improvement for Broadcast mode delivery of OTT content/TV services by making use of non-3GPP DTT/NTN network

- They are ensuring Coverage (and service continuity) of Broadcast Channels when a user moves outside the coverage area of the 3GPP NR/MBS network.

In all the above scenarios, seamless management of the sessions between 3GPP and non-3GPP networks can be done with a 5GC Integration.

The following figure provides a high-level view of the interworking architecture as envisioned by the proponents. It proposes to utilize the concept of converged core in the 5G system. The non-3GPP NTN and the non-3GPP DTT networks can use a new interworking function (non-3GPP Broadcast IWF) along with the N2 and N3mb interfaces to interwork with the 5G core. The N2 and the N3mb interfaces used here are expected to be similar to the one used between NG-RAN and the 5G Core for multicast/broadcast service delivery, as far as possible. A UE has the capability to receive content via NG-RAN as well as non-3GPP NTN/DTT networks.

It should be noted that it is only an early view of the proposed architecture, and the study phase is expected to come up with the final interworking architecture.

- 3GPP TS 22.261, " Service requirements for the 5G system; Stage 1 ", Release 18

# 4	Objective

This study item aims to investigate the interworking of non-3GPP satellite networks and non-3GPP Digital Terrestrial Broadcasting Technologies with 5G system and would undertake the following (work) tasks:

- WT_#1 would study how to support Multicast/Broadcast (MB) Session via non-3GPP NTN/DTT networks within the framework of 5GS and how to support a unified MB session management across 3GPP NG-RAN and non-3GPP NTN/DTT networks.

  - It would study how a multicast broadcast session in a 5GS will utilize the non-3GPP NTN/DTT networks for delivery of content to UEs, i.e., how to make the 5GS interwork with non-3GPP NTN/DTT technologies for the MBS data delivery. As shown in the figure above, apparently the 5G (Converged) Core can support non-3GPP NTN/DTT Network with the help of Interworking Functions(s) and WT_#1 would study if there is a need to define Non-3GPP Broadcast Interworking Function (s) for interworking between the 5GC and non-3GPP NTN/DTT networks, similar to those defined for other non-3GPP access technologies like WLAN.

  - It will also identify the changes that may be required in the existing N2 and N3mb interfaces for integrating the potential Non-3GPP Broadcast Interworking Function (s) with 5GC. For example, the changes may be required in case of interworking with non-3GPP DTT network as there may not be any uplink channel via the non-3GPP DTT network unlike other access technologies like 3GPP NG-RAN. It will also study if there is any impact on functions in the 5GC in order to integrate non-3GPP NTN/DTT network within 5GS.

  - WT_#1 will also identify the mechanism to be used to switch the delivery of an MB session from NG-RAN (i.e., shared and/or individual MBS traffic delivery via NG-RAN) to non-3GPP NTN/DTT network (MBS traffic delivery) and vice versa, focusing on the signalling exchange between various elements in the network, e.g., between 5GC (AMF) and non-3GPP NTN/DTT interworking functions or between the UE & 5GC (NAS interface) and UE & NG-RAN (AS interface).

  - To support the multicast/broadcast data delivery through non-3GPP NTN/DTT networks, the 5GS will need to know if non-3GPP NTN/DTT network is accessible to an individual UE and if data to that UE can be served via non-3GPP NTN/DTT network. The accessibility of the network may also change dynamically due to UE mobility. WT_#1 would answer the following questions in this regard

    - Does the UE report the non-3GPP NTN/DTT networks available in its vicinity to 5GC? or

    - Does the UE send the regular measurement reports for the non-3GPP NTN/DTT to NG RAN and NG RAN provide the relevant information to 5GC? or

    - Can it be determined based on UE’s location by 5GC? etc.

  - It will also study the impact on authentication, authorization and security related requirements in order to deliver content via non-3GPP NTN/DTT networks to UEs.

- Evaluation of the proposed changes and mechanisms to integrate non-3GPP NTN/DTT networks with 5GS

- Architectural modifications in the 5G System

  - High-Level Architecture of the Non-3GPP Broadcast Interworking Function(s)

  - Modifications required in the Core Network and the CN-AN interface (N2, N3mb)

  - Modifications required in the 3GPP NG RAN, e.g., because the non-3GPP DTT network may not allow any uplink channel for UEs unlike 3GPP NG-RAN or other non-3GPP access technologies.

  - Modifications required in UE-5GS signalling exchange (NAS & AS protocol), if any

  - Modifications required in the UICC APPs and the ME. For example, there may be following modifications in the UICC APP (mentioned for the purpose of illustration only)

    - There may be additional entries for the non-3GPP NTN/DTT based multicast/broadcast service in the elementary file, "USIM Service Table".

    - There may be additional authentication and security requirements to receive data over non-3GPP broadcast networks requiring new elementary files.

    - There may be information for selection of available non-3GPP NTN/DTT broadcast networks to receive multicast/broadcast data requiring additional elementary files.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 | 4 |  | May be | WT#1 is self-contained |
| WT#2 | 2 |  | May be | WT#2 is dependent on WT#1 |
| WT#3 | 2 |  | May be | WT#3 is dependent on WT#1 and WT#2 |
| WT#3.1 | 0.5 |  |  |  |
| WT#3.2 | 0.5 |  |  |  |
| WT#3.3 | 0.5 |  |  |  |
| WT#3.4 | 0.5 |  |  |  |

Total TU estimates for the study phase: 8

Total TU estimates for the normative phase: 0

Total TU estimates: N + M = 8

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur | Remarks |
| External TR | 23.xyz | Study on interworking of non-3GPP multicast and broadcast networks with the 3GPP 5GS | TBD | TBD | Saha, Anindya, Saankhya Labs, anindya@saankhyalabs.com Jha, Pranav, IIT Bombay, pranavjha@ee.iitb.ac.in | Tasks for the Secondary Rapporteur is TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| NA |  |  |  |

# 6	Work item Rapporteur(s)

Saha, Anindya, Saankhya Labs, anindya@saankhyalabs.com

Jha, Pranav, IIT Bombay, pranavjha@ee.iitb.ac.in: Tasks for the secondary Rapporteur is <TBD>

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None Identified Yet

# 9	Supporting Individual Members

Supporting IM name

Saankhya Labs

Ligado Networks

ONE Media 3.0

Fraunhofer IIS

CEWiT

Tejas Networks

IIT Bombay

IIT Kanpur

IIT Madras

IIT Hyderabad

IIT Kharagpur