3GPP TSG-SA2 Meeting #148-e	S2-2108601

Nov 15 - 22, 2021, Electronic

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0060 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to NF services for MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Nmbsmf_MBSSession_ContextUpdate service operation definition, parameters are to be aligned for the consumer (AMF or SMF). - The parameter "MBS Service Type (multicast or broadcast)" needs to be added to Nmbsmf_MBSSession_Create service operation, for the MB-SMF to decide e.g. whether to trigger broadcast session towards the AMF. - Other corrections/clarifications are needed, e.g. adding "unsubscribe to" for Nmbsmf_MBSSession_StatusUnsubscribe description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align parameters in Nmbsmf_MBSSession_ContextUpdate service operation definition for different consumers (AMF or SMF). - Add parameter "MBS Service Type (multicast or broadcast)" to Nmbsmf_MBSSession_Create service operation. - Other corrections/clarifications, including: change "notification target address" to "Notification Target Address", add "unsubscribe to" for Nmbsmf_MBSSession_StatusUnsubscribe description, etc. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/incomplete NF service definitions and usage for supporting MBS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1, 9.1.3.2, 9.1.3.3, 9.1.3.4, 9.1.3.5, 9.1.3.6, 9.1.3.8, 9.1.3.10, 9.1.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

9.1.2	Nmbsmf_TMGI service

9.1.2.1	General

Service description: NF Service Consumer can use this service to request the allocation of TMGIs and release allocated TMGIs.

# * * * * Next change * * * *

9.1.3.2	Nmbsmf_MBSSession_ContextUpdate service operation

Service operation name: Nmbsmf_MBSSession_ContextUpdate

Description: NF Service Consumer can use this service to request or terminate the reception of data of a multicast session.

Inputs, Required: MBS Session ID, if consumer is AMF: N2 container (Establishment or Release, MBS session ID, Possible Area session ID, Possible GTP Tunnel info for unicast transport), AMF ID, if consumer is SMF: SMF ID, MBS session ID, Action(Establishment or Release).

Inputs, Optional: if consumer is SMF: Area Session ID, Unicast GTP Tunnel info of the UPF.

Outputs, Required: Success or not.

Outputs, Optional: if consumer is AMF: N2 container (MBS session ID, Possible Multicast DL tunnel info); if consumer is SMF: Multicast DL tunnel info.

9.1.3.3	Nmbsmf_MBSSession_ContextStatusSubscribe service operation

Service operation name: Nmbsmf_MBSSession_ContextStatusSubscribe

Description: Service Consumer NF can use this service operation to request information (e.g. QoS information) about a multicast session and to subscribe to notification of events about the multicast session context.

Inputs, Required: MBS Session ID, Notification Target Address, Events ID(s).

Inputs, Optional: immediate reporting indication.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: If immediate reporting indication is set: Event information (e.g. QoS information for multicast session, multicast session status (activated, deactivated), multicast session service area for local multicast service), Start time of multicast session, if consumer is SMF: indication that the MBS session allows any UE to join.

9.1.3.4	Nmbsmf_MBSSession_ContextStatusNotify service operation

Service operation name: Nmbsmf_ MBSSession_ContextStatusNotify

Description: This service operation, which is applicable to multicast MBS session, is used by the MB-SMF to notify its consumers about events of an MBS Session.

Inputs, Required: MBS Session ID, Event ID

Inputs, Optional: Event information (e.g. QoS information of MBS Session, MB service area).

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause.

9.1.3.5	Nmbsmf_MBSSession _ ContextStatusUnsubscribe service operation

Service operation name: Nmbsmf_MBSSession_ContextStatusUnsubscribe

Description: This service operation, which is applicable to multicast MBS session, is used by the consumer to unsubscribe to notifications about MBS context events.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS session configuration. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (source specific multicast address or TMGI) or TMGI request, MBS Service Type (multicast or broadcast).

Input, Optional: DNN, S-NSSAI, MBS service area, MBS activation time, MBS termination time, service description, , QoS flow information, Input Transport Address Request, session activity status (active/inactive). For a multicast session, indication that any UE may join. For subscription to notifications event ID(s), Notification Target Address.

Output, Required: Result Indication.

Output, Optional: TMGI, Cause, MB-UPF tunnel info.

# * * * * Next change * * * *

9.1.3.8	Nmbsmf_MBSSession_ Delete service operation

Service operation name: Nmbsmf_MBSSession_Delete

Description: Release the multicast session or broadcast session. The MBS session is deleted and the subscription (if any) to notifications about events related to the status of the MBS session is terminated.

Input, Required: MBS Session ID.

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

# * * * * Next change * * * *

9.1.3.10	Nmbsmf_MBSSession_StatusSubscribe service operation

Service operation name: Nmbsmf_MBSSession_StatusSubscribe

Description: This service operation is used by the NF service consumer to subscribe notification about events related to the status of the MBS session.

Input, Required: MBS Session ID, event ID(s), Notification Target Address.

Output, Required: When the subscription is accepted: Subscription Correlation ID.

9.1.3.11	Nmbsmf_MBSSession_StatusUnsubscribe service operation

Service operation name: Nmbsmf_MBSSession_StatusUnsubscribe

Description: This service operation, which is applicable for broadcast MBS session, is used by the NF service consumer to unsubscribe to notification about events related to the status of the MBS session.

Input, Required: Subscription Correlation ID.

Output, Required: Result Indication.

# * * * * End of changes * * * *