3GPP TSG-SA WG2 Meeting #148E e-meeting	S2-2108628r01

# Elbonia, November 15 – 22, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0039 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications and corrections on UAV Re-authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. In clause 5.2.4.3, “PDU Session identifier” in the step 3b suggests to be replaced with “PDU Session IP address ” . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace ‘PDU session identifier’ in step 3b of clause 5.2.4.3 with ‘PDU session IP address’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear to be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.2.4.1, 5.2.4.2, 5.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change *****

5.2.4.3	USS initiated UAV Re-authorization procedure in 5GS

Figure 5.2.4.3-1: UAV Re-authorization procedure in 5GS

UAS NF stores the UE UUAA context after successful UUAA procedure as explained in clause 5.2.2.2 for UUAA-MM and in clause 5.2.3 for UUAA-SM procedure. The UUAA context may be stored in the UDSF or may be stored locally in the UAS NF depending on deployments.

1.	The USS sends a request to UAS NF for re-authorization of the UAV. The USS includes GPSI, CAA-Level UAV ID, an authorization message to be transparently delivered to the UAV. The CAA-Level UAV ID may be a new CAA-Level UAV ID. The authorization message may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information.

2.	Based on the received GPSI the UAS NF retrieves the correspondingly stored UUAA context which includes information on whether a UUAA-MM or a UUAA-SM has been performed and in case of UUAA-SM the identity of the SMF and the IP address of the PDU session that is serving the UUAA.

3a	If UUAA-MM was performed, the UAS-NF/NEF retrieves the AMF ID that serves the UE from the UDM based on the GPSI. and then sends a Nnef_Auth_Notification request including the CAA-Level UAV ID and the authorization message to the serving AMF.

3b	If UUAA-SM was performed, the UAS-NF/NEF sends a Nnef_Auth_Notification request to the SMF serving the UUAA which includes the corresponding PDU session IP address, CAA-Level UAV ID and the authorization message.

4.	The UAS NF responds back to the USS indicating that re-authorization request has been successfully initiated.

5a.	In the case of UUAA-MM:

If the UE is in CM_Idle state, the AMF initiates the Network Triggered Service Request procedures as described in clause 4.2.3.3 of TS 23.502 [3].

The AMF initiates re-authentication of the UAV as described in steps 4c to 9 of UUAA-MM procedure, clause 5.2.2.2. during which the CAA-Level UAV ID and the authorization message is sent to the UE.

5b	In the case of UUAA-SM:

The SMF identifies, based on the received information, the PDU Session that is serving the UUAA-SM and invokes the Network Requested PDU Session Modification procedure (figure 4.3.3.2-1 of TS 23.502 [3] triggering event SMF Requested modification) by sending Namf_Communication_N1N2MessageTransfer, including the CAA-Level UAV ID and the authorization message in the N1_SM_Container (step 3b in figure 4.3.3.2-1 of TS 23.502 [3]) .

The Network Triggered service request procedure is invoked by AMF to forward the CAA-Level UAV ID and the authorization message included in the N1_SM_container to the UE (from step 3a in figure 4.2.3.3-1 of TS 23.502 [3]).

6.	The UE receives the CAA-Level UAV ID and the authorization message (which may e.g. include a UUAA Authorization Payload, a C2 Authorization Result and/or (C2) Security Information) and act on it accordingly (outside the scope of 3GPP).

*** End of changes ***