3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108667r01

# Elbonia, November 15 –22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0064 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NF services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current specification, clause 9 is for defining the NF services. In this clause we define the inputs/outputs for each services, and a few ENs left for further clarification. Since currently the associated procedures are defined, it is sufficient to specifiy the NF service parameters and resolve the ENs now. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the MB-SMF service: - For Nmbsmf_TMGI_Allocate service clarify the optional input parameters could be already allocated TMGI, to extend the expiry time. For the AMF service: - Remove the ENs; - Correct the service operation name of Namf_MBSCommunication_N2MessageTransfer; - For the exact IEs in N2 container, use "e.g.". For the NEF service: - Remove the ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved EN, and inaccurate NF service description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9, 9.1.2.2, 9.1.3.6, 9.3.2, 9.3.3.2, 9.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

9	Network Function Services

# * * * * Second change * * * *

9.1.2.2	Nmbsmf_TMGI_Allocate service operation

Service operation name: Nmbsmf_TMGI_Allocate

Description: This service is used by the NF Service Consumer to request the allocation of TMGI(s) or request to refresh previously allocated TMGI(s).

Inputs, Required: Number of TMGIs.

Inputs, Optional: TMGI(s) (i.e., the TMGI(s) to be extended the expiry time).

Outputs, Required: TMGI(s), Expiry Time of the TMGI(s).

Outputs, Optional: None.

# * * * * Third change * * * *

9.1.3.6	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS session configuration. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Session ID (source specific multicast address or TMGI) or TMGI request.

Input, Optional: DNN, S-NSSAI, , MBS service area, MBS activation time, MBS termination time, service description, , QoS flow information, Input Transport Address Request, session activity status (active/inactive). For a multicast session, indication that any UE may join. For subscription to notifications event ID(s), notification target address.

Output, Required: Result Indication.

Output, Optional: TMGI, Expiry Time of the TMGI, Cause, MB-UPF tunnel info.

# * * * * Fifth change * * * *

9.3.2	Namf_MBSBroadcast service

9.3.2.1	General

Service description: This service is used by the consumer for broadcast communication.

9.3.2.2	Namf_MBSBroadcast_ContextCreate service operation

Service operation name: Namf_MBSBroadcast_ContextCreate

Description: This service operation is used to create the broadcast session context towards the AMF.

Inputs, Required: MBS Session ID, Broadcast service area, N2 container (e.g., MBS Session ID, MBS QoS profile, Broadcast service area).

Inputs, Optional: Area Session ID, Optional parameters in the N2 container: MBS IP Multicast Tunnel Info, Area Session ID.

Outputs, Required: Result Indication.

Outputs, Optional: N2 container (NG-RAN MBS Tunnel Info).

# * * * * Sixth change * * * *

9.3.3.2	Namf_MBSCommunication_N2MessageTransfer service operation

Service operation name: Namf_ MBSCommunication _N2MessageTransfer

Description: This service operation is used by the NF Consumer to request the AMF to transfer the MBS related N2 message to the NG-RAN nodes serving the MBS multicast session.

Input, Required: MBS Session ID, N2 SM information.

Input, Optional: MBS area session ID.

Output, Required: Result Indication.

Output, Optional: Cause.

# * * * * Seventh change * * * *

#### 9.4.3.1	General

Service description: This service operates on the multicast and broadcast MBS sessions. The following are the key functionalities of this NF service:

-	Create/Update Delete for both multicast and broadcast MBS sessions;

-	Provide a Notification about subscribed events. The following events are supported:

-	MBS session release due to TMGI expiry.

-	Broadcast delivery status.

# * * * * End of changes * * * *