3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108688r04

# Elbonia, November 15 – 22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0484 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Termination request or rejection of NWDAF data managment service due to user consent not granted |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  | Date: |  | 2021-11-15 |
| Category: | F |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 23.288, clause 6.1.1, 6.1.2 and 7.2.4, there is some description that NWDAF can detetmine whether to provide analytics further according to the user consent information, if user consent is revoked, then the NWDAF shall send a termination request in Nnwdaf_AnalyticsSubscription_Notify to requests consumer to terminate the analytics subscription. In S2-2107825, it has been approved that NWDAF can detetmine whether to accept or reject analytics subscription according to the user consent information, if user consent is not granted, then the NWDAF shall send an error response in Nnwdaf_AnalyticsSubscription_Response, indicating the subscription is not accepted due to user consent not granted. Actually, in addition to analytics subscription/request service, the NWDAF can also provide data or historical analytics(as a kind of data) to consumer by using data management service. However, there is no corresponding description that the NWDAF can reject or request the consumer to cancel the subsciption to the required data based on user consent not (or no longer) granted. |  |  |  |  |  |
| Summary of change: |  | - Add a clarification that if user consent is not granted, then the NWDAF shall send an error response to the consumer in Nnwdaf_DataManagment _Response indicating the data subscription is not accepted due to user consent not granted - Add a clarification that if user consent is revoked, then the NWDAF shall send a termination request in Nnwdaf_DataManagment _Notify to requests consumer to terminate the data subscription. |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear that NWDAF can reject or request the consumer to cancel the subsciption to the required data based on user consent not (or no longer) granted. |  |  |  |  |  |

| Clauses affected: | 6.2.6.2, 7.4.2, 7.4.4 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | N |  |  |
| Other specs |  | x | Other core specifications | TS/TR ... CR ... |
| affected: |  | x | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | x | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |
| This CR's revision history: |  |  |  |  |

*************** Start of changes ***************

#### 6.2.6.2	Procedure for Data Collection from NWDAF

The procedure in Figure 6.2.6.2-1 is used by NWDAF service consumer to invoke the data management services at NWDAFs in order to retrieve runtime and historical data.

Figure 6.2.6.2-1: Data Collection from NWDAF via Data Management Service

1.	NWDAF service consumer (e.g. NWDAF, DCCF) identifies that further data from an NWDAF is required in order to perform some operation related to analytics ID. The triggers for further data collection are related to:

a)	the local policies of NWDAF, DCCF (e.g. preparation for future requests for analytics ID as specified in clause 6.2.2.1);

b)	a request for analytics generation requiring data not available or not directly reachable via the NWDAF service consumer (e.g. out of the serving area);

c)	a request for model training;

d)	a request for data collection that NWDAF service consumer cannot provide by itself.

NOTE 1:	If the NWDAF service consumer is a DCCF, the discovery of the proper NWDAF is defined in clause 6.2.6.3.6. If the NWDAF service consumer is a NWDAF, the NWDAF service consumer can discover the appropriate NWDAF(s) as defined in clause 5.2.

2a.	NWDAF service consumer invokes Nnwdaf_DataManagement_Subscribe service from NWDAF to request a required data. The request comprises the Data Specification as well as Bulked Data Formatting and Processing as defined in clause 6.2.6.1, Notification Target Address (+ Notification Correlation ID).

When the required data is Event IDs, the NWDAF service consumer may include the Data Source, e.g. NF Instance (or NF Set) ID from which the data needs to be collected.

The NWDAF service consumer may include ADRF information indicating whether the data are to be stored in an ADRF, and optionally an ADRF ID.

The NWDAF service consumer may include ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data (optional), directing NWDAF to the repository containing historical data.

The NWDAF checks if required data is related to a user, i.e., SUPI or GPSI, then, depending on local policy and regulations, as described in clause 6.2.9, the NWDAF checks or has checked the user consent by retrieving the user consent information from UDM using Nudm_SDM_Get including data type "User consent". If user consent is not granted, NWDAF sends a response to the NWDAF service consumer in step 2b, indicating that user consent for data collection was not granted, and the data collection for this SUPI or GPSI stops here. If the user consent is granted, the NWDAF can provide the required data to the NWDAF service consumer by performing the following steps 2b-7, and the NWDAF subscribes to UDM to notifications of changes on subscription data type "User consent" for this user using Nudm_SDM_Subscribe. When receiving the notification that user consent has been revoked, the NWDAF shall provide a Termination Request in Nnwdaf_DataManagement_Notify to request the NWDAF service consumer to cancel the subscription to the required data.

2b.	Based on the received request, NWDAF creates a new bulked data for the requesting consumer. NWDAF sends Nnwdaf_DataManagement_Subscribe service response with a confirmation of successful request and the subscription correlation ID identifying the requested data.

NOTE 2:	Subscription Correlation ID allows the NWDAF service consumer to request to NWDAF any changes in the generation of a requested data.

3.	NWDAF determines whether the request data is available at such NWDAF.

NWDAF maintains a local association of requested Event IDs or Analytics IDs to the list of triggered event subscription identifications from data sources to generate the requested data. Based on this local association, the NWDAF checks if the data to be collected is available at itself. If the data is available, NWDAF uses such data to generate the bulked data.

When data sources are NFs, the NWDAF discovers the proper NFs as defined in clause 6.2.2.1.

When the data sources are other NWDAFs, the NWDAF discovers the other NWDAFs as defined in clause 5.2.

When the data source is DCCF, the NWDAF discovers the proper DCCF as defined in clause 6.3.19 of TS 23.501 [2].

4a.	(Optional) If NWDAF receives a request for data that is not available or not reachable by such NWDAF (e.g. out of serving area), NWDAF determines the sources for the data that is not available, if the information has not been included in the subscription to the requested data.

4b.	(Optional) NWDAF may trigger further data collection using any of the available mechanisms in clause 6.2.2 (e.g. if the data subscribed in step 2a partially matches data that are already being collected by the NWDAF from a data source and a modification of the subscription to the data source would satisfy both the existing data collection as well as the newly requested data) and clause 6.2.6 (e.g. recursively using data collection services from other needed NWDAFs, DCCFs, ADRFs, NFs).

NWDAF updates its local association of the mapping of the requested data (Event ID or Analytics ID) to the identification of the request/subscription for data collection from the further data sources.

5.	Based on the properties of the received request and considering local bulked data instructions, NWDAF generates the requested data including the available or collected data (e.g. from other NWDAFs, DCCFs or ADRFs, NFs).

6a.	If the fetch flag is set to true in step 2a, NWDAF waits until the requested data is ready and sends a Nnwdaf_DataManagement_Notify service message with the Notification Correlation ID, the Fetch Correlation ID and the target address where the data may be retrieved.

The requested data is ready when NWDAF generated the bulked data based on the available or collected data samples from the event notifications and the applicable bulked data processing and formatting instructions.

6b.	If the fetch flag is set to false in step 2a, NWDAF uses the Nnwdaf_DataManagement_Notify service to send the Notification Correlation ID and requested data to the NWDAF service consumer.

NWDAF waits until the bulked data is generated or the bulked data deadline expires and uses Nnwdaf_DataManagement_Notify service with the indication of the outcome. In case of successful bulked data generation, NWDAF provides the bulked data via Nnwdaf_DataManagement_Notify service as defined in clause 6.2.6.1. In case of bulked data deadline expiration and unsuccessful bulked data generation, NWDAF provides the bulked data via Nnwdaf_DataManagement_Notify an indication of an unsuccessful bulked data generation with expired deadline. The Notification Target Address is used by the Nnwdaf_DataManagement_Notify service operation to deliver the message to the NWDAF service consumer.

7(a.b).	Alternatively, when the fetch flag is set to true in Step 2a, the NWDAF service consumer shall fetch the required data from NWDAF via Nnwdaf_DataManagement_Fetch service operation. The NWDAF service consumer invokes the Nnwdaf_DataManagement_Fetch service operation with the input parameters including the Notification Correlation ID (+Fetch Correlation ID), that identifies the data to be fetched and receives a response with the requested data, as defined in clause 6.2.6.1.8.	The NWDAF service consumer uses the requested data for performing further processing. If the NWDAF service consumer is an NWDAF the requested data can be used for analytics generation or model training or for further exposing such data to other NWDAFs. If the NWDAF service consumer is a DCCF, the requested data can be used to be provided to a DCCF data consumer.

9.	When the NWDAF service consumer determines that no more data is required or if receiving a Termination Request from the NWDAF, e.g. due to user consent revocation for the data collection related to a user, it unsubscribes to the requested data from NWDAF. If NWDAF had triggered further data collection in Step 3a and 3b, NWDAF also unsubscribe to all data sources.

NOTE 3:	It is also possible that instead of providing the dataset of the generated bulked data in steps 6a, 6b, 7b, the NWDAF provides a reference to where the dataset can be retrieved by the NWDAF service consumer.

*************** Next changes ***************

### 7.4.2	Nnwdaf_DataManagement_Subscribe service operation

Service operation name: Nnwdaf_DataManagement_Subscribe.

Description: The consumer subscribes to receive data or analytics, or if the data is already defined in NWDAF, then the subscription is updated.

Inputs, Required: Data Specification or Analytics Specification, Notification Target Address (+ Notification Correlation ID).

When the required data is a bulked data for Event IDs received from NFs, the Data Specification includes: set of Event IDs, Event Filter Information, Target of Event Reporting, and bulked data type as defined in clause 6.2.6.1.

When the required data is a bulked data for Analytics ID, the Data Specification includes: Target of Reporting with the set of Analytics ID(s) to generate bulked data, bulked data type, and analytics stage; Filter Information with Target of Analytics Information, Analytics Filter Information as defined in clause 6.2.6.1.

When Analytics Specification is included in the required input parameters it identifies the historical analytics to be collected.

NOTE 1:	Event Filter Information, Target of Event Reporting and Bulked data type can be provided per individual Event ID in a set of Event IDs to generate bulked data.

NOTE 2:	Bulked data type, analytics stage, Target of Analytics Information, Analytics Filter Information can be provided per individual Analytics ID in a set of Analytics IDs to generate bulked data.

Inputs, Optional: Service Operation, Bulked Data Formatting and Processing, Data Source, ADRF information to store data used for generated bulked data, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data to be used for bulked data generation.

The Bulked Data Formatting and Processing parameters include: the parameters defined in clause 5A.4 as well as Periodic bulked data notification, Feature type, Time Window, Minimum and/or maximum number of samples, Fetch flag, Bulked data deadline, Notification Event Clubbing, Processing rules.

NOTE 3:	The Service Operation (in the case of Event IDs), Bulked Data Formatting and Processing, Data Source, and ADRF information to store data used for generated bulked data, ADRF ID or NWDAF ID (or ADRF Set ID or NWDAF Set ID) storing historical data to be used for bulked data generation can be provided per individual Event ID or Analytics ID included in the Data Specification.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of the requested subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

NOTE 4:	When the Target of Event Reporting or Target of Reporting is a SUPI or a GPSI then the subscription may not be accepted, e.g., for user consent is not granted, and an error is sent to the consumer. When the Target of Event Reporting or Target of Reporting is an Internal Group Id, or a list of SUPIs/GPSI(s) or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

*************** Next changes ***************

### 7.4.4	Nnwdaf_DataManagement_Notify service operation

Service operation name: Nnwdaf_DataManagement_Notify.

Description: NWDAF notifies the consumer instance of the requested data according to the request.

Inputs, Required: Notification Correlation ID, time stamp.

Inputs, Optional:

- Requested Data,

- Fetch Correlation ID,

- Target address where the data may be retrieved in case the Fetch Correlation ID is included in the notification,

- Unsuccessful bulked data generation,

- Expired bulked data deadline.

- Termination Request: this parameter indicates that NWDAF requests to terminate the data management subscription, i.e., NWDAF will not provide further notifications related to this subscription.

NOTE:	The Requested Data or Target address can be provided per Event ID or per analytics ID specified in the Data Specification field of the subscription.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

*************** End of changes ***************