3GPP TSG-WG SA2 Meeting #148E e-meeting  	S2-2108721r01

# Elbonia, November 15 – 19, 2021	(revision of S2-21XXXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3278 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for network slice notification procedure for untrusted AF case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In SA2#147E, a CR (S2-2107114) was discussed and one aspect to check is whether the AF considered to be untrusted by the operator (untrusted AF) can be assumed to be able to provide the S-NSSAI when the untrusted AF wants to subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NEF. Considering the security aspect, in case of 3rd party AF or untrusted AF, it is normal that such AF should not know about the S-NSSAI. But it is supposed that such AF will provide an external parameter, which would then be translated or mapped into the internal parameter. For example, in TS 33.501, the untrusted AF sends the External Network Slice Information (ENSI) to the NEF, and the NEF translates it further to S-NSSAI. Alternatively, the AF-Service-Identifier may also be used for such translation/mapping to the S-NSSAI as specified in clause 6.2.5.0 in TS23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies that external parameter (e.g., AF-Service-Identifier or ENSI) should be used for translation/mapping to the S-NSSAI, when the untrusted AF wants to subscribe or unsubscribe for the number of UEs or the number of PDU Sessions per network slice notification with the NEF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear on how the untrusted AF would communicate with NEF for subscribing for the number of UEs or the number of PDU Sessions per network slice notification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 4.15.3.2.10a	Number of UEs and PDU Sessions per network slice notification procedure for untrusted AF

The AF considered not to be trusted by the network operator subscribes or unsubscribes for the number of UEs or the number of PDU Sessions per network slice notification with the NEF as per the description in Figure 4.15.3.2.10-1 with the following differences:

-	Step 1, in the Nnef_EventExposure_Subscribe/Unsubscribe Request message to the NEF, the AF considered not to be trusted by the network operator does not provide the S-NSSAI but provides the AF-Service-Identifier as defined in TS 29.522 as part of the Event Filter parameter, for which reporting is required. The AF-Service-Identifier is used by the NEF to translate it to the S-NSSAI.

- 	Step 7b, and 9 where the AF-Service-Identifier is included in the returned notification replacing the S-NSSAI.

# * * * * End of changes * * * *