3GPP TSG-SA2 Meeting # 148E (e-meeting)	S2-2108977

# Elbonia, November 15 - 19, 2021												   (revision of S2-2108014)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0021 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the local MBS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Huawei, HiSilicon, LG Electronics,] Nokia, Nokia Shanghai-Bell, [Samsung, ZTE, vivo] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - To support the individual delivery, the SMF need subscribe the UE mobility event from AMF. - For the location dependent MBS session, the Area Session ID and MBS service area are also need be registered to NRF. So it is unsuitable to assume if the MBS session ID is TMGI, the registration can be skipped. - Add new clause about the local MBS session activation and termination of the local MBS session. - For the broadcast MBS service, there are no SMF need be involved. So the NOTE about the MB-SMF need be same for the one SMF service area is unclear on how these two are related. 148e - Rev03 - Editor´s note from 147e needs to be addressed: Editor's note: Procedures for the UE mobility between cells of one RAN node, e.g. when the UE leaves, enters or switches service areas due to mobility are ffs. For shared delivery it is FFS whether, the RAN node handles content switching due to mobility between service areas it serves autonomously or the core network needs notifications about this mobility to select new service areas. For the UE leaving a service area it is ffs whether the RAN node defers related notification for a grace period or the SMF defers removing UEs from the multicast session. Details require coordination with RAN |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the UE mobility event subscritption description - Remove the MB-SMF can skip the registration part description. - Add new clause about the local MBS session activation and termination of the local MBS session. - Remove unclear NOTE description. - The RAN node switches autonomously between service areas for location based services - The RAN node notifies the SMF only if the UE leaves the combined service area |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local MBS for individual delivery will not be supported for the Activtion, procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.3, 7.2.4.1, 7.2.4.2.1, 7.2.4.2.2, 7.2.4.3.1, 7.2.4.3.2, , 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 6.2.1	General

A Local MBS service is an MBS service provided in one MBS service area. A location dependent MBS service is an MBS service provided in several MBS service area(s). An MBS service area is identified by a cell list or a tracking area list. The MBS service area could be geographical area information or civic address information, and NEF/MBSF translates the location information to Cell ID list or TAI list as MBS service area, see clause 7.1.1.2.

For multicast communication, local MBS and location dependent MBS services may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS.

For more details, refer to clause 7.2.4 for multicast MBS Session and refer to clause 7.3.4 for broadcast MBS Session.

# * * * * Next change * * * *

### 6.2.2	Local MBS service

For a local MBS service, only UEs within the MBS service area may receive content data, while UEs outside the MBS service area are not allowed to receive location specific content. For multicast MBS service, UEs outside the MBS service area are not allowed to join the MBS service, and the network shall not deliver location specific content anymore to the UEs moved out of the MBS service area. Depending on policy, for the multicast MBS service the network may remove UEs outside the MBS service area of the MBS session from the MBS session context after a grace period (e.g. by using a local configured timer). The SMF may subscribe at the AMF to notifications about UE moving in or out of a subscribed "Area Of Interest"" event using the Namf_EventExposure service.

For multicast communication, local MBS may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS. If the SMF obtains a notification via the Namf_EventExposure service about the UE is no longer in the MBS service area, the SMF terminates the 5GC Individual MBS traffic delivery towards the UE.

The UE shall be able to obtain service area information of the local multicast service via MBS service announcement or via NAS signalling (UE Session Join Accept/Reject including Cell ID list or TAI list). If the UE Session Join procedure fails due to the UE being outside the MBS service area, the UE does not attempt to join the multicast session again until the UE moves inside the MBS service area. When the UE Session Join succeeds and if the multicast session is deactivated, the UE does not perform monitoring the session activation notification and any other information related to the multicast session identified by an MBS Session ID over the radio if outside the MBS service area.

NOTE:	Broadcast communication service is the service provided simultaneously to all UEs in a geographical area, therefore for broadcast it is naturally a local MBS service.

For more details, refer to clause 7.2.4 for multicast MBS Session and refer to clause 7.3.4 for broadcast MBS Session.

# * * * * Next change * * * *

### 6.2.3	Location dependent MBS service

A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location-dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE. For multicast MBS service, UEs outside the combined MBS service area (consisting of the area covered by all MBS service areas of the location dependent MBS service) are not allowed to join the MBS service, When UE moves out of an MBS service area and there is no other MBS service area for the MBS session, the network ceases to deliver the content data to the UE. Depending on policy, for the multicast MBS service the network may remove UEs outside the combined MBS service area of the MBS Session from the multicast MBS session context after a grace period (e.g. by using a local configured timer). The SMF may subscribe at the AMF to notifications about UE moving in or out of the combined MBS service area of the location dependent MBS session.

For multicast communication towards a RAN nodes supporting MBS, the RAN nodes handle the movemt of UEs within the MBS session between MBS service areas it serves autonomously and obtains information about all service areas it serves from the MB-SMF.

For multicast communication, location dependent MBS services may be supported via 5GC Individual MBS traffic delivery towards RAN nodes not supporting MBS. The SMF additionally subscribes at the AMF to notifications about the "UE location" or UE moving in or out of a subscribed "Area Of Interest" event (with a a single service area as area of interest) using the Namf_EventExposure service. If the SMF obtains a notification about the UE location, it checks whether the UE is still in the MBS service area of the multicast session. If the UE is no longer in the current MBS service area, the SMF determines whether the UE is in another MBS service area of the multicast session; if so, the SMF configures the UPF to send multicast data relating to the new MBS service area towards the UE. If the SMF terminates the 5GC Individual MBS traffic delivery towards the UE, it unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

Information about different MBS service areas for a location dependent MBS service may be provided by one or several AFs or may be configured. Different ingress points for location dependent points for the MBS session are supported for different MBS service area dependent content of the MBS session; different MB-SMFs and/or MB-UPF may be assigned for different MBS service areas in an MBS session. When the different MB-SMFs are assigned for different MBS service areas in an MBS session, the same TMGI is allocated for this MBS session.

The Area Session ID is allocated by MB-SMF in MBS Session configuration procedure. MB-SMF allocates Area Session ID for each MBS services area which is unique within the MBS session. MB-SMF needs to further ensure there is no MBS service area overlapping with other MBS service areas that share the same MBS session ID.

NOTE 1:	In this release, deployments topologies with specific SMF Service Areas are not supported, as a result, location dependent service using multicast communication is not supported when a UE moves outside its SMF service area.

NOTE 2:	For location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

NOTE 3:	An example of Location-dependent MBS is a nationwide weather forecast service with local weather reports.

NOTE 4:	Area Session ID is equivalent to Flow ID as specified in TS 23.246 [8].

For more details, refer to clause 7.2.4 for multicast MBS Session and refer to clause 7.3.4 for broadcast MBS Session.

# * * * * Next change * * * *

### 7.2.4	Support of Local multicast service and Location dependent multicast service

#### 7.2.4.1	General

The clause captures the procedural enhancement to supportLocation-dependent MBS service and the Local MBS service described in clause 6.2.

#### 7.2.4.2	Support of location-dependent multicast service

##### 7.2.4.2.1	UE join location dependent multicast session and establishment procedure

The location dependent multicast session join and establishment procedure is performed as defined in clause 7.2.1 with the following additions:

-	The location dependent multicast session is configured as described in clause 7.2.4.2.2.

-	The UE may have information about thze location dependent multicast service including a combined MBS service area (consisting of the area covered by all MBS service areas of the location dependent MBS service) and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the combined MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the combined MBS service area, the UE does not send the Join Request.

Editor´s note: the following addition to this step is FFS: The AMF provides the User Location Information of the UE towards the SMF.

The AMF includes this information in all Nsmf_PDUSession_UpdateSMContext request if the DNN and S-NSSAI allow multicast.

-	If SMF has no information about the multicast MBS session context for the indicated MBS Session ID, the SMF requests MB-SMF information via Nnrf_NFDiscovery_Request Request (MBS Session ID, UE location), the NRF provides information about the MB-SMF(s) serving the multicast session at the indicated location and service areas and service area IDs for the multicast session, via Nnrf_NFDiscovery_Request Response (MB-SMF profile (Area Session ID(s), MBS service area(s)). The SMF selects the MB-SMF based on the location area where the UE is residing and interacts with MB-SMF to retrieve QoS information of the multicast QoS flow(s) for the MBS Session ID.

Editor´s note: the following addition is FFS The SMF determines whether the UE is inside the combined MBS service area by comparing the User Location Information of the UE received together with the join request with the MBS service areas received from the MB-SMF.

-	If the UE is out of the combined MBS service area of the whole MBS session, the SMF rejects the multicast session join request.

-	The SMF requests the AMF to transfer an N2 message to the RAN node using the Nsmf_PDUSession_UpdateSMContext response, to provide the NG-RAN with multicast session information which additionally includes the Area Session ID and MBS service area associated with the cell where the UE camps..

-	If NG-RAN node supports MBS, the NG-RAN uses the received MBS Session ID(s) and Area Session ID(s) to determine the local multicast session context and whether the user plane for the local multicast session is already established. If the target RAN determines the shared delivery is not established for the MBS session ID and area session ID, the target NG-RAN node initiates the shared delivery establishment as specified in clause 7.2.1.4. The MB-SMF provides MBS service areas (Area session ID(s), MBS service area(s)) in the shared delivery establishment reply.

Editor´s note: It is FFS whether the MB-SMF provides only the service areas assoziated with the target RAN node or all service areas in the shared delivery establishment reply.

-	The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE leaving the combined service area (consisting of all service areas) of the location-dependent MBS session.

-	If the NG-RAN node serving the UE does not support MBS and UE is in the MBS service area, the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via individual delivery within a PDU session of the UE. The SMF additionally subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the later case the SMF supplies the service area of the multicast session where the UE resides as Area Of Interest.

##### 7.2.4.2.2	Configuration for location-dependent MBS

For location-dependent MBS, the configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same multicast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the MBS Session ID, MBS service area and Area Session ID.

NOTE:	For a location dependent service provided in different MBS service areas within the same SMF service area, it is assumed that one MB-SMF is used for an MBS Session.

-	The policy of Multicast session is determined based on the service requirements per MBS session. MB-SMF associate the same service requirement QoS flow in different Area Session with the same QFI

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The whole MBS service area(s) for the MBS session are indicated to the UE in the Service Announcement as defined in clause 6.11.

##### 7.2.4.2.3	Handover procedure

The Handover procedure for the UE is performed as defined in clause 7.2.3.2, 7.2.3.3, and 7.2.3.4 with the following additions:

-	If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Shared MBS traffic delivery, for the Xn Handover (compare with clause 7.2.3.2) the following applies:

-	If the UE is camping at Source RAN supporting MBS, the Source RAN node includes MBS Session ID, Area Session ID and MBS service area where the UE resides to the Target RAN node.

-	If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Shared MBS traffic delivery, for the N2 Handover (compare with clause 7.2.3.3) the following applies:

-	The source RAN node includes the MBS session area information (MBS Session ID, Area Session ID and MBS service area) where the UE is residing to the Target RAN node in Handover Required message in the transparent container

-	When sending the Handover Request, the SMF forwards the transparent information about the (MBS Session ID, Area Session ID and MBS service area) and always includes the associated QoS flows.

NOTE 1:	The SMF cannot determine the UE location and a possible new service area at this stage. Depending on implementation it can include the MBS session area information based on the last UE location it knows; however the information in the transparent container takes precedence for the target RAN node.

-	If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Individual MBS traffic delivery before the Handover, the following applies:

NOTE 2: During the Xn handover procedure, the associated QoS flow is established at the Target RAN side regardless whether the UE is still in the MBS service area associated with the original area session ID or not

For the N2 handover the SMF includes MBS session area information (MBS Session ID, Area Session ID, and MBS service area) associated with the last known service area of the UE in N2 SM information to the Target RAN node in Handover Request message.

-	The SMF determines the MBS service area where the UE is located by comparing the Cell ID and current tracking area IDs of the RAN node (for N2 handover received in Handover Notify message and for Xn handover received in the Path Swich message) with the MBS service areas received from the MB-SMF.

-	The SMF uses the determined service area and user location as follows:

-	If the UE is outside all service areas of the MBS session, per operator’s policy (e.g. when a local configured timer expires since the UE left the whole MBS service area), the SMF may remove the UE from the MBS session as defined in clause 7.2.4.2.5.

-	The SMF updates the area session ID if the area session ID in the locally stored the UE MBS session context with the corresponding area session ID if the area session ID is changed.

-	If the UE is in a new service area of the MBS session compared to the service area previously indicated in the PDU session, and the Target NG-RAN supports MBS, the SMF provide the NG-RAN node with multicast session information which includes the Area Session ID and MBS service area where the UE is residing. For Xn handover, the SMF uses the Path switch response message. For N2 handover, the SMF updates the PDU session after the completion of the handover procedure.

-	If the Target RAN node does not support MBS:

-	If the UE is in a service area of the MBS session, and the Source RAN supported MBS, the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session and service area via unicast delivery within a PDU session of the UE. The SMF subscribes at the AMF using the Namf_EventExposure service to notifications about UE location changes, or to notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event. In the later case the SMF supplies the service area of the multicast session as Area Of Interest. If associated QoS flows are not included in the PDU session, the SMF updates the PDU session with associated QoS flows after the completion of the handover.

-	If the UE is in a service area of the MBS session, the SMF updates the UPF to forward the MBS data packet from the tunnel associated with the old Area session ID to the new Area session ID. If the SMF did not configure the UPF receiving the MBS data packet from the tunnel associated with the new Area session ID before, the SMF informs the MB-SMF of the new Area session ID and UPF DL N19mb Tunnel information. MB-SMF configures the MB-UPF to transmit the multicast session data towards UPF using the received downlink tunnel ID. If the SMF subscribed to the "Area Of Interest"" event, the SMF also updates the subscription with the new service area as "area of interest".

-	If the UE is in no service area of the MBS session, and associated QoS flows are included in the PDU session, the SMF updates the PDU session by removing associated QoS flows after the completion of the handover.

  - NOTE 3: The subscription for the UE entering or leaving the complete service area does not need to be changed

-	If the Target RAN node supports MBS, it determines whether to establish the resources for multicast distribution for MBS Session ID and Area Session ID , based on the received MBS Session ID, Area Session ID and MBS service area, and based on the location of the UE.

-	If the target RAN node supports MBS and determines the shared delivery is not established for the multicast session ID and area session ID, the target NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4. The RAN node provides its ID and tracking areas it serves in the establishment request. The MB-SMF determines the service areas served by the RAN node based on that information and provides the MBS service area (Area session ID(s), MBS service area(s)) associated with the same MBS session and served by the target RAN node.

NOTE 4:	During the handover procedure the associated QoS flow information as applicable before the handover is not modified to separate mobilty management and session management.

# * * * * Next change * * * *

7.2.4.2.4	Activation of location dependent MBS session

When the location dependent MBS session is activated, the MBS session is activated in the whole MBS service area of the MBS session. It is not supported that the same MBS session is in active state in one MBS service area but in inactive state in another MBS service area.

For the location dependent MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which includes the whole MBS service area associated with the MBS session, i.e. the sum of all MBS service area associated with the MBS session ID regardless of the Area session ID.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request or Namf_MT_UEReachabilityInfoNotify message. Based on that information, the SMF determines how to activate the MBS session, handling the user location information in the same manner defined in clause 7.2.4.2.3

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area, and Area Session ID, in the N2 SM information.

- * * * * Next change * * * *

##### 7.2.4.2.5	UE mobility within the same NG-RAN node between cells belonging to different MBS service areas during shared delivery

The RAN node handles content switching due to mobility between service areas it serves autonomously. If the first UE enters a new location area the RAN node requests shared delivery from the MB-SMF for the corresponding area session ID. If the last UE leaves a location area the RAN node requests the termination of shared delivery from the MB-SMF for the corresponding area session ID.

Figure 7.2.4.2.5-1:	UE moving out of any service area for shared delivery and location dependent multicast session

1.	The UE was previously inside the combined service area and moves to another cell served by the same RAN node (intra-RAN node mobility) which is outside any service area of the multicast session, or the UE was previously outside the combined service area and moves to another cell served by the same RAN node (intra-RAN node mobility) which is inside the combined service area of the multicast session

2.	If the SMF subscribed to a notifications about the "UE moving out of any service area " it obtains a notification.

If the UE leaves the combined servive area, Tthe SMF may decide based on operator policy to remove the UE from the multicast session. Steps 3 to 5 then apply.

3.	The SMF may wait for a grace period before removing the UE from the multicast MBS session.

4.	The SMF updates the PDU session to inform the UE that it was removed from the multicast session (via a N1 container sent to the NG-RAN node and then forwarded to the UE) and to remove information related to the multicast session from the PDU session (e.g. QoS flows to deliver multicast data, via N2 contained to be processed by RAN node)

5.	The SMF unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

If the UE enters the combined servive area, steps 6 to 7 apply.

6.	The SMF determines the service area where the UE is located.

7.	The SMF updates the PDU session with information about the MBS service area and area session ID (via N2 contained to be processed by RAN node)

# * * * * Next change * * * *

##### 7.2.4.2.6	UE mobility within the same NG-RAN node between cells belonging to different MBS service areas during Individual delivery

Figure 7.2.4.2.6-1:	UE moving between service areas during Individual delivery for location-dependent multicast session

1.	The UE moves to another cell, either served by the same RAN node (intra-RAN node mobility) or by a different RAN node (handover).

2.	If the SMF subscribed to location changes, it obtains a notification about every cell change. If the notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event, it obtains notification when the UE enters or leaves the area of interest. If the SMF obtains a UE location change notification, it checks whether the new UE location is inside or outside the old service area of the MBS session where the UE was previously located (if any).

3.	If UE left a service area of the multicast session, the SMF decides to terminate individual delivery of MBS data for that old service area to UE. As detailed in the subsequent steps, the SMF also checks if UE is in another service area of multicast session. If so, the SMF configures individual delivers of MBS data for the new service area to UE. Otherwise the SMF may remove the UE from the MBS session

Steps 4 to 6 apply when the UE leaves the previous service area of the multicast session.

4.	The SMF configures the UPF to stop sending data related to the multicast session and service area via unicast delivery within a PDU session of the UE.

5.	If the UPF indicates back to SMF that it does not require data related to the multicast session and service area for other UEs, the SMF requests from the MB-SMF that data related to the multicast session and service area are no longer delivered to the UPF.

6.	If step 5 occurs, the MB-SMF configures the MB-UPF accordingly.

7.	If the SMF subscribed at the AMF to notifications related to the area of interest event and was only informed that the UE left the area of interest in step 2, the SMF may inquire the UE location from the AMF using the Namf_EventExposure service.

Steps 8 to 11 apply when the UE enters a new service area of the multicast session.

8.	If the SMF subscribed at the AMF to notifications related to the area of interest event, the SMF updates the subscription by supplying the new service area as Area Of Interest.

9.	The SMF configures the UPF to send data related to the multicast session and service area via unicast delivery within a PDU session of the UE.

10.	If the UPF indicates back to SMF that it does not yet receive data related to the multicast session and service area, the SMF requests from the MB-SMF that data related to the multicast session and service area are delivered to the UPF.

11.	If step 10 occurs, the MB-SMF configures the MB-UPF accordingly.

If the UE is in no service area of the multicast session, the SMF may decide based on operator policy to remove the UE from the multicast session. Steps 12 and 13 then apply.

12.	the SMF updates the PDU session to inform the UE that it was removed from the multicast session (via a N1 container sent to the RAN node and then forwarded to the UE) and to remove information related to the multicast session from the PDU session (e.g. QoS flows to deliver multicast data, via N2 contained to be processed by RAN node)

13.	The SMF unsubscribes at the AMF from the notifications about the UE location or the "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

# * * * * Next change * * * *

##### 7.2.4.2.7	SMF removing joined UEs from location dependent MBS session

Besides the condition as defined in clause 7.2.2.3, per operator's policy (e.g. when a local configured timer expires since the UE left the combined MBS service area) the SMF may remove the UE from the location dependent MBS session if the UE is out of the whole service area of the MBS session. When the UE is removed from the location dependent MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed Area Of Interest" event related to that MBS session.

# * * * * Next change * * * *

#### 7.2.4.3	Support of multicast service available within a limited area

##### 7.2.4.3.1	Local MBS service area information provided by AF

For local multicast services, the MBS service area information is provided to the UE and the 5GC as specified in clauses 7.1.1.2 and 7.1.1.3 with the following differences and clarifications:

-	For the Service Announcement, MBS service area information is included unless it is preconfigured.

-	When performing the MBS session request to the 5GC, the MBS service area information for a multicast session is provided by the AF unless it is preconfigured.

##### 7.2.4.3.2	Multicast session join and session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The multicast session join and session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request. The AMF provides the User Location Information of the UE towards the SMF.

NOTE 1:	The AMF includes this information in all Nsmf_PDUSession_UpdateSMContext request if the DNN and S-NSSAI allow multicast.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	The SMF determines whether the UE is inside the MBS service area by comparing the User Location Information of the UE received together with the join request with the MBS service area received from the MB-SMF.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-	The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

-	The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the RAN node serving the UE does not support MBS the SMF applies individual delivery towards the UE. The SMF configures the UPF to send data related to the multicast session via individual delivery within a PDU session of the UE.

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 2:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 3:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set by MBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6]. The SMF supplies the service area associated with the multicast session as Area Of Interest.

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

# * * * * Next change * * * *

##### 7.2.4.3.3	Handover procedure with limited area MBS session

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-	If the UE is camping at a Source RAN node supporting MBS, for Xn based handover in clause 7.2.3.2, the Source RAN node includes MBS Session ID and MBS service area to the Target RAN node during Handover Preparation phase. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request and Handover Required message, respectively. For N2 handover, when sending the Handover Request, the SMF forwards the transparent information about the (MBS Session ID, and MBS service area) and always includes the associated QoS flows and location area

NOTE 1:	The SMF cannot determine the UE location and whether it is within the service area at this stage.

NOTE 2:	During the Xn or N2 handover procedures, if the target RAN node does not 5MBS, the associated QoS flow is established at the Target RAN side regardless whether the UE is still in the MBS service area or not.

-	If the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID and Area Session ID via the 5GC Individual MBS traffic delivery before the Handover, the following applies:

-	For the N2 handover the SMF includes MBS session area information (MBS Session ID, Area Session ID, and MBS service area) associated with the last known service area of the UE in N2 SM information to the Target RAN node in Handover Request message.

NOTE 3:	During the Xn handover procedure, the associated QoS flow is established at the Target RAN side regardless whether the UE is still in the MBS service area or not.

-	If the Target RAN node supports MBS but the UE is no longer in the MBS service area, the Target RAN node does not allocate RAN resources for the MBS Session to the UE. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request acknowledge message.

-	The SMF determines whether the UE is outside the MBS service area by comparing the received Cell ID and tracking area ID (for N2 handover received in Handover Notify message and for Xn handover received in the Path Swich message) with the MBS service area received from the MB-SMF.

-	The SMF uses the determined UE location and the information whether the UE is outside the MBS service area as follows:

-	If the UE is outside the service area of the MBS session, per operator’s policy (e.g. when a local configured timer expires since the UE left the whole MBS service area), the SMF may remove the UE from the MBS session as defined in clause 7.2.4.2.5.

-	If the target RAN does not support MBS:

-	If the UE is inside the the MBS service area, the SMF applies individual delivery towards the UE. If associated QoS flows are not yet included in the PDU session, the SMF updates the PDU session with associated QoS flows after the completion of the handover. If the SMF did not configure the UPF to receive the MBS data packet from the tunnel associated with the multicast session before before, the SMF informs the MB-SMF of the MBS session and UPF DL N19mb Tunnel information. MB-SMF configure the MB-UPF to transmit the multicast session data towards UPF using the received downlink tunnel ID.

-	If the UE is in no service area of the MBS session, and associated QoS flows are included in the PDU session, the SMF updates the PDU session by removing associated QoS flows after the completion of the handover.

-	If the target RAN node support MBS, the target RAN determines whether to establish the resources for multicast distribution for MBS Session ID provided by SMF, based on MBS Session ID, MBS service area and UE location

NOTE 4:	During the handover procedure the associated QoS flow information as applicable before the handover is not modified to separate mobilty management and session management.

# * * * * Next change * * * *

##### 7.2.4.3.4	Activation of limited area MBS session

For the limited area MBS session activation, the differences comparing to the procedure defined in clause 7.2.5.2 are as below.

- The SMF invokes Namf_MT_EnableGroupReachability service operation to AMF, which include the MBS service area associated with the MBS session.

- For the UE in CM-IDLE state, when the AMF triggers the paging, it take the receiving MBS service area information into account. Only the NG-RAN node which is included in the MBS service area need trigger the paging.

- SMF checks whether the UE is in or out of the MBS service area based on the change notification or UE location information included in Nsmf_PDUSession_UpdateSMContext Request or Namf_MT_UEReachabilityInfoNotify message. Based on that information, the SMF determines how to activate the MBS session, handling User Location Information in the same manner as described in clause 7.2.4.3.3.

- When the SMF provides the MBS session information to the NG-RAN nodes, it includes the MBS service area in the N2 SM information.

# * * * * Next change * * * *

##### 7.2.4.3.5	UE mobility in or out of service areas

Figure 7.2.4.3.6-1:	UE moving in or out of service area during Individual delivery for local multicast session

1.	The UE moves to another cell served by the same RAN node (intra-RAN node mobility).

2.	If the SMF subscribed to location changes, it obtains a notification about every cell change. If the notifications about the "UE moving in or out of a subscribed "Area Of Interest"" event, it obtains notification when the UE enters or leaves the area of interest. If the SMF obtains a UE location change notification, it checks whether the new UE location is inside or outside the service area of the MBS session.

Steps 3 to 7 apply when the UE leaves the service area of the multicast session.

Step 3 to 5 apply if individual delivery is used before

3.	The SMF configures the UPF to stop sending data related to the multicast session via unicast delivery within a PDU session of the UE.

4.	If the UPF indicates back to SMF that it does not require data related to the multicast session for other UEs, the SMF requests from the MB-SMF that data related to the multicast session are no longer delivered to the UPF.

5.	If step 4 occurs, the MB-SMF configures the MB-UPF accordingly.

The SMF may decide based on operator policy to remove the UE from the multicast session. Steps 6 to 8 then apply.

6.	The SMF may wait for a grace period before removing the UE from the multicast MBS session.

7.	The SMF updates the PDU session to inform the UE that it was removed from the multicast session (via a N1 container sent to the NG-RAN node and then forwarded to the UE) and to remove information related to the multicast session from the PDU session (e.g. QoS flows to deliver multicast data, via N2 contained to be processed by RAN node)

8.	The SMF unsubscribes at the AMF from the notifications about the UE location or "UE moving in or out of a subscribed "Area Of Interest" event using the Namf_EventExposure service.

Steps 9 to 11 apply when the UE enters the service area of the multicast session and the UE is served by a non MBS capable RAN node.

9.	The SMF configures the UPF to send data related to the multicast session via unicast delivery within a PDU session of the UE.

10.	If the UPF indicates back to SMF that it does not yet receive data related to the multicast session, the SMF requests from the MB-SMF that data related to the multicast session are delivered to the UPF.

11.	If step 10 occurs, the MB-SMF configures the MB-UPF accordingly.

# * * * * Next change * * * *

##### 7.2.4.3.7	SMF removing joined UEs from limited area MBS session

Besides the condition as defined in clause 7.2.2.3, per operator's policy (e.g. when a local configured timer expires since the UE left the MBS service area) the SMF may remove the UE from the limited areaMBS session if the UE is out of the service area of the MBS session. When the UE is removed from the limited area MBS session, the SMF also unsubscribes to the AMF from the notifications about the "UE moving in or out of a subscribed Area Of Interest" event related to the MBS session.

# * * * * Next change * * * *

### 7.3.4	Support for Local Broadcast Service

The procedures for Local broadcast service contains the ones for local broadcast service with the location-dependent content as described in clause 6.2.

The configuration procedure is performed as defined in clause 7.1.1.2 with the following additions:

-	Multiple AFs may start the same broadcast session with different content in different MBS service areas. The NEF selects MB-SMF as ingress control node(s) for different MBS service areas.

-	If presented, the NEF maps possible external identifiers for MBS service areas to network-internal identifiers (e.g. list of cells, TAIs).

-	MB-SMF allocates Area Session ID, and updates its NF profile towards the NRF with the TMGI and Area Session ID.

-	The policy of broadcast session is determined based on the service requirements per MBS service area.

-	The MB-SMF may select the MB-UPF based on the MBS service area.

-	The MBS service area(s) are indicated to the UE in the Service Announcement as defined in clause 6.11.

The MBS session establishment procedure is performed as defined in clause 7.3.1 with the following additions:

-	MB-SMF requests the AMF to transfer an N2 message (i.e. MBS Session Resource Setup Request) to the NG-RAN nodes of the MBS service area with broadcast session information which additionally includes the Area Session ID and MBS service area.

-	The NG-RAN uses the received MBS Session ID and Area Session ID to determine the local broadcast session context.

# * * * * End of change * * * *