3GPP TSG-SA WG2 Meeting #148E	S2-2108984r02

E-Meeting, 15th – 22nd November 2021	(revision of S2-2107810)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0457 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on transfer of analytics context and analytics subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The transfer of analytics context and analytics subscripton can be done for analytics subscriptions related to specific NF(s), e.g. when the transfer is initiated by source NWDAF due to internal triggers (e.g. load balancing, graceful shutdown). - As specified in clause 6.1B.2.2, "If the source NWDAF discovers that the analytics consumer may change concurrently to this (Analytics Subscription Transfer) procedure, the source NWDAF should not perform the procedure." And NOTE 1 following this description says "To discover if the analytics consumer may change, the source NWDAF can, for example, check if the serving area of target NWDAF is within the serving area of the analytics consumer, if the serving area information is available." But before discovering whether the analytics consumer may change or not, the source NWDAF does not select the target NWDAF. So the description in NOTE 1 is wrong/misleading. - As proposed in CR0414, "Analytics subscription aggregation information" is removed from the AnalyticsSubscription_Transfer. Correspondingly, the descriptions related to this parameter should also be removed from the procedure in clause 6.1B.2.2. - In clause 6.1B.2.2 step 6, it states "the new subscription Id, that had been assigned by the target NWDAF, in the Subscription Change Notification Correlation Id parameter of this message". This is not aligned with the definition of Nnwdaf_AnalyticsSubscription_Notify message in clause 7.2.4. - In clause 6.1B.2.2 step 11-12, there is a NOTE 8 describing "Notification correlation information", while it is more appropriate to move it to step 6 for analytics subscription transfer procedure. - Some descriptions only apply to analytics context/subscription transfer for UE related analytics. These need to be clarified/corrected. - In clause 6.1B.4, the "Aggregation related information" and "Model related information" in the listed output information are part of the analytics context information. - In clause 6.1B.4, the Note include the text “Information allowing to identify an anlytics context” which is not clear and needs further clarificaiton |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the transfer of analytics context and analytics subscripton also applies to analytics subscriptions related to specific NF(s). - Correct the descriptions regarding how the the source NWDAF discovers that the analytics consumer may change. - Remove descriptions related to the parameter "Analytics subscription aggregation information" from the procedure in clause 6.1B.2.2. - Correct the description about new/old subscription Id in the procedure to align with the Nnwdaf_AnalyticsSubscription_Notify service operation definition. - Move the NOTE related to "Notification correlation information" to step 6 for the procedure in clause 6.1B.2.2. - Clarify the descriptions only applying to analytics context/subscription transfer for UE related analytics. - In clause 6.1B.4, fix the format of "Aggregation related information" and "Model related information" in the listed output information, to show they are part of the analytics context information. - In clause 6.1B.4, the text “Information allowing to identify an anlytics context” is replaced by the “a list of context identifier(s)” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete, wrong or unclear specification on analytics context/subscription transfer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.1, 6.1B.2.2, 6.1B.4. |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

### 6.1B.1	General

In a multiple NWDAFs deployment scenario, transfer of Analytics context and Analytics subscription information is used to support the target NWDAF to produce the needed Analytics.

When the Analytics consumer provides the target NWDAF with information on the subscription that could be transferred from the source NWDAF, the target NWDAF may initiate the transfer of Analytics context information. The Analytics consumer provides the information via Nnwdaf_AnalyticsSubscription_Subscribe service operation. When the Analytics consumer is an AMF, the old subscription information (if related to a UE) may be shared between the source AMF and target AMF in a UE context transfer procedure as specified in TS 23.502.

An Analytics subscription transfer to a target NWDAF may be initiated by a source NWDAF, followed by an Analytics context transfer initiated by the target NWDAF. An NWDAF may transfer one or more of its analytics subscriptions to another NWDAF instance due to internal (e.g. load balancing, graceful shutdown) or external triggers (e.g. UE mobility). For external triggers, the NWDAF may subscribe with NF(s) to be notified about corresponding events. As for UE mobility, upon the UE location change event notified by the AMF subscribed by the NWDAF, the NWDAF determines whether it can continue to provide the analytics service. If the NWDAF cannot continue to serve the consumer, it should either initiate analytics subscription transfer as described in clause 6.1B.2.2, or notify the analytics consumer that it cannot provide the service anymore, so that the analytics consumer can select a new NWDAF as described in clause 6.1B.2.1.

Procedures for analytics subscription transfer allow one NWDAF instance to transfer its ongoing analytics subscriptions to another NWDAF instance. The transfer can be done for all subscriptions or just a selected subset of subscriptions related to specific area(s), specific Analytics ID(s), specific NF(s) and/or specific UE(s).

The analytics subscription transfer can also be prepared if the source NWDAF instance anticipates that it will soon not be able to continue its current analytics tasks.

******************** 2nd Change ********************

#### 6.1B.2.2	Analytics Subscription Transfer initiated by source NWDAF

The procedure in Figure 6.1B.2.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

If the source NWDAF discovers that the analytics consumer may change concurrently to this procedure, the source NWDAF should not perform the procedure. In such a case, the source NWDAF may send a message to indicate to the analytics consumer that it will not serve this subscription anymore.

NOTE 1:	To discover the possible change of analytics consumer, if the Analytics ID is UE related, the source NWDAF takes actions responding to external trigger (such as UE mobility), for example, checking if the Target of Analytics Reporting is still within the serving area of the analytics consumer, if the serving area information is available.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.2.2-1 is applicable for analytics subscription transfer across NF Sets or if the NWDAF is not deployed in a Set.

Figure 6.1B.2.2-1: Analytics subscription transfer initiated by source NWDAF

0.	The analytics consumer subscribes to analytics from source NWDAF. The analytics consumer may send its NF ID or serving area, enabling NWDAF to determine whether the following analytics subscription transfer procedure is applicable. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in clause6.1B.2.3.

2.	Source NWDAF determines, e.g. based on the UE location information received and the analytics consumer's serving area either directly received in step 0 or indirectly received via NRF, to perform an analytics subscription transfer to target NWDAF(s). Therefore, the source NWDAF determines the analytics subscription(s) to be transferred to a target NWDAF.

3.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer. In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics (see clause 6.1.3) to preferably select a target NWDAF that is already serving the consumer.

4.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF. The request contains a callback URI of the analytics consumer. The request also contains active data source ID(s), ML model information and/or ML model provider NWDAF ID(s), which are related to the analytics subscriptions requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 1). The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

5.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation based on the information received from the source NWDAF.

Target NWDAF may use the ML model information, if provided in the Nnwdaf_AnalyticsSubscription_Transfer request, to retrieve the ML model(s) and use it for the transferred analytics subscription. If ML model provider NWDAF ID(s) is provided in the Nnwdaf_AnalyticsSubscription_Transfer request, target NWDAF may request or subscribe to the ML model(s) from the indicated ML model provider NWDAF(s) as specified in clause 6.2A, taking account of the locally configured set of NWDAFs containing MTLF if any, and use the ML model(s) for the transferred analytics subscription.

NOTE 3:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new subscription Id to the received analytics subscriptions.

NOTE 4:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

6.	Target NWDAF informs the analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message. A new subscription Id, which was assigned by the target NWDAF, is provided in the Subscription Correlation Id parameter; and the old Subscription Correlation Id, which was allocated by the source NWDAF, is provided in the Subscription Change Notification Correlation ID parameter of this message as specified in clause 7.2.4.

NOTE 5:	Notification correlation information in the Nnwdaf_AnalyticsSubscription_Notify message allows the analytics consumer to correlate the notifications to the initial subscription request made with the source NWDAF in step 0.

NOTE 6:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

NOTE 7:	If this subscription is used as input for analytics aggregation by the analytics consumer, the analytics consumer may inform the other NWDAFs instance participating in this analytics aggregation that the Set of NWDAF identifiers of NWDAF instances used by the NWDAF service consumer for this analytics aggregating (see clause 6.1.3) has changed using the Nnwdaf_AnalyticsSubscription_Subscribe service operation.

7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the target NWDAF requests the "analytics context". The analytics context information transfer procedure is specified in clause 6.1B.3.

8.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

9.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

10.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions. In addition, Source NWDAF unsubscribes with the ML model provider NWDAF(s), if exist, which are no longer needed for the remaining analytics subscriptions.

NOTE 8:	At this point, the analytics subscription transfer is deemed completed, i.e. the source NWDAF can delete all information related to the successfully transferred analytics subscription.

11-12.	Target NWDAF at some point derives new output analytics based on new input data and notifies the analytics consumer about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

******************** 3nd Change ********************

### 6.1B.4	Contents of Analytics Context Information

The consumers of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3) provide the following input parameters:

-	A list of analytics context identifier(s): identify a set of analytics context information that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance. The analytics context identifier is provided as the following:

-	Subscription Correlation ID: identifies the analytics subscription for which the related analytics context information is requested; or

-	A set of SUPI and associated Analytics ID for UE related Analytics; or

-	An Analytics ID for NF related Analytics.

-	[OPTIONAL] Requested Analytics Context Information per analytics context identifier: indicates which part of the analytics context information the consumer wishes to receive. Following values are specified:

-	Pending output Analytics;

-	Historical output Analytics;

-	Analytics subscription aggregation information;

-	Data related to Analytics;

-	Aggregation related information;

-	Model related information.

NOTE:	A list of "analytics context identifier(s)" can be provided by the source NWDAF to the target NWDAF in an analytics subscriptions transfer request as described in clause 6.1B.2.2. A list of "analytics context identifier(s)" can also be provided by the NWDAF service consumer to the target NWDAF in the Nnwdaf_AnalyticsSubscription_Subscribe request as defined in clause 6.1B.2.1.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3), the output information listed below:

-	(Set of) Analytics context information matching the input parameters of the Nnwdaf_AnalyticsInfo_ContextTransfer request. If no Requested Analytics Context Information parameters are available in the request, all available analytics context information is sent. Analytics context information includes the following information parts, if available:

-	Analytics related:

-	Pending output analytics (i.e. not yet notified to the consumer);

Historical output analytics information. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

-	Timestamp(s) of the last output analytics provided to the analytics consumer(s). Value is set to 0 if no output analytics had been sent yet.

-	Analytics subscription aggregation information (only provided when analytics context is related to analytics aggregation): information about the analytics subscriptions that the source NWDAF has with the NWDAFs that collectively serve the transferred analytics subscription, which includes IDs and analytics metadata information of these NWDAFs for specific analytics ID(s), and optionally input parameters of analytics exposure as defined in clause 6.1.3.

-	Data related to Analytics:

-	Historical data that is available at the source NWDAF and that is related to the analytics to be handed over to the target NWDAF. If available, the time period of the collected data, NF ID(s) of the data source(s) and information about subscriptions with the data sources that are related to the analytics is included.

-	Aggregation related information: Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	(Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytic subscriptions.

-	Model related information:

-	Model provider NWDAF ID(s): Instance ID(s) of the ML model provider NWDAF(s) from which the consumer NWDAF currently subscribes to the ML model information used for the analytics.

******************** End of Change ********************