3GPP TSG-SA WG2 Meeting #148E	S2-2109014r01

Elbonia, November 15 - 22, 2021

Title:	[Draft] LS on Interworking with eMBMS for Transport Only Mode

Response to:	-

Release:	Rel-17

Work Item:	5MBS

Source:	[Ericsson, will be] SA2

To:	SA4, SA6

Cc:

Contact Person:

Name:	Robbie Ling

Tel. Number:

E-mail Address:	Robbie.Ling@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Agreed version of CR 0066 of TS 23.247

1. Overall Description:

In clause 5.2 and 6.8 of TS 23.247 v17.0.0, the function of interworking with LTE MBMS (i.e. eMBMS) has been specified. Annex A includes configuration variants that SA2 understands relevant for applications on how they utilise MBS.

The MBSF and MBSTF are not involved in Transport Only Mode without interworking with LTE MBMS. In this case, the AF interacts either directly with the MB-SMF or indirectly via NEF (for AF outside the trusted domain). The MBS data are sent directly to the MB-UPF using Nmb6.

If interworking with LTE MBMS is required, MBSF and MBSTF are used regardless of Transport Only Mode or Service Mode. In this case, the AF interacts with MBSF and MBSTF (or via NEF for AF outside the trusted domain), as defined in clause 5.2 of TS 23.247.

For the case of interworking with LTE MBMS at 5GC for Transport Only Mode, SA2 agrees that the interface provided by MB-SMF to MBSF (i.e., Nmb1) is identical as MB-SMF to AF (i.e., Nmb13) or NEF (i.e., N29mb), and the interface provided by MB-UPF to MBSTF (i.e., Nmb9) is identical as MB-UPF to AF (i.e., N6mb).

2. Actions:

To SA4:

ACTION: 	SA2 would like to understand if same principles of interworking with LTE MBMS for Transport Only Mode can be applied for Service Mode. And SA2 kindly asks SA4 to take the above information into account and provide feedback, if any.

To SA6:

ACTION: 	SA2 kindly asks SA6 to take the above information into account and provide feedback, if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #149E		February 14 - 25, 2022			Elbonia