SA WG2 Meeting #149e	S2-2200410v1

# February 14th – 25th, 2022; Elbonia               		(revision of S2-220)

Source: 	China Telecom

Title: 	FS_5WWC_Ph2 Solution related with WT#2

FS_5WWC_Ph2 TR Solution related with WT#2

Document for: 	Approval

Agenda Item: 	9.13

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution:

1	Discussion

Void

2 Proposal

It is proposed to create TR 23.XXX on FS_5WWC_Ph2 with the attached skeletton

*** Start of the change (all new text) ***

6.X	Solution #X: New method for non-3GPP device connected behind a 5G-RG

6.X.1	Description

This solution addresses Key Issue“Providing differentiated service for UE and Non-3GPP devices connected behind a 5G RG.”

Figure 6.x.1 depicts the architecture for devices connected behind a 5G-RG. In this solution, the AF, which supports the functionality of assigning virtual identifier (e.g. virtual SUCI) can provide a list of the virtual identifier to the UDM through NEF. After the 5G-RG registers to the 5GC, a virtual identifier would be assigned to it as the identifier of the device which has been connected to 5G-RG. Then the 5G-RG could use the virtual identifier to initial registration procedure and PDU session establishment procedure via W-AGF on behalf of the device. Different QoS profiles may be applied per device behind the 5G-RG by 5GC.

NOTE 1: AF in figure 6.x.1 is management by operator.

NOTE 2: The interaction between the AF and 5G-RG may be out of 3GPP scope.

Figure 6.x.1 architecture for non-3GPP devices connected behind a 5G-RG

6.X.2	Procedures

Figure 6.x.2-1: Procedure for 5G RG and Device Registration

0. The AF sychronizes the list of virtual identifier (e.g. virtual SUPI) to UDM through the NEF.

1. The 5G-RG registration procedure, as descibed in clause 7.2.1.1 of in TS 23.316.

2. The UDM informs the AF that the 5G-RG has been successfully registered with 5GC. AF should save a list of SUCI of registered 5G-RGs.

3. The device connects to the 5G-RG via a layer-2 (L2) connection.

4. The 5G-RG applies to the AF for a virtual identifier by carrying the SUCI of the 5G RG and the MAC address of the device.

5. The AF checks if the 5G-RG is in the list of registered 5G-RGs. If it has registered successfully, the AF will inform the UDM of the mapping between the virtual identifier and the MAC address of the non-3GPP device.

6. If the 5G-RG is in the list of registered 5G-RGs, the AF responds to the 5G-RG by assigning a virtual identifier to the device. The AF saves the timestamp and the mapping information between the virtual identifier and the MAC address of the device.

NOTE: 	Step 4 and 6 may be out of the 3GPP scope.

7. The 5G-RG uses the virtual identifier to initial registration procedure on behalf of the device, as described in clause 7.2.1.1 of TS 23.316.

Editor’s Note: modification of registration procedure in 23.316 clause 7.2.1.1 is FFS

8. 5G-RG initates PDU Session Establishment procedures on behalf of device. And differentiated QoS can be provided to non-3GPP device according to the virtual identifier.

6.X.3	Impacts on Existing Nodes and Functionality

UDM:

- Inform AF that a 5G-RG has been successfully registered.

- Store the mapping between the virtual identifier and the MAC Address of the non-3GPP device

5G-RG:

- Apply to the AF for a virtual identifier

- Store the mapping information between the virtual identifier and the MAC address of the non-3GPP device

- Support performing registration and PDU Session related procedure on behalf of non-3GPP device behind.

### *** End of the change ***