# SA WG2 Meeting #149-e (e-meeting)	S2-2101267

# 14-25 February 2022, Elbonia

Title:		Draft Reply LS on alternative IMSI for MUSIM

Reply to:	LS to SA2 and CT1 on alternative IMSI for MUSIM (R2-2201718, S2-2201252)

Release:	Release 17

Work Item:	MUSIM

Source:	SA2

To:		RAN2

Cc:		CT1, RAN3

Contact person:	Lars Nord

Lars.nord@sony.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

SA2 thanks RAN2 for the LSs (R2-2111329/S2-2200018 and R2-2201718/S2-2201252) on alternative IMSI for MUSIM. SA2 has discussed the LSs and would like to provide the following information.

When SA2 selected and specified the solution using Alternative IMSI Offset, the expectation from SA2 was that minimal impact would be required on the current UE, MME and eNB behaviour, except the calculation of the Alternative IMSI value.

TS 23.401 clause 4.3.33.5 specifies that the UE may receive Accepted IMSI Offset over NAS signalling. Then:

-	For the UE:

-	The UE shall then use the Accepted IMSI Offset to calculate the Alternative IMSI value which is then used in the UE to derive UE ID which is later used in the PO/PF calculations.

-	The behaviour inside the UE is not SA2 responsibility, so any behaviour could be compatible as long as the outcome is what is expected in terms of computation of the paging occasions, therefore there is no misalignment between SA2 and CT1/RAN2 about how UEs NAS and the UEs AS use Alternative IMSI.

-	For the MME:

-	The MME provides the UE ID, derived from the Alternative IMSI value calculated based on the IMSI and the Accepted IMSI Offset, to the eNB.

-	S1AP does not require any updates to support this MUSIM feature and no impacts to eNB PO/PF calculations.

# 2	Actions

To RAN2

ACTION: 	SA2 asks RAN2 to consider the above.

# 3	Dates of next TSG SA WG2 meetings

3GPPSA2#150-e	4 - 8 April 2022	Electronic Meeting

3GPPSA2#151-e	16 - 20 May 2022	Electronic Meeting