3GPP TSG-WG SA2 Meeting #149E e-meeting	S2- 2200099r08

Elbonia, 14th – 25th Feb 2022

Source: 	China Mobile

Title: 	Key issue of supporting UPF expose information to other NFs

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Abstract of the contribution: This paper introduces a key issue for supporting UPF expose information to other NFs to the TR 23.700-62.

1 Discussion

The FS_ UPEAS SID objectives :

WT#2: Study UPF event exposure service(s) that would support, e.g.

-	Consumption of UPF exposure services by the PCF, NWDAF, CHF, NEF, Trusted AF and other NFs (if needed).

-	(To support the UPF exposure service, if needed) Use of SMF services, PCF services, NWDAF services, CHF services, NEF services, Trusted AF services by the UPF.

NOTE 1: This will not define solutions where UPF exposes information that it is not originator of, i.e. not re-expose information owned and exposed by other NFs.

-	Relevant Event IDs.

2 Proposal

It is proposed to add the following key issue to TR 23.700-62 FS_UPEAS.

FIRST CHANGE “all the text are new”

5.X	Key Issue #X: Support UPF expose information to other NFs

5.X.1	General description

To enable flexible communication between UPF and other 5GC NFs, the UPF can expose network information to NFs through UPF event exposure service.

In Release 17, TS 23.548 has supported that UPF can expose QoS monitoring results to local NEF. In Release 18, we need further study whether UPF can also expose other useful information to other NFs to optimize the network performance..

The key issue is to define use cases for UPF event exposure (including the related Event Id) and for each use case determine whether the consumer directly contacts the UPF for its subscription or whether the consumer goes via an intermediate function like the SMF.

The following aspects should be studied to support UPF event exposure service(s):

-	What specific information the UPF can expose to NWDAF for deriving existing or potential new data for analytics as specified in TS 23.288[5]). Whether and how UPF is subscribed and exposes this specific information to the NWDAF to meet the data collection requirement with specific aspects as defined in TS 23.288, e.g. bulked data collection, event muting mechanism, data collection for any UE of AOI, etc.

-	How and what specific information the UPF can expose to NEF/Local NEF/trusted AF.

NOTE: The Relevant Event IDs of the UPF event exposure services above can be introduced.

- 	Whether other NFs that need to invoke UPF event exposure service., If yes, how and what specific information the UPF can expose to these NFs.

SECOND CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services

.

End of CHANGE