3GPP TSG-WG SA2 Meeting #149E e-meeting	S2- 2200100r12

Elbonia, 14th – 25th Feb 2022

Source: 	China Mobile

Title: 	TR 23.700-62 FS_UPEAS Architectural assumptions and requirements

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS / Rel-18

Architectural Assumptions and Requirements description for TR23.700-62.

1	Discussion

void

2 Proposal

It is proposed to update TR 23.700-62 on FS_UPEAS as follows.

* * * Start of changes * * * * All text is new

4	Architectural Assumptions and Requirements

Editor's note:	This clause will document the Architectural Assumptions and Requirements applicable for the study.

4.1	Architectural Assumptions

The architecture and framework as specified in TS23.501 [2], TS23.502 [3], and TS23.503 [4] are regarded as the baseline for the present study.

- The SMF is responsible for controlling UPF packet processing.

- The UPF can only expose 5GC information which is originated in the UPF.

4.2	Architectural Requirements

Solutions shall build on the 5G System architectural principles as in TS 23.501 [2], including flexibility and modularity for newly introduced functionalities.

- The Rel-18 N6, N4 interface shall be based on the existing interface design and backward compatible. No impact is introduced on N3, N9 interfaces from this study.

NOTE1:	This study is limited to event exposure, and thus interfaces can only be impacted due to this study.

NOTE2:	Some aspects as following should be considered when the solution evaluation is proposed.

1) The usage of direct UPF event exposure for exposure of some data when these data are already available via usage of SMF + N4 needs to be justified.

2) The performance of UPF user plane traffic handling shall not be degraded due to mechanisms defined in this study.

3) The co-existence for the UPF supporting UPEAS feature and UPF supporting R17 (i.e. not supporting the feature) should be considered.

* * * End of changes * * * *