3GPP TSG-SA2 Meeting # 149E (e-meeting)	S2-2200145r01

# Feb 14 – 25, 2022		  										                             (revision of S2-22xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3469 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct PEI used for MUSIM and network subscriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2022-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As described in LS S2-2200040. LS from GSMA: Reply LS on IMEI for Non-Public Networks/Private Networks without using USIM, for the specific two-part question in the LS, GSMA TSG provides the following answers: Question part 1: “SA2 would therefore like to ask GSMA TSG whether a device where the device can register simultaneously to a PLMN using (U)SIM and a private network/SNPN that does not require (U)SIM is required to have and signal a separate IMEI for each simultaneous registration.” GSMA TSG reply 1: Yes. Question part 2: “SA2 would also like GSMA TSG to clarify whether the requirement applies only to the IMEI as PEI used for 3GPP registration over 3GPP RAT or also to registration via N3IWF as long as different UE permanent identities are registered simultaneously for the same IMEI?” GSMA TSG reply 2: The GSMA requirement applies only to the IMEI (and IMEI part of IMEISV) as PEI used for 3GPP registration with PLMN/private network/SNPN over 3GPP RAT. When other type of PEI is used, it is out of scope of GSMA to provide feedback on that. Furthermore, the number of the network subscription is more than the number of PEI, the UE only need to keep UE used for the network unchanged during the registered states, after deregistered, the PEI is possible not used for the network subscription anymore. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When SNPN network subscription is USIM, the PEI is IMSI based type in general. When the UE has registered to a network by using one network subscription, during the UE registered state in the network, the PEI used for the network subscription shall not change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not align with GSMA reply for the SNPN network credentials and IMEI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.0, 5.38.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGE *****

### 5.30.2	Stand-alone non-public networks

#### 5.30.2.0	General

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3, the architecture for 5GC with untrusted non-3GPP access (Figure 4.2.8.2.1-1) for access to SNPN services via a PLMN (and vice versa) and the additional functionality covered in clause 5.30.2.

Alternatively, a Credentials Holder (CH) may authenticate and authorize access to an SNPN separate from the Credentials Holder based on the architecture specified in clause 5.30.2.9. Idle mode mobility is supported as defined in clause 5.30.2.11. In this Release, direct access to SNPN is specified for 3GPP access only.

The following 5GS features and functionalities are not supported for SNPNs:

-	Interworking with EPS is not supported for SNPN.

-	Also, emergency services are not supported for SNPN when the UE accesses the SNPN over NWu via a PLMN.

-	While roaming is not supported for SNPN, e.g. roaming between SNPNs, it is possible for a UE to access an SNPN with credentials from a CH as described in clause 5.30.2.9.

-	Handover between SNPNs, between SNPN and PLMN or PNI NPN are not supported.

-	CIoT 5GS optimizations are not supported in SNPNs.

-	CAG is not supported in SNPNs.

A UE with two or more network subscriptions, where one or more network subscriptions may be for a subscribed SNPN, can apply procedures specified for Multi-USIM UEs as described in clause 5.38. The UE shall use a separate PEI for each network subscription when it registers to the network. When the UE has registered to a network by using one network subscription, during the UE registered state in the network, the PEI used for the network subscription shall not change.

NOTE:	The number of preconfigured PEIs for a UE is limited. If the number of network subscriptions for a UE is greater than the preconfigured number of PEIs, the number of network subscriptions that can be registered with the network simultaneously is restricted by the number of pre-configured number of PEIs.

***** NEXT CHANGE *****

***** END OF CHANGES *****