SA WG2 Meeting #149E (e-meeting)	S2-2200166r16

# Feb 14 – 25, 2022

Source:	CATT, OPPO

Title:	Architectural assumptions

Spec:	3GPP TR 23.700-80 v0.0.0

Agenda item:	9.21

Document for:	Agreement

1. Introduction

KPIs for AI/ML model transfer in 5GS is specified in clause 7.10 of TS 22.261. e.g.:

"The 5G system shall support AI/ML model downloading with performance requirements as given in Table 7.10-2.

Table 7.10-2 KPI Table of AI/ML model downloading

| # Max allowed DL end-to-end latency | # Experienced data rate # (DL) | # Model size | # Communication service availability | # Reliability | # User density | # # of downloaded AI/ML models | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1s | 1.1Gbit/s | 138MByte | 99.999 % | 99.9% for data transmission of model weight factors; 99.999% for data transmission of model topology |  |  | AI/ML model distribution for image recognition |
| 1s | 640Mbit/s | 80MByte | 99.999 % |  |  |  | AI/ML model distribution for speech recognition |
| 1s | 512Mbit/s(see note 1) | 64MByte |  |  |  | Parallel download of up to 50 AI/ML models | Real time media editing with on-board AI inference |
| 1s |  | 536MByte |  |  | up to 5000~ 10000/km2 in an urban area |  | AI model management as a Service |
| 1s | 22Mbit/s | 2.4MByte | 99.999 % |  |  |  | AI/ML based Automotive Networked Systems |
| 1s |  | 500MByte |  |  |  |  | Shared AI/ML model monitoring |
| 3s | 450Mbit/s | 170MByte |  |  |  |  | Media quality enhancement |
| NOTE 1: 512Mbit/s concerns AI/ML models having a payload size below 64 MB. TBD for larger payload sizes. NOTE 2: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |

"

However, since network resources are required to support transmission of AI/ML model or data for AI/ML-based services/applications, it should be avoided that too much network resources are engaged for this purpose, leading to poor network performance, e.g. network overload or congestion, and/or failing to meet the QoS requirements as specified in clause 5.7 of TS 23.501 for current non-AI/ML based services (e.g. Conversational Voice).

This consideration is also addressed in Objective 2 as proposed in SP-211648 WID of study on System Support for AI/ML-based Services:

"Objective 2 (WT#1.2): Study possible QoS, Policy enhancements to support Application AI/ML operational traffic while supporting regular (non Application-AI/ML) 5GS user traffic."

So it is proposed to add as architectual assumptions that the overall network performance and support of non-AI/ML based services (e.g. QoS guarantee) should not be deteriorated due to supporting Application AI/ML operational traffic.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-80.

* * * First Change * * * *

## 4.1	Architecture Requirements

The Application AI/ML operation logic is controlled by an Application Function (AF).

Any AF request to the 5G System in the context of 5Gs assistance to Application AI/ML operation should be authorized by the 5GC.

The 5G System should ensure the performance KPIs specified in TS 22.261 [xx], clause 7.10, to support the Application AI/ML traffic.

## 4.2	Architectural Assumptions

To support AI/ML based services/applications via 5GS, the following architectural assumptions are made in the present study:

-	In Rel-18, an Application AI/ML operation is conducted within a single slice, i.e. all UEs which are involved in a given Application AI/ML operation are served by the same S-NSSAI and the Application Function (AF) belongs to this S-NSSAI.

-	In Rel-18, roaming is not supported, i.e. inter PLMN coordination aspects will not be studied.

NOTE : The 5G System architecture to support the Application AI/ML operation should not preclude the support of roaming in a future release.

Editor’s Note: whether 5GC needs to be aware of the Application Layer AI/ML operation type needs further discussion.

## 4.3	Reference Architecture

The architecture, framework, definitions and terminology specified in TS 23.501 [X], TS 23.502 [Y] and TS 23.503 [Z] and TS 23.288 [M] are the baselines for the present study.

* * * End of Change * * * *