SA WG2 Meeting #S2-149E	S2-2200175

14 - 25 February, 2022, Electronic meeting

Title:	Reply LS on conclusions of authentication methods for initial access for onboarding

Response to:	S2-2200029 / S3-214335

Release:	Release 17

Work Item:	eNPN

Source:	SA2

To:	SA3

Cc:	-

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA3 for their LS on conclusions of authentication methods for initial access for onboarding (S2-2200029 / S3-214335).

With the respect to the following text in the conclusion (S3-214335) related to the scenario with no DCS involvement:

-	Primary authentication between UE and AUSF in the O-SNPN as currently described in TS 33.501 [2], including Annex B. No interaction with DCS during primary authentication. Optionally, after primary authentication during the establishment of onboarding PDU session, O-SNPN can trigger secondary authentication with the DCS using default credentials as per clause 11.1 in 33.501[2].

as well as the implementation of this conclusion in TS 33.501 Annex I.9.2.2 as follows:

### I.9.2.2	Primary authentication without using DCS

When the primary authentication is performed between the UE and the ON-SNPN, any one of the existing authentication methods defined in the present document may be used, i.e., 5G AKA, EAP-AKA’ or any other key-generating EAP authentication method (e.g., EAP-TLS).

SA2 is of the opinion that the highlighted text may be lumping together the following two cases:

1.	UE has only Default UE Credentials and uses the Rel-17 procedures for UE onboarding to obtain user plane connecitivity with a Provisioning Server, with a view of obtaining a first set of SNPN credentials.

2.	UE is already provisioned with SNPN credentials and uses Rel-16 procedures to obtain user plane connectivity with a Provisioning Server, with the view of obtaining a new set of SNPN credentials.

The latter case is captured in TS 23.501 clause 5.30.10.2.1 in the following NOTE:

NOTE:      If the UE is already provisioned with a set of CH credentials and needs to be provisioned with an additional set of SNPN credentials, the UE can leverage the User Plane connection enabled by the available set of CH credentials to get access to a PVS.

In the former case, SA2 wonders if all the authentication methods listed in TS 33.501 Annex I.9.2.2 would still apply.

SA2 respectfully asks SA3 to take the feedback provided above into account and provide clarifications in TS 33.501.

2. Actions:

To SA3 group.

ACTION: 	SA2 respectfully asks SA3 to take into account the feedback provided above and provide clarifications in TS 33.501.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#150-e			04 - 08 April 2021			Electronic Meeting